K4_K4r.DK

Loading in the file Kawai_K4.DK will set up Band-in-a-Box for the Kawai K4

synthesizer,and the K4r Synth Module. The K4/K4r sounds work well with Band-

in-a-Box.   It will set up :

1. Drum #'s for Kawai synths.

2. Patch Map (assigned to factory settings of Kawai K4/K4r).

3. Startup Patches and Channels in Midi window.

The first thing for you to do is to setup the Kawai K4/K4r in Multi-Timbral

mode to receive multiple instruments at once. In "Single" mode, the K4

will only play 1 instrument at a time. In "Multi" mode it will play

up to 8 instruments + drums at once- so you want to be in "Multi" mode

and have a setup made with 7 instruments/midi channels. To ensure that the

K4/K4r will receive what it is supposed to, select the SYSTEM button.Keep

pressing it until you see MIDI =.With the 'yes'button go to RCV and under

the options,choose SECT.This will enable Band-in-a-Box to send patch changes.

To setup your K4/K4r do the following:

1. Go to "Multi" Mode by selecting the MULTI button. You will be on patch 1.

2. Now press the EDIT button,then the C-sec-Ch button to select MIDI

channels for the setup.

3. You will then select the Section Select buttons and use the 'yes'

and 'no' buttons to scroll through MIDI channels for each of the 8

parts.(**if not working,hit buttons 9-16 to "unmute" Parts 1-8**).

Each part (1-8) represents a different instrument (except Drums).

To setup instruments you will use the A-inst button.

Setup your Multi patch as follows:

Bass   Piano Guit  Horns Strings Melody Thru Drums

Section Select -  1     2     3     4      5      6      7    8

MIDI channel -    2     3     7     8      6      4      5    -

Instrument -     D9    C2    A6    C13    A3     A6     A6    -

Section 8 is left blank because the Drums are set to channel 10 and

act as a seperate section. Some drum instruments may be too loud/soft. To

correct this you would select the DRUM button and keep pressing it until you

see a note number on the upper right portion of the diplay (eg C1).Press the

drum notes on the K4 keyboard that you want to adjust and then press DRUM

until Level S1 appears.Using the Value slider or the 'yes'/'no' buttons,

adjust the level of S1 and S2(next page) accordingly.To save this ,select

the Write button and follow the instructions from within that window. The

same procedure will be done to save your MULTI patch with the only

difference being that you'll have to be in MULTI mode before choosing WRITE.

Have Fun!