SG-10 Printer Info

General Info:

In order to get proper screen dumps with the alt/help

function, you must install the printer from the desktop with

960 pixels/line.  Save your desktop after you do this so that

the printer will be installed when you boot in the future.

You must also set the printer DIP switches as follows:

1-2       OFF(down)      IBM Character set #2 on

2-2       OFF(down)      IBM Mode on

Since your Star printer now thinks it is an IBM you will need

to use an IBM compatible printer driver for DEGAS or any

other printing programs.  SG-10.PRT is actually a renamed

upload of IBMPRO.PRT.

Pressing alt/help while DEGAS is displaying a picture will

produce a horizontal print out (not nearly as good, of

course!)

SG-10.HEX Info:

I used the IBM mode so that no DIP switches need to be

changed when using 1st Word.  The advantage over Star mode is

that most of the foreign and scientific symbols are directly

supported by the ST.

Unfortunately, you can only change IBM/Star mode with the DIP

switch setting.  Therefore, the nice Star arrows and

copyright symbol are unavailable.  The arrows are simulated

using backspace commands.  The arrows will not print properly

in NLQ, only draft.  Light Printing is not supported by the

SG-10.

In Draft mode, Bold (F1) selects emphasized print.  All other

functions are self-explanatory.

Since the NLQ mode is actually a seperate character set, the

bold, italic, and other custom fonts require a little more

effort.  When the other fonts are selected, NLQ is turned off

and then back on again when finished.  Bold is double-strike,

emphasized mode.  Italic gets double-strike. Super and Sub-

script work in the normal fashion.

NOTE:  On page 35 of GUIDE.DOC, there is an example of multi-

line underlining.  In NLQ, when the font is changed, the

underline also changes and the space between the NLQ and

alternate font is NOT underlined.  You could bakspace and add

the NLQ underline if desired.

The NLQ printout of GUIDE.DOC will really show off your SG-

10.  Forty-two NLQ pages will take quite a bit of time,

though.

If you want a Star configuration, the Gemini 10X file by Dave

Witzke is a good place to start.

HAPPY PRINTING

Mike Atkinson CIS -  71777,3510     Delphi - Resorter