02200000000801

1

2

9[...............................................................]0

MC 68000 MACHINE LANGUAGE COURSE PART VII by Mark van den Boer

I would like to dedicate this part to Willeke,  who gives  Richard

sleepless  nights and the inspiration to write even more  exciting

issues of ST NEWS.  I only saw Willeke on photograph, but she must

be  a fine girl.  In my opinion there are three qualities which  a

girl  must have,  to qualify as a fine girl.  These are:  1)  like

Queensrche,  2) like ST NEWS (no,  she doesn't have to like  this

particular machine language course).

Now,  you're all anxious to know the third quality, aren't you? If

you think you know the third one,  send your answer to ST NEWS.  A

bottle  of  wine will be raffled among the persons  who  gave  the

right answer.  There will be another bottle for the most  original

answer!

Bit Manipulation instructions

Instruction:   BTST

Syntax:        BTST Dn,<ea> or BTST #,<ea>

Data sizes:    only byte when <ea> is an address.  Only long  when

<ea> is a data register.

Condition codes affected:

X    not affected

N    not affected

Z    set if the result is zero, cleared otherwise

V    not affected

C    not affected

Addressing modes allowed:

Destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#         (only when source is Dn)

Function: Test a single bit of an effective address operand.  Bits

are  numbered  from  0  to 31,  where  0  is  the  least

significant bit (you could use this instruction to  test

if  a number is odd).  This instruction is  useful  when

specific  bits of an operand have to  be  checked.  E.g.

when reading joystick information one could test with  a

single  instruction whether the fire-button was  pressed

or not.  Compared to the 6502 and 6809 this  instruction

(in  fact all bit manipulation instructions) are a  step

forward,  since  with these older processors one had  to

put  the data in a register first,  then filter the  bit

with  an AND-operation and then the Z-bit in the  status

register  was at last set.  Viva el 68000!!  Since  this

instruction has the rather odd property of only  working

on  byte  and  long operands it is  important  that  you

remember   what  I  wrote  in  a  previous  part   about

specifying data sizes.

Examples:

Instruction              Before              After

BTST.B #5,$345678        $345678             $345678 contains

contains $78        $78

Z-bit is 1

BTST.L d0,d1             d0=0                d0=0

d1=$12345678        d1=$12345678

Z-bit is 0

Instruction:   BCLR

Syntax:        BTST Dn,<ea> or BTST #,<ea>

Data sizes:    only byte when <ea> is an address.  Only long  when

<ea> is a data register.

Condition codes affected:

X    not affected

N    not affected

Z    set if the result is zero, cleared otherwise

V    not affected

C    not affected

Addressing modes allowed:

Destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#         (only when source is Dn)

Function: Bit  test and CLeaR.  First tests the bit to be  cleared

and  sets  the  Z-bit  accordingly,   then  clears   the

specified bit.

Examples:

Instruction              Before              After

BCLR.B #5,$345678        $345678             $345678 contains

contains $78        $58

Z-bit is 1

BCLR.L d0,d1             d0=0                d0=0

d1=$12345678        d1=$12345678

Z-bit is 0

Instruction:   BSET

Syntax:        BSET Dn,<ea> or BSET #,<ea>

Data sizes:    only byte when <ea> is an address.  Only long  when

<ea> is a data register.

Condition codes affected:

X    not affected

N    not affected

Z    set if the result is zero, cleared otherwise

V    not affected

C    not affected

Addressing modes allowed:

Destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#         (only when source is Dn)

Function: Bit test and SET. First tests the bit to be set and sets

the Z-bit accordingly, then sets the specified bit. This

instruction  and  the BCLR instruction can  be  used  as

alternatives to the TAS-instruction.

Examples:

Instruction              Before              After

BSET.B #5,$345678        $345678             $345678 contains

contains $78        $78

Z-bit is 1

BSET.L d0,d1             d0=0                d0=0

d1=$12345678        d1=$12345679

Z-bit is 0

Instruction:   BCHG

Syntax:        BCHG Dn,<ea> or BCHG #,<ea>

Data sizes:    only byte when <ea> is an address.  Only long  when

<ea> is a data register.

Condition codes affected:

X    not affected

N    not affected

Z    set if the result is zero, cleared otherwise

V    not affected

C    not affected

Addressing modes allowed:

Destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#         (only when source is Dn)

Function: Bit test and CHanGe.  First tests the bit to be  changed

and  sets  the  Z-bit  accordingly,   then  changes  the

specified bit.

Examples:

Instruction              Before              After

BCHG.B #5,$345678        $345678             $345678 contains

contains $78        $58

Z-bit is 1

BCHG.L d0,d1             d0=0                d0=0

d1=$12345678        d1=$12345679

Z-bit is 0

Binary Coded Decimal (BCD) instructions

To understand this instructionclass we must first know what a BCD-

digit is.  It is a representation of decimal digits in an array of

bytes  (array may be of length 1 or greater).  In every  byte  the

decimal  number  0  to 99 can be  represented.  This  is  done  as

follows:  a  byte can be divided into two four-bit  parts,  called

nibbles.  In every nibble,  one decimal digit is represented. This

implicates that the binary combination 1010 can never occur in BCD

representation,  since it isn't in the decimal range from 0 to  9.

The BCD-representation is especially convenient when printing such

a digit, since it doesn't take much calculation to convert it to a

printable character.  A disadvantage of the BCD-representation  is

that one doesn't use the full storage capacity of a byte or word.

The 68000 has three special BCD-artithmetic instructions.

Instruction:   ABCD

Syntax:        ABCD Dn,Dn or ABCD -(An),-(An)

Data sizes:    byte

Condition codes affected:

X    set by carry out of most significant

BCD-nibble, cleared otherwise

N    undefined

Z    set if the result is zero, cleared otherwise

V    undefined

C    same as X-bit

Function: Add two BCD-digits.  The predecremeting addressing  mode

has   been  provided  for  computations  with   multiple

precision  BCD-numbers.   This  implies  that  the  most

significant  BCD-numbers  must be stored  in  the  lower

memory addresses.

Examples:

Instruction              Before              After

ABCD.B d0,d1             d0=$53              d0=$53

d1=$32              d1=$85

Instruction:   SBCD

Syntax:        SBCD Dn,Dn or SBCD -(An),-(An)

Data sizes:    byte

Condition codes affected:

X    set by carry out of most significant

BCD-nibble, cleared otherwise

N    undefined

Z    set if the result is zero, cleared otherwise

V    undefined

C    same as X-bit

Function: Subtract two BCD-digits.

Examples:

Instruction              Before              After

ABCD.B d0,d1             d0=$53              d0=$53

d1=$32              d1=$21

Instruction:   NBCD

Syntax:        NBCD <ea>

Data sizes:    byte

Addressing modes allowed:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Condition codes affected:

X    set by borrow out of most significant

BCD-nibble, cleared otherwise

N    undefined

Z    set if the result is zero, cleared otherwise

V    undefined

C    same as X-bit

Function: Negate  a  BCD-number.  How  it functions  can  be  best

described with an example.  Let's negate $23.  The  NBCD

operation yields $77.  Now,  how did we get this result?

It's easy, just subtract $23 from $99 and you've got it.

Examples:

Instruction              Before              After

NBCD.B d0                d0=$43              d0=$56

This  is  the end of part seven.  Next time I will deal  with  all

program flow instruction, such as branches and jumps.

Originally published in ST NEWS Volume 2 Issue 6.