THE ATARI 130XE MEMORY UPGRADE

The Atari 130XE lends itself to memory increases quite

easily.  For those who may care the Freddie chip (Part #

CO61991-29) has been around for a while.  Remember the 1400

and 1450XL computers?  These machines used Freddie also.

Freddie is used for multiplexing the address's fo th RAM

chips and provides system timing.  The chip that does all

of the actual work in providing for the memory management

is the custom chip next to Freddie. (Part # CO25953)  This

chip takes it's input's from th 6520 PIA and from Antic for

CAS, RAS, etc.  The pin that we are concerned with is pin

14 (CASBANK).

.c

Technical Overview

Now for the good part.  If we redirect the output of

pin 14 of the CO25953 to another bank of 64K ram chips we

can, via software, select any one of 8 banks of 64K.  In

this configuration one would have a maximum of 589,815

bytes of memory.  The only disadvantage of this is the fact

that under these constraints the hardware becomes more

complex and the software to drive it does too.  I recommend

that for general purposes the 130XE be upgraded to a

maximum of 320K.  It gives the user sufficient "horsepower"

and yet at the same time the software does not become too

intense.  Now for the bad news.  To implement the 320K mod

one has to remove the internal basic ROM which really isn't

that bad because most users are using Basic XL/XE anyway.

The reason for this is quite good.  That bit is required

for the addressing of the extra bank(s) of memory.

Actually one could use the Self-test bit but that requires

additional hardware or a reburn of the O.S. ROM.  More on

that later.

.c

The Mod

What we will do is to wire up a 74LS138 to the 6520

PIA and to pin 14 of the CO25953.  The 74138 can be best

described as a routing switch.  The data that we want to

re-direct is presented at pin 4 of the 138 and the data

output is pins 15, 14, 13 and 12.  The pins that tell the

chip to what bank to directs it's output to are pins 1, 2,

and 3.  Now for the installation.

1)  Take 24 64K x 1 RAM chips and bend out pin 15 on

all of them.  Pin 15 is the CAS line.

2)  Solder these chips onto the existing RAM chips in

the 130.  Do not solder anything to pin 15 yet!  For ease

of servicing and soldering I staggered these new RAM chips

onto the existing ones.  I recommend this highly.  When you

has completed soldering in each new bank take a piece of

insulated wire and solder this wire to pin 15 of each new

RAM chip.  Wire wrap wire works nicely for this.  Continue

with this process until all 3 additional banks are wired.

3)  Just to the right of Freddie is an area to solder

in a 14 pin IC chip.  We will use this area to supply +5

and ground to our 74138.  Take a 74LS138 and bend out all

of the pins except pins 8 and 16.  Solder pins 8 and 16

into the holes of the unused chip area next to Freddie.

4)  Next to the RAM chips is the chip with the part #

of CO25953.  Behind the chip are two 33 ohm resistors.

(orange, orange, black, gold)  Unsolder the right-most lead

of the rear resistor.  (R111)  Solder a wire from the free

end of the resistor to PIN 15 of the 74138.  Solder another

wire from the land where the resistor used to go to pin 4

of the 74138.

5)  Solder two wires from pins 11 and 16 of the 6520

PIA chip (Part # CO14795-12) to pins 1 and 2 of the 74138.

Also ground pins 3 and 5 of the 74138.

6)  Solder the CAS line from each new bank of 64K to

pins 14, 13, and 12 of the 74138.

7)  Unsolder the Basic ROM chip from the board.  This

is the 24 pin chip that is located closest to the front of

the machine just to the left of the 555 timer IC.

Thats it.

.c

Conclusion and Tech notes:

To test each bank boot with DOS 2.5 with your basic

cartridge, POKE 5439,49, set the appropriate PIA port bit,

go to DOS and reformat D8.  Continue through all the banks

and check your directory.  If you write a file to one bank,

switch banks, and write another file, you will not lose

what you wrote to the first.  The real advantage is that

you can have your ramdisk and Basic XE too.  Around the

bbs's here in Chicago there is a file floating around

called RAMDISK2.  The source code is available on

compuserve.  By re-writing the routine you can have your

ramdisk invisible to Basic XE and/or double density.

I mentioned previously about using the Self-test bit

in the PIA port.  To use this bit you have to disconnct the

line on the PIA that runs to the PAL MMU and connect it to

a 556 timer that will enable the self-test input to the PAL

for about 3-5 seconds.  Half of the timer is used for

timing and the other half is used as an inverter.  The

reason for using a timer is that on boot-up the OS uses

some of the routines in the self test to check for valid

RAM and to determine RAM size.  I have done this and then

pulled the circuit out.  It was just too "messy" to suit

me.

One potential problem is the 555 timer used in the

130.  This chip is used for system reset timing.  If you

press your system reset key down and hold it there the

system should not reset.  If it does you will notice that

if you release it the system will reset again.  The reason

for this is because the 555 timer is putting out a spike

that the system sees as a valid reset.  To solve the

problem replace the chip.  It seems about 1 in 10 are bad.

That's it!  Hope you enjoy the mod.

Rich Andrews

Box 229-1 RR#7

Lockport, Il 60441