06601030301800

6L....T....T....T....T....T....T....T....T....T....T....T....T....R

The 65XE/320K upgrade, by Scott Peterson.

Copyright (C) July 1987, released to the public.

The following text file will explain howto upgrade your 65XE to 320K total

ram, 64K standard and 256K page mapped. It is 100% compatible with my older

130XE/320Kmodandabout99%compatiblewithaunmodified 130XE, you will, as

usual, lose the "video Enhanced" memory option that is rarely used on the

130XE. There is very little de-soldering in this upgrade,there is quite a

bit of piggy-backing and jumpers however.

TOOLS NEEDED;

Low wattage(15-25) soldering iron.

Vacuum de-soldering tool(Radio Shack PN#64-2098)

#2 Philip head screwdriver.

Small flat blade screwdriver and fine tip needle-nose pliers(Optional)

PARTS AND SUPPLIES;

Z1     74LS158

Z2     CO25953(Atari custom chip, stocked by Best Electronics)

Z3-Z10 41256(150ns.) 256K Ram Chips.

R1-R2  33 ohm resistor.

Also some 30 gauge wire(Radio Shack PN#278-501) and 1/8 inch heat shrink tubing

Remove the 65XE case and metal RF shield to get down to the mother board,set

it aside for the moment. Take your 8 256K ram chips(Z3-Z10) and bend up pin 15

on all of them, trim them back so only the "fat" part is left. Take the 65XE

mother board and locate the 8 64K ram chips that run along the left side of

the mother board(U9-U16). Solder Z3 thru Z10 on top of U8 thru U15, make sure

you get the little notches on the same end,things get a bit tight between U9

and U16 so becareful. When you get done take a piece of wire and solder a

jumper to all the pin 15's on Z3 thru Z10, you should have no excess. Flip

over the motherboard and solder a jumper to all the pin one's of U9 thru U16,

and leave about 1 foot of extra wire attached to pin one of U12. Next take

Z1(74LS158) and bend up all pins except 8 and 16, trim them back till just

the "fat" part is left. Locate U24(front right) on the 65XE mother board and

piggy back Z1 on top of it. Solder pin 1of Z1 to pin 1 of U24, and solder pin

16 of Z1 to pin 16 of U24. Now, take Z2(CO25953) and bend up all pin except 8

and 16, again trim back the rest so that just the fat part is left. Locate

U2(center) on the motherboard and piggyback Z2 on top of it, solder pin 1 of

Z2 to pin 1 of U2, and pin 16 of Z2 to pin 16 of U2. Take a 33 ohm resistor

and trim the leads back to about 1/4in. Bend one end at a 90 degree angle and

solder it to Z3,pin 15.Its the 256K ram chip piggy backed ontop of U12.

Solder a wire to the other end of the resistor, run it to Z2,pin 10. Take

another 33 ohm resistor and trim the leads to 1/4 inch and bend one end at 90

degree's and solder it to Z1,pin 4. Take the excess wire you left attached to

U12,pin 1 and snake it thru a hole in the mother board. Attach it to the other

end of the resistor you just soldered to Z1,pin 4. Locate R108(front,left) on

the mother board and desolder the end closest to the front of the computer.

Trim the end you desoldered back to about 1/4 inch, solder a wire to it and

use a piece of heat shrink tubing or tape to keep it from touching anything.

Run the wire over to Z2, and solder it to pin 11. Next locate U6(front,

center), and desolder pins 23 and 24. Carefully, with a sharp pointed object,

bend up pins 23 and 24 of U6, wiggling them with a pair of needle nose pliers

first helps to free them from the mother board, trim them back so that only

the fat part remains(NOTE:Ifyouarenotsureyoucanbendupthepinswithout

damagingU6,usea40pinsocketandcompletelydesolderU6fromthemotherboard).

Solder a wire into the hole that pin 23 just came from,connect it to Z2 pin 1.

Solder another wire into the hole that pin 24 camefrom and connect it to

Z2 pin 2. What follows is a list of solder connectionsto be made between chips.

chip/pin     chip/pin

---------------------

Z1/1    to   U17/30

Z1/2    to   U23/15

Z1/3    to   U23/16

Z1/15   to   Z1/8

Z2/6    to   U6/35

Z2/7    to   Z2/8

Z2/9    to   U17/26

Z2/12   to   U6/23

Z2/13   to   U6/24

Z2/14   to   Z2/16

Z2/15   to   U6/5

U23/12  to   Z2/3

U23/13  to   Z2/4

U23/14  to   Z2/5

----------------------

The control number's for location 54017(PORTB)

----------------------------------------------

BANK#       CONTROL#    HEX#

----------------------------------------------

BANK 1  -----------131 |83

BANK 2  -----------135 |87

BANK 3  -----------139 |8B

BANK 4  -----------143 |8F

BANK 5  -----------163 |A3

BANK 6  -----------167 |A7

BANK 7  -----------171 |AB

BANK 8  -----------175 |AF

BANK 9  -----------195 |C3

BANK 10 -----------199 |C7

BANK 11 -----------203 |CB

BANK 12 -----------207 |CF

BANK 13 -----------227 |E3<-----\

BANK 14 -----------231 |E7       \

BANK 15 -----------235 |EB       / 130XE BANKS.

BANK 16 -----------239 |EF<-----/

---------------------------------------------

Well, that about does it for now, I hope you enjoy your 320K 65XE. As far as

support software goes, I recommend Spartados 3.2D, it's one of the easierto

use DOS's for the upgrade. Also, MYDOS 4.1 or higher and Topdos 1.5+ supports

the 320K upgrades. Allthesoftwarewrittenforthe130/320Kmodwillrun

onthismod.(thetesters,Scopyprogram's,ect.).If you have a problem, drop me

a line on GENIE(S.PETERSON)or contactBest Electronic's at 408-243-6950 for

some help.

Good Luck

Scott Peterson