USING HOMETERM WITH THE AXLON RAMDISK AND MPP MODEM               You must have the following to use HomeTerm with the Axlon 128K     board and the MPP 1000E or C modems:          The file called HTMPPRD.XMO.  This is the HomePak AUTORUN.SYS file     with the Ramdisk initialization program and the HomeTerm MPP handler     already appended to it.  Since you will be using SMARTDOS, this     program should be named HTMPPRD.AR1.          The Axlon 128K board (of course).          The 8K bank-switching version of OMNIMON installed in your ATARI 800.      The Ramdisk init program will make a call to the OMNIMON Ramdisk     handler routines.          SMARTDOS 8.2d.  This is a single or double density DOS which is still     available from mail-order houses and such.  The reason you need this     DOS is that ATARI DOS 2.0 and 2.5 use up a little too much memory,     causing a conflict with the Hometerm MPP handler.  SMARTDOS should be     set (using the DEFAULT program supplied on the master disk) to 3     drives, 3 file I/O buffers, and one .AR? file.                    If you have all of the above, then you're ready to proceed.      First, format and write DOS files to a new disk with SMARTDOS (after     setting the options as described above).  Then copy HTMPPRD to this     disk and rename it to HTMPPRD.AR1.  Then you can copy the HOMETERM     program to this disk along with its .SET configuration file.  If you     want this disk to automatically boot to HomeTerm, rename HomeTerm to     HOMEMENU.OBJ.          If you've done everything correctly you should now have a     self-booting Ramdisk version of HomeTerm for the MPP modem.  When the     Ramdisk init program is executed it will format the Ramdisk as drive     3.  Then it will copy DUP.SYS to the Ramdisk and alter SMARTDOS so     that if you go to DOS, DUP.SYS will load from the Ramdisk.           What, you say?  Go to DOS?  From HomeTerm?  Well, with OMNIMON     many strange and wonderful things are possible!  Here's how to enter     DOS from HomeTerm.           First, go to the option menu screen of HomeTerm.  Be sure you're     finished using HomeTerm for the moment, and then enter OMNIMON by     pressing SELECT/RESET.  Execute the command "J 1780".  In the 8K     version of OMNIMON, you will need to switch to the upper bank when     this command is entered.  Flip the switch and press RETURN.  Then hold     START and press RETURN to exit OMNIMON.  You have to switch back to     the lower bank to execute this command.  So when your screen turns     upside down (OMNIMON's signal to flip the switch) go ahead and flip     it, then press START/RETURN again.  DUP.SYS will load from the Ramdisk     very quickly.  I find that using SMARTDOS to copy files I've     downloaded to the Ramdisk is much easier and faster than using the     HomeTerm copying option.               Enjoy!  -- Charles F. Johnson 75066,404                                                                                >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>