STROMBOLI SOFTWARE PROUDLY PRESENTS:

ATARI FILE SERVER, VERSION 1.0, REL FEB 1992

This document contains:

"READ THIS FIRST!"-section

1 Intro

1.1 Interface

1.2 Requirements

1.3 Getting started

1.4 The display of AFS

1.5 How to quit the program

2 The Getting-Quite-Technical section

2.1 About Atari diskdrives

2.2 Disk sectors

2.3 Operating systems

3 AFS techniques

3.1 How does it work?

3.2 File-format

4 Unsupported features

4.1 Big files

4.2 Renaming

4.3 Format

4.4 Overwriting

4.5 Verify

4.6 Subdirectories

4.7 Locking

4.8 Space free

5 References

6 Who and what's Stromboli Software?

-------------------------------------------------------------------------

"READ THIS FIRST!"

First of all, the author takes no responsibility for any data lost while

using the AFS-program. The program is intended for simple loading

(testing/running) of Atari programs from a PC-harddisk.

The AFS doesn't support through-printing and other transfer-methods to/from

the PC, it's just a simple (but easy to use) disk-emulating program.

Besides the AFS program running on a IBM PC compatible and a small interface

for the Atari 400/800/XL/XE, no other software than a normal DOS is needed.

The AFS software is PD. It's free for anyone to use. It should be distributed

in complete form (with this document accompanying the program) giving other

users the required information about the program.

-------------------------------------------------------------------------

1 Intro

The technological advances have made home-computers bigger and better. And

the mass-storages have moved from tapes and floppy disks to hard-disks and

CD-ROMs.

As many Atari 8-bit owners have moved to bigger (hard-disk based) PC-systems,

using the Atari with tapes/floppies seems outdated. This is where the disk-

emulating programs comes into the picture! With a simple serial-interface

between the Atari and the PC using the PC's hard-disk for storing data

becomes possible.

1.1 Interface

The AFS program requires some additional hardware to enable the Atari to get

linked to a PC. The software is written with the use of a existing interface

in mind, namely the SIO2PC interface [1]. This piece of hardware is needed to

connect the Atari SIO-port to the PC RS232-port. It essentially consists of a

voltage-converter and a tri-state enabler. The Atari signals are 0/5V while

the RS232-standard uses -12/12V. The conversion is easily  established with a

MAX232 or ICL232 converter, which converts 2 signals in each direction. The

tri-state device makes it possible for the PC to disconnect from the SIO-bus

and entering high-impedance mode for correct function of other devices

connected to the bus. If this doesn't mean anything to you, don't worry! No

technical knowledge is really needed to build or use the interface, just

follow the instructions carefully. See [1] for building-instructions.

1.2 Requirements

The AFS program is written in C++. It programs the serial-port hardware on

the PC (8250 UART) directly to accomplish the 19.2kbaud transferrate that the

SIO-port uses. The program has been developed and tested on my 12Mhz AT-clone

but should work on other ATs too. I don't know about XTs though.

The program is about 50kbytes in size and requires additional RAM for buf-

fering. When the Atari asks for data from AFS the data is fetched into the

buffer and then downloaded to the Atari. This means that the access time for

the Atari depends on the speed of the PC hard-disk. Usage of hard-disk cache

is recommended (I use the MS-DOS Smartdrive).

1.3 Getting started

The program AFS.EXE could be installed in a directory called C:\ATARI\ on

your PC harddisk. A subdirectory called C:\ATARI\GAMES\ might be created and

filled with some games plus the DOS.SYS, DUP.SYS and BOOT.SYS files included

in the AFS.ARC.

Type the following at the MS-DOS prompt to execute AFS

C:\ATARI\> AFS GAMES\ /2

If you get an error-message that AFS was unable to load bootsectors, check

the games subdirectory for the BOOT.SYS file, which has to be present.

The path given to AFS tells the program which directory to scan and simulate

a disk for. The optional switch /2 tells AFS to use COM2 port for communica-

tion with the Atari. Use the /1 switch for COM1. COM2 is default.

1.4 The display

Upon successful loading of the program the main-display is shown. It is

separated in 4 parts.

The upper part contains a three column display of the directory-contents.

Information about names, sizes and sectorallocation is shown. The display is

updated when the Atari changes the directory-sectors of AFS.

The lower part three separate boxes show information about the total

directory contents, the buffer and the current program action.

Bottom-left: The directory-information window shows the pathname, the total

number of files and their total size.

Bottom-middle: The buffer-info window tells you about the current file in

buffer, its size and the sector-position of the file on the imaginary Atari

hard-disk. The display changes as AFS loads-saves files to-from the buffer

memory.

Bottom-right: The action-information tells you the last command received and

executed by AFS. The commands shown are the same one-letter codes used on the

SIO-bus, see below.

P xxxxx = write sector xxxxx

R xxxxx = read sector xxxxx

S xxxxx = read drive status

W xxxxx = write sector xxxxx and verify

N xxxxx = read configuration

O xxxxx = write configuration

The action-window is continuously updated as the Atari sends commands to the

PC.

All displays on the PC-screen are really not needed for the function of AFS,

as all user input is entered at startup by switches from the command-line.

All manipulation of the disk naturally takes place from the Atari!

1.5 How to quit AFS

Pressing any key after the loading of AFS terminates the program and returns

the user to the command-line.

-------------------------------------------------------------------------

2 The Getting-Quite-Technical section

This section tells you how Atari diskdrives work, and how AFS simulates them.

All information is only provided for your pleasure and can be skipped!

2.1 About Atari diskdrives

Peripherals such as a diskdrive is normally attached to the Atari serial

in/output (SIO) port. A diskdrive compatible with a standard Atari OS should

accommodate certain commandframes communicated on the SIO-bus. A commandframe

consists of a device-number, command-identifier and some optional data. For

instance the Atari might send a read-command to a diskdrive for a certain

sector. The device being addressed should respond (perform handshake) and

return the desired data.

When the PC is connected to the Atari to perform as a diskdrive there must be

a program running on the PC to receive and interprete commands, perform

handshakes and transfer data from/to the PC harddisk. AFS is such a program.

2.2 Disksectors

A Atari-disk normally contains 720 sectors holding 128 or 256 bytes each. Not

all sectors are available for storage of files as the OS uses some to keep

track of the diskformat and contents of the disk. These sectors normally

include sector 1-3 for diskformat, sector 360 for a sector allocation table,

sectors 361-368 for root-directory contents. Other sectors are free for the

user.

2.4 Operating systems

Fortunately the user doesn't have to allocate and manually put data onto

sectors - the OS does the job for you. If you want to write a file to disk

the OS looks for vacant sectors in the allocation map. Then the OS divides

the file into blocks matching the sectorsize and writes the data onto the

vacant sectors of the disk. After this is done the allocation map is updated

and the filename as inserted into the directory-sector.

OS for the Atari include Atari DOS, MyDos and SpartaDOS. The AFS is developed

with/for MyDos. The dos.sys and dup.sys files included in the archive are

both MyDos 4.5 files. Read [2] for more information.

-------------------------------------------------------------------------

3 AFS techniques

This sections explains how the AFS program simulates a Atari diskdrive.

Another way is explained in [1].

3.1 How does it work?

AFS scans a specified directory on the PC harddisk and creates standard Atari

directory-sectors. It attaches some sectors to each file according to the

filesize. So, when the Atari asks for sector x AFS knows which file the Atari

wants to read a sector from. The AFS then loads the wanted file into a buffer

and sends the wanted part of the file to the Atari. The AFS allocates the

sectors of a file continuously which makes it much easier to decide which

file owns which sector.

As the directory-sectors are created by scanning a PC-directory the sector-

allocating map is updated. So, there are up-to-date sectors corresponding to

all map/directory sectors that can be sent to the Atari on request. The first

three sectors holding disk-format information are also created by AFS and

kept in a file called boot.sys.

Reading any legal sector on a disk can therefore be successfully simulated.

But it becomes tricky when we want to write to the disk. If you want to save

a file the OS finds vacant sectors in the map and starts writing to these

sectors. The AFS receives the write-command and wants to load the file

corresponding to the sectornumber into the buffer. But there is no file to

load as the file is about to be created and isn't entered into the directory-

sectors yet. The AFS handles the situation by allocating a big, temporary

buffer that stores the written sectors until the new filename/information is

entered into the directory-sector. Then AFS knows what filename is associated

with the written sectors and the file can be written to the PC hard-disk.

3.2 File-format

AFS uses 256 byte blocks (as all large disks) yielding a maximum of 253 bytes

actual data transferred. The other bytes are used to store the address of the

following sector and the number of bytes in the block. Files of this format

uses the 46h status-byte in Atari DOS. Other fileformats are not supported by

AFS. All transferred blocks (except the last in every file) are filled up to

maximum (253 bytes) .As sectors are allocated continuously, the next sector

is easily calculated: the current sector plus one.

-------------------------------------------------------------------------

4 Unsupported features

Certain features aren't supported by AFS for the time being. The reason for

the lacking implementation of these features are simple cost/benefit

considerations. For example, the support for subdirectories would require a

much more complex scanning of PC directories and creation of files

corresponding to subdirs. Reading/writing should be performed in the correct

MS-DOS directory, which is information that would have to be dealt with in

addition to the Atari-directory sectors.

4.1 Big files

Files greater than the available memory on the PC can't be created  (the

temporary buffer can't be bigger than actual free memory under MS-DOS).

4.2 Renaming

Renaming a file doesn't work properly. This is because the Atari OS only

reads a directory-sector, changes it and sends it back to AFS. The change in

the directory-sector isn't corresponded by actual change on the physical

hard-disk.

Solution: Copy the file to a new file with the desired filename and delete

the old file or exit AFS, use the MS-DOS rename-command and restart AFS.

4.3 Format

Formatting the diskdrive isn't included as a mean of security. If you really

want to format your hard-disk do this with the MS-DOS format-command. You

might also use a delete-command with wildcard *.* mask to delete all files in

a directory.

4.4 Overwriting

Overwriting existing files with bigger ones doesn't work properly. When the

new contents of a file is written over the old file everything works fine,

but when the Atari allocates new sectors and wants to continue writing the

file on these, AFS gets confused. According to AFS the new sectors have no

connection with the others as the directory still specifies the sectorrange

of the old file. Therefore AFS thinks a new file is being created and

allocates a temporary buffer instead. The solution is to avoid overwriting

large files by first deleting the old ones.

4.5 Verify

The Atari OS setting of verify on/off doesn't matter as AFS doesn't support

verifying files. This is too slow and there is a rather effective error-

trapping checksum included in every commandframe. The commandframe is

repeated until it's transferred without errors.

4.6 Subdirectories

Some OS for the Atari (such as MyDos) supports subdirectories that allows a

hierarchical file-structure. When you create a directory it corresponds to

making a file which contents is similar to the root-directory created by AFS.

The action of creating a subdirectory therefore only results in the creation

of a file and isn't corresponded by actual creation of a subdir on the PC

hard-disk. Creation of subdirs should be performed from MS-DOS and AFS should

be started with a path to the new subdir.

4.7 Locking

Locking files is only temporary as the locking-information only is kept as

long as AFS is running. Only the AFS directory-sectors and not the actual

files in the PC harddisk are affected.

4.8 Space free

The reported free sectors reported on the Atari is an imaginary number. It

doesn't correspond to the actual free space on the PC harddisk. Anyway, the

free space on the hard-disk probably isn't a big problem: The 64 file limit

pr directory in Atari DOS is a bigger one. Don't put more than 64 files in a

directory or you can't use them from the Atari. Create subdirs in MS-DOS and

distribute the files in these.

-------------------------------------------------------------------------

5 References

[1] SIO2PC Interface/Program by Nick Kennedy

[2] MyDos 4.50 User Guide by Charles Marslett & Robert Puff

Both references are available at the atari.archive.umich.edu FTP-site.

-------------------------------------------------------------------------

6 Who and what's Stromboli Software?

AFS is written by Soeren Aldenryd, who produces PD-software (labelled under

Stromboli Software) for the Atari and PC.  When he doesn't transform

powerfull PC's into file-servers for 10 year old Ataris, he's a computer-

science student at Aalborg University of Denmark (that's in Europe!).

Buggs and good ideas can be E-mailed to the author: soj@iesd.auc.dk

-------------------------------------------------------------------------