Simple docs for ptest.

Test Program only supports LPT1 (0x378 address)

The actual pokey emulator lets you set the printer address.

Options 1-4 play a middle c on that channel.  This is the same as a

SOUND(0,121,10,8) in Atari Basic.

Option 5 turns off all channels.

Option 7 will write the specified value to the 74HC574 (U2).

You can check the outputs on pins 12-19 if you want to verify if

the data output side of your circuit works.  You can also read these

outputs on the Pokey Chip itself. (38,39,40,2,3,4,5,6) D0-D7.

Option 8 will write the specified value to the other 74HC574 (U3).

You can check the outputs on pins 16-19.  These are the address lines

that go to the Pokey chip (pins 36-33).

Option 9 will strobe whatever you have on the data lines (U2) into the

adrress specified by the address lines (U3) into the pokey chip.

Option 10 will initilize the Pokey chip.  The test program does this

when you run it, but if you would power down your Pokey card and back on

without reruning the test program, you should use this command.  The

symptom of not having this option set is that you can only get sound on

channels 2 and 4.