XL/ST Interface

User Manual and Programmers Reference

This document describes the ST/XL Interface

Ton van Rosmalen, Brabantse Hardware Freaks

Translated by Ernest R. Schreurs

Release 1.0, february 26, 1989

Copyright 1988 The 'Brabantse Hardware Freaks',

P.O. Box 2095, 5202 CB Den Bosch, The Netherlands

Page 2

26 February 1989

CONTENTS

CHAPTER 1       OVERVIEW

1.1     INTRODUCTION . . . . . . . . . . . . . . . . . . . 1-1

1.2     DOCUMENT STRUCTURE . . . . . . . . . . . . . . . . 1-1

CHAPTER 2       REQUIREMENTS

2.1     INTRODUCTION . . . . . . . . . . . . . . . . . . . 2-1

2.2     HARDWARE . . . . . . . . . . . . . . . . . . . . . 2-1

2.3     SOFTWARE . . . . . . . . . . . . . . . . . . . . . 2-1

2.4     DEFINITIONS  . . . . . . . . . . . . . . . . . . . 2-2

2.5     DISCLAIMER . . . . . . . . . . . . . . . . . . . . 2-2

CHAPTER 3       CONNECTING THE CIRCUIT

3.1     INTRODUCTION . . . . . . . . . . . . . . . . . . . 3-1

3.2     HOOKING IT UP  . . . . . . . . . . . . . . . . . . 3-1

CHAPTER 4       LOADING THE ST PROGRAM

4.1     INTRODUCTION . . . . . . . . . . . . . . . . . . . 4-1

4.2     STARTING THE PROGRAM WITH AN ATARI 810/1050/551

DISK DRIVE . . . . . . . . . . . . . . . . . . . . 4-1

4.3     STARTING THE PROGRAM WITHOUT AN 8-BIT DISK DRIVE . 4-2

4.4     SUMMARY OF LOADING INSTRUCTIONS  . . . . . . . . . 4-2

CHAPTER 5       THE MENU OPTIONS AND THEIR MEANING

5.1     INTRODUCTION . . . . . . . . . . . . . . . . . . . 5-1

5.2     DESK . . . . . . . . . . . . . . . . . . . . . . . 5-1

5.3     DISK 1 - DISK 2  . . . . . . . . . . . . . . . . . 5-1

5.3.1     Directory  . . . . . . . . . . . . . . . . . . . 5-1

5.3.2     Save Disk  . . . . . . . . . . . . . . . . . . . 5-1

5.3.3     Load Disk  . . . . . . . . . . . . . . . . . . . 5-1

5.3.4     Format SD  . . . . . . . . . . . . . . . . . . . 5-2

5.3.5     Format ED  . . . . . . . . . . . . . . . . . . . 5-2

5.3.6     Set  . . . . . . . . . . . . . . . . . . . . . . 5-2

5.4     PRINTER  . . . . . . . . . . . . . . . . . . . . . 5-2

5.4.1     Direct . . . . . . . . . . . . . . . . . . . . . 5-2

5.4.2     Buffer . . . . . . . . . . . . . . . . . . . . . 5-2

5.4.3     List . . . . . . . . . . . . . . . . . . . . . . 5-2

5.4.4     Print Buffer . . . . . . . . . . . . . . . . . . 5-3

5.4.5     Show Buffer  . . . . . . . . . . . . . . . . . . 5-3

5.4.6     Clear Buffer . . . . . . . . . . . . . . . . . . 5-3

5.4.7     Save Buffer  . . . . . . . . . . . . . . . . . . 5-3

5.4.8     Set  . . . . . . . . . . . . . . . . . . . . . . 5-3

Page 3

26 February 1989

5.5     COPY . . . . . . . . . . . . . . . . . . . . . . . 5-3

5.5.1     Disk 1 To ST - Disk 2 To ST  . . . . . . . . . . 5-3

5.5.2     ST To Disk 1 - ST To Disk 2  . . . . . . . . . . 5-3

5.6     SETTINGS.  . . . . . . . . . . . . . . . . . . . . 5-4

5.6.1     128 Bps  . . . . . . . . . . . . . . . . . . . . 5-4

5.6.2     256 Bps  . . . . . . . . . . . . . . . . . . . . 5-4

5.6.3     Show Sector #  . . . . . . . . . . . . . . . . . 5-4

5.6.4     Clearw Disk 1 - Printer - Disk 2 - Status  . . . 5-4

5.7     END  . . . . . . . . . . . . . . . . . . . . . . . 5-4

5.7.1     * QUIT * . . . . . . . . . . . . . . . . . . . . 5-4

CHAPTER 6       THE INNER WORKINGS OF THE PROGRAM

6.1     INTRODUCTION . . . . . . . . . . . . . . . . . . . 6-1

6.2     WHAT YOU NEED IN ORDER TO MODIFY THE PROGRAM . . . 6-1

6.3     TECHNICAL INFORMATION  . . . . . . . . . . . . . . 6-1

6.4     THE PROGRAM  . . . . . . . . . . . . . . . . . . . 6-2

6.5     IF YOU HAVE PROBLEMS . . . . . . . . . . . . . . . 6-2

CHAPTER 1

OVERVIEW

1.1  INTRODUCTION

This document is the user manual for the XL/ST Interface,  a  hardware

project  published in the December 1988 issue of ATARI MAGAZINE, which

is the newsletter of the S.A.G.  Also  published  in  the  March  1989

issue of PSAN.

The XL/ST interface project was  developed  by  the  S.A.G.   Hardware

Freaks.    The   S.A.G.    is  a  Dutch  Atari  user  group.   It  has

approximately 12,000 members.  S.A.G.  is the  acronym  for  Stichting

Atari  Gebruikers.   This project is Copyright 1988, by the 'Brabantse

Hardware Freaks' and is released into the public domain.  This is  not

to be sold for profit.

1.2  DOCUMENT STRUCTURE

In this document, the following items will be discussed:

o  Chapter 1 is this overview

o  Chapter 2 explains what You need to make it work

o  Chapter 3 describes how to connect the circuit

o  Chapter 4 describes the loading of the ST program

o  Chapter 5 explains the menu options and their usage

o  Chapter 6 describes the inner workings of the program

CHAPTER 2

REQUIREMENTS

2.1  INTRODUCTION

This chapter list what You need in order to make this interface work.

2.2  HARDWARE

This interface should work with any  type  of  8-bit  Atari  computer.

This  includes  the  Atari 400, Atari 800, Atari XL and XE systems and

the Game System.  The 1200 XL users should  make  sure  that  a  power

source  of  5 volts is connected to the interface, since it is assumed

that the power can be drawn from the serial port on the XL.

This interface connects any of the  above  mentioned  systems  to  any

Atari  ST  system, or any system that supports data communication at a

speed of 19,200 Baud.

The two systems are connected using an interface.  This  interface  is

required, and it is documented in the description of the interface.

2.3  SOFTWARE

The software only runs  on  the  Atari  ST  systems,  using  GFA-Basic

version  2.0.   A  run-only  version  of  this  basic  interpreter  is

supplied.  Modifying the program requires that You have the  GFA-Basic

Version  2.0  program.   The program might be adapted to work on other

computer systems, and sufficient information is provided to enable the

user to convert the program if so desired.

REQUIREMENTS                                                  Page 2-2

DEFINITIONS                                           26 February 1989

2.4  DEFINITIONS

Throughout this document, the term XL appears.  This means  any  Atari

8-bit  computer  system,  and  is  therefore  not restricted to the XL

machines.  It  is  merely  a  convenient  abbreviation.   ST  is  used

whenever  any Atari ST system is meant.  These are trademarks of Atari

Corp.

2.5  DISCLAIMER

Over 100 of these interfaces have been successfully built as  of  this

writing.  S.A.G.  cannot be held responsible for any damages caused by

applying this interface.

CHAPTER 3

CONNECTING THE CIRCUIT

3.1  INTRODUCTION

If You have the interface and the software, You can  start  connecting

the  interface.   If  You  do not yet have the interface, refer to the

document describing the interface.  This manual  is  not  intended  to

teach  You  how  to  assemble  the interface.  We will assume that You

already mounted all components on the circuit board.

3.2  HOOKING IT UP

Make sure both the ST and the XL are switched off.  Take the interface

and  connect  the  DB-25  RS232 connector to the modem port of the ST.

Next, You will have to connect a standard XL I/O cable between the  XL

and  the  8-bit  port  of the interface.  Once You completed all these

connections You are about ready to switch on the circuit.

CHAPTER 4

LOADING THE ST PROGRAM

4.1  INTRODUCTION

Before loading the program, make sure the XL is switched off.   Insert

this  disk  into  the  ST  disk drive and switch on the ST.  After the

desktop  appears,  double  click  on  the  icon  for  the  file  named

XLST_INT.BAS which will automatically start up the RUN ONLY version of

GFA BASIC.  The program will be  loaded  and  executed  automatically.

This  is  because  the  GFABASRO.PRG  file  has  been  installed as an

application.  If You boot the ST using a different  DESKTOP.INF  file,

the  program  must be loaded through running the GFABASRO.PRG program.

If everything works properly, You will see four  windows  and  a  menu

bar.   What  You  are  about  to  do  next  depends  on Your XL system

configuration.  If You do not have an 8-bit disk drive connected, some

of the following section might not apply to You.

4.2  STARTING THE PROGRAM WITH AN ATARI 810/1050/551 DISK DRIVE

Your system is now ready to be booted.  Make sure that DRIVE 1 in  the

ST  program is NOT active.  Insert a disk with DOS on it in Your 8-bit

disk drive, either an 810, 1050, XF551 or whatever You've got.  Switch

on Your XL.  It will start booting and the DOS-menu will appear on the

screen.  If You end up in BASIC,  type  DOS  and  press  return,  this

should  get  You  to  the  DOS-MENU.   If anything out of the ordinary

happens, check all connections again.  Now You can perform all actions

You  would  be  able  to  do  on  a 'normal' disk drive.  For instance

formatting, copying files, deleting files and so forth.  Note that the

ST  will  only be disk 2, and the printer.  Superdos users should take

special care, since Superdos tests for the  presence  of  Happy-drives

while  initializing.   Since  this  is not the case for the ST, the ST

will report an error in the status window.  This can also happen  with

the  format  command.   You can simply ignore these errors.  Note that

You can change the device number of Your 8-bit drive  by  setting  the

switches  appropriately,  and  then  the ST can be used as drive 1, as

documented below for booting without an 8-bit disk drive.

LOADING THE ST PROGRAM                                        Page 4-2

STARTING THE PROGRAM WITHOUT AN 8-BIT DISK DRIVE      26 February 1989

4.3  STARTING THE PROGRAM WITHOUT AN 8-BIT DISK DRIVE

If You do not own an 8-bit disk drive, or no longer own one,  this  is

no  problem.   Before  switching  on  the  XL,  You  must first do the

following action.  On the ST, choose from the  DISK  1  menu  the  SET

option.   Now drive 1 will become active on the ST, but it will not be

formatted.  Now You can use the LOAD DISK option from the DISK 1  menu

to  load  the  ATARI DOS 2.5 from the system disk.  Once You have done

this, You can turn on your XL.  DOS 2.5 will be  loaded  from  the  ST

into  Your  XL, and You will be able to work with it as usual.  If You

also want to transfer other programs and files to  the  ST,  You  will

have  to  borrow  an  8-bit  disk drive from a friend to be able to do

this.  Like mentioned earlier, Superdos  might  cause  error  messages

that can be ignored, for the reasons mentioned above.

4.4  SUMMARY OF LOADING INSTRUCTIONS

A quick re-cap:

0  Activate the XLST_INT.BAS program by double clicking on  it's

icon

0  If not booting from the 8-bit disk, load DOS 2.5 from the  ST

disk

0  If booting from the 8-bit disk  is  desired,  make  sure  the

8-bit  disk  drive is on, and that DISK 1 on the ST is set to

inactive.

0  Boot Your XL like normal.

0  Take it from there, based on what You'd normally do.

CHAPTER 5

THE MENU OPTIONS AND THEIR MEANING

5.1  INTRODUCTION

This chapter will explain all the menu options that can  be  selected.

It  briefly  explains  how  to  use them and why You would want to use

them.

5.2  DESK

Atari ST/XL Interface.  A copyright message from the Hardware  Freaks.

Accessory's can be selected if present.

5.3  DISK 1 - DISK 2

5.3.1  Directory

A directory listing  for  drive  1  or  drive  2  is  printed  in  the

appropriate window.

5.3.2  Save Disk

The entire contents of the disk drive is written as a file to  the  ST

disk.  The file name has a .DSK extender.

5.3.3  Load Disk

A file, previously stored with the Save Disk option,  is  loaded  into

the memory of the ST.  To the XL, this will become the contents of the

'disk'.

THE MENU OPTIONS AND THEIR MEANING                            Page 5-2

DISK 1 - DISK 2                                       26 February 1989

5.3.4  Format SD

The memory contents are erased.   It  is  better  to  use  the  format

command on the DOS menu.

5.3.5  Format ED

The same as Format SD, except now the 'disk' is  formatted  in  DOS  3

format.

5.3.6  Set

With this function, the drive can be made active  or  inactive.   This

means  that  to  the  XL,  the  drive  can  me  made to exist or to be

non-existent, causing the afore mentioned options from the Drive  menu

to be disabled.

5.4  PRINTER

5.4.1  Direct

The ST sends all printer  output  from  the  XL  directly  to  the  ST

printer.   Note that the printer must be attached to the parallel port

on the ST, since the serial port  is  used  by  the  interface.   This

feature relieves You of the burden of switching printers all the time.

You also no longer need the 850 interface module,  or  the  centronics

printer interface for the XL.

5.4.2  Buffer

The ST stores all printer output in a buffer.  Functions to  use  this

buffer are described in more detail later.

5.4.3  List

The ST prints all printer output to the printer window.

Combinations of these 3 functions  are  also  possible,  i.e.   Buffer

List.  The status line of the window will show the selected options.

THE MENU OPTIONS AND THEIR MEANING                            Page 5-3

PRINTER                                               26 February 1989

5.4.4  Print Buffer

All data stored in the printer buffer is sent to the printer.

5.4.5  Show Buffer

All data stored in the printer buffer is printed to the screen.

5.4.6  Clear Buffer

The entire printer buffer is erased.  All information stored in it  is

irreversible lost.

5.4.7  Save Buffer

The contents of the printer buffer is written to  the  ST  disk  as  a

file.   This  enables You to process the data before sending it to the

printer.

5.4.8  Set

The printer functions can be enabled/disabled.  All buffer  operations

remain selectable.  All other operations are disabled.

5.5  COPY

5.5.1  Disk 1 To ST - Disk 2 To ST

This provides for storing individual files from the 8-bit  disk  drive

as  ST files.  Every file You want to copy must be entered separately.

For text files, there is an option to translate  the  ASCII  character

$9B (155 - Carriage return and line feed on the XL), to the values $0D

(13 - Carriage return) and $0A (10 - Line feed).

5.5.2  ST To Disk 1 - ST To Disk 2

This provides for copying files from Your ST disk to  an  8-bit  disk,

under  the  condition  that  the  file  is not larger than the storage

capacity of the 8-bit disk.  There is no option to translate the codes

for carriage return and line feed.

THE MENU OPTIONS AND THEIR MEANING                            Page 5-4

SETTINGS.                                             26 February 1989

5.6  SETTINGS.

5.6.1  128 Bps

This function allows You to set the  sector  size  to  128  bytes  per

sector  (Bps).   If  the  sector size was previously 256 Bps, You must

first acknowledge that it is O.K.  to change the sector size.  This is

because  this  causes  all  data  to  be lost.  If this sector size is

already selected, this is indicated by a check mark in  front  of  the

menu item.

5.6.2  256 Bps

This is similar to the 128 Bps option.  However the sector  size  will

now  be  256  Bps.  If this sector size is already selected, this will

also be denoted by a check mark.

5.6.3  Show Sector #

If this option is selected, also indicated by a check mark,  a  status

message  will  be  displayed  at  the  top of the window.  This can be

'Getting ..  ($..)', or 'Sending ..  ($..)'.  The first message  means

that  the specified sector number is being sent from the XL to the ST,

meaning that the XL is writing a sector.   The  second  message  means

that  the specified sector is being transmitted from the ST to the XL,

meaning that the XL is reading a sector.  The number in parenthesis is

the number of the sector in a hexadecimal notation.

5.6.4  Clearw Disk 1 - Printer - Disk 2 - Status

After clicking on this item, the corresponding window will be cleared.

All data stored in it disappears.

5.7  END

5.7.1  * QUIT *

After acknowledging the  message  in  the  alertbox,  the  program  is

terminated and You will be returned to the desktop.

CHAPTER 6

THE INNER WORKINGS OF THE PROGRAM

6.1  INTRODUCTION

The following section of the manual is intended for the ones  planning

on  improving  or  expanding the program.  If You do not plan on doing

this, You should now have sufficient information to get started.

6.2  WHAT YOU NEED IN ORDER TO MODIFY THE PROGRAM

The program is written in  GFA-BASIC  v2.0.   Therefore  the  Run-only

version  of  this  programming language is supplied on this disk.  The

program is stored in the SAVE format.  For modifying the program,  You

will  need  the normal GFABASIC.PRG program.  An experienced GFA-BASIC

programmer will have no problem modifying the program.  You might only

run  into  trouble  getting  information from the modem port.  More on

this later.

6.3  TECHNICAL INFORMATION

First some technical information about the XL and the ST.   The  modem

port  on the ST works at a voltage level of 12 Volts.  The serial port

of the XL to which this interface is connected operates at  a  voltage

level  of 5 Volts.  The Integrated Circuit used in this interface is a

voltage level converter.  This means all it does is converting the  12

Volts signals from the ST to 5 Volts signals for the XL, and the other

way round of course.  Note that the COMMAND line on the 8-bit port  is

not  used  in  this  interface.   Therefore  it is hard to distinguish

between command frames and data frames.  However this usually does not

appear to be a problem.

THE INNER WORKINGS OF THE PROGRAM                             Page 6-2

THE PROGRAM                                           26 February 1989

6.4  THE PROGRAM

Then now for the program itself.  First off, a couple  of  arrays  are

being  DIMensioned.  All XL data is stored in a string.  Then all menu

items are stored in a string.  The menu bar for  them  is  built  from

this  string.   Then some more variables are initialized, and all four

windows are opened.  From now on the program waits  for  either  input

from  the modem port, i.e.  commands from the XL, or for the selecting

of any of the menu items.  What happens after a  menu  item  has  been

selected, well, that can easily be figured out, possibly with the help

of the GFA-BASIC manual.

The input from the modem port works slightly different than  what  one

would  expect,  because this requires some knowledge about the way the

XL  works.   The  XL  sends  commands  on  the  serial  port  to   its

peripherals.  These commands are always intended for a certain device.

However all connected devices receive this information.  The  software

in  these  devices  determines whether the command is intended for it.

If this is not the case,  it  will  ignore  all  data  regarding  this

command.

The program works in a similar way, It  sits  waiting  for  a  command

containing  its  device number.  This is the first byte in the command

block.  The program looks for commands for drive 1, drive  2,  or  the

printer.   However  the program will only respond to these commands if

the status line shows an active status.  If the ASCII value for '1' is

sent,  the  program knows that drive 1 is intended.  It will therefore

process all the information for it.  This is the reason that if a real

8-bit  disk  drive is connected as drive 1, the program's drive 1 must

not be active.  Otherwise the ST would interfere with the  real  8-bit

disk drive's communication.

The program processes all commands for  the  activated  devices.   The

comments  in  the program should allow You to understand what is going

on.

6.5  IF YOU HAVE PROBLEMS

After reading this document, You  should  be  able  to  do  with  this

interface  whatever  You want to do with it.  If, however, it does not

work properly, You might find some help at Your local user group.   If

You still experience problems, You can also write the Hardware Freaks.

We hope You will enjoy using this interface

Ton van Rosmalen

Brabantse Hardware Freaks

#