RULE) FOR DISK DRIVE USAGE - REMINDERS

Never insert or remove disks when the disk drive is active (when the BUSY light is on).

Never turn the disk drive on or off while a diskette is inserted This only applies to 810 disk drives as properly working 1050 disk drives have circuitry that prevents glitching the disk should the power 6e turned on v: off while the disk is inserted, whereas the 810 may glitch (erase) data on your disk if you do this The 810 can glitch the disk even if it is write protected!

Always place a write protect ;a . a disk, remove this only when you intend to write on the disk.

NEW RULES FOR THE HAPPY ENHANCEMENT

If your ENHANCEMENT equipped 810 ?::c: has an add on write protect disable switch this switch must have a position where the normal write protect detection mechanism is !nabIed, and the switch should be left in that position at all times on your ENHANCED drive, except when you wish to write on the backside or on a wrote protected disk, for the ENHANCEMENT to work correctly and pass diagnostics. The write protect system en the 810 drive is used by the ENHANCEMENT to sense the change in disks. This dogs not apply to the 105Q drive as the change in disks is sensed 6p a separate switch.

When fir-` powered on the ENHANCED disk drive is in the buffered reading "FAST MODE". DOS formatted disks are usually safe to use m the "FAST MODE"

Purchased software items on diskettes should always be protected with a write protect tab unless they require writing to the disk It is not a good idea to use your disk drive to write to purchased software disks even though the program intends for you to do this, if possible only write to your backup copy. acme copy guarded disks may not work with the "FAST READING MODE". In fact some copy guardod disk programs may misty misinterpret "FAST MODE" as a pirated copy and may try to erase your disk or backup.

P?tore trying to read in 3 purchased software item for the first time on an ENHANCED disk drive, be sure there is a write protect tab on it However, some disks map net work correctly with a write protect tab installed Always backup !here disks before removing the write protect tab, or select UNHAPPY MODE

before running them

!TAFFY !:0 MPU TERS has found that nearly 100 of alt purchased software that does not hoot correctly in the "FAST READING MODE" will boot correctly when the SLOW/FAST switch on the 1050 CONTROLLER OPTION or the user installed 810 SLOW/FAST switch is in the 'SLOW position. The same can be said of backup disks that were made with "FORCED SLOW MOLE" selected. The need is seldom to actually have to select "UNHAPPY" mode if you have the SLOW/FAST switch

FAST, SLOW, and UNHAPPY MODES

The controlling program inside the HAPPY ENHANCEMENT hardware has built in buffered fast reading and buffered fast write with verify. The HAPPY ENHANCEMENT also has other special functions besides buffered writing and reading that set it apart from unenhanced Atari 810 and 1050 disk drives.

When the .disk drive is first turned on the fast reading is enabled and fast writing is disabled, also all special HAFFY functions are allowed whereby the drive is sand to be in HAPPY mode. The SLOW/FAST snitch option can be used to select both slow and fast reading and writing. Without the SLOW/FAST switch, fast writing can only be selected through software commands. The SLOW/FAST switch does not

a'fe

, -f HAPPY mode It should be noted that some conditions can cause the disk drive to automatically switch into the SLOW mode of reading and writing independent of the SLOW/FAST switch or programming. One such condition is when a disk created by the HAPPY BACKUP with PDB mode, PDBC mode=YES, or FORCE SLOW MODE=YES has been inserted in the drive.

Whether the ENHANCED drive is in slow or fast mode, it can also 6e in NAPPY mode. In HAPPY mode the special functions are allowed which are utilized by NAPPY COMPUTERS software to perform such tasks as disk backup, booting of PDB and COMPACTED disks, disk load tracing, and HAPPY diagnostics. In UNHAPPY mode, the drive can do none of these things, the drive is equivalent to a standard disk drive. Also in UNHAPPY mode only slow unbuffered writing and reading are available.

Once UNHAPPY mode is selected through the drive programming options, the HAPPY mode cannot 6e restored without turning the disk drive off and on The disk drive should be allowed a "POWER OFF" time of 10 seconds before turning the power back on, for proper operation. A disk drive in the HAPPY mode will always be listed in the HAPPY DRIVES = display of the first menu of the WARP SPEED SOFTWARE disk. It should be realized that slow disk drive operations of reading and writing can 6e selected independently of UNHAPPY mode, whereas UNHAPPY mode provides the maximum compatibility with the standard disk drive.

If you have trouble booting a purchased software disk, try booting it after selecting UNHAPPY mode. Note that all backup disks created by the PDB backup mode of the HAPPY BACKUP program require that the drive be in HAPPY mode when they are first hooted.

WARP SPEED SOFTWARE MENU PROGRAM

PURPOSE

The WARP SPEED SOFTWARE MENU PROGRAM provides access to most other programs on the WARP SPEED SOFTWARE disk. The menu program is also used to enable the tracer, display tracer information, select drive programming options, and display information about the current operating environment in the personal computer. The menu program is always the first program loaded into the personal computer when the WARP SPEED SOFTWARE disk is first booted DOS accessible programs on the WARP SPEED software disk are not accessible through the WARP SPEED SOFTWARE menu program. Access to the DOS accessible programs in the DOS directory of the WARP SPEED SOFTWARE disk is explained in other sections of this manual

The menu program is the self booting portion of the WARP SPEED software disk. When first booted the menu program unlocks and recovers the tracer information from any ENHANCED disk drive on which the tracer was previously enabled. The menu program is always booted in drive one in the system, drive one may be any ATARI serial bus, single density compatible disk drive Drive one does not have to be an ENHANCED drive The menu program allows the user to select a function by pressing the number on the personal computer keyboard that corresponds to the number of that function displayed next to the name of that function on the vide. screen.

QUILT IN FUNCTIONS

Once the menu program is booted, and the menu displayed, access is immediately provided to those functions which are built into the menu program without reaccessing the WARP SPEED SOFTWARE DISK. These include the following.

00) OTHER FEATURES (message printed only)

?> DRIVE OPTIONS

?) ENABLE TRACER

8) DISPLAY TRACER INFORMATION

A description of each of the above built in functions is provided in this section on the menu program

FUNCTIONS THAT ARE LOADED BY TIDE MENU PROGRAM

Other functions available through the WARP SPEED SOFTWARE menu require accessing a disk drive and loading that program into memory The menu program will automatically, provide the setup needed by the selected program, load that program into the person:) computer memory, and transfer control to that program Once the requested program begins t^ load into memo;-y it is not possible to return to the menu program without rebooting the personal computer from the WARP SPEED SOFTWARE DISK Those functions which require accessing a disk drive to load include the following

2? DIAGN OSTIC

3) SECTOR COPIER

4) HAPPY BACKUP PROGRAM

5) HAPPY C 0111-1 ACTOR PROGRAM

;? MULTI DRIVE PROGRAM

Separate sections describing the purpose, function, and usage of each of these programs are included in these instructions

MENU PROGRAM - INFORMATION DISPLAYED

The menu of the self booting menu program displays information about the environment in which the WARP SPEED SOFTWARE is operating This information includes the personal computer memory rise, the RAMDISK type (only same are detected and supported), which disk drives connected to the personal computer are enabled as HAPPY ENHANCED drives, and whether or not the tracer information has been recovered from the most recent disk execution trace This information is of importance to the WARP SPEED SOFTWARE user. The meaning of these items is discussed below.

RAMDISK & TYPE

The WARP SPUD SOFTWARE's SECTOR COPIER PROGRAM is at present the only program in the WARP SPEED SOFTWARE disk the supports RAMDISKS. At present only RAMDISKS that appear to the computer as AXLON TYPE, or 13OXE type are detected, displayed, and utilized The RA MDISK status on the menu display will tell you (if you have a RAMDISK) whether or not you have the select switch in the correct position (if there 15 3 Select switch? You must re-boot the WARE SPEED SOFTWARE if you change a RAMDISK select switch There are many varieties of RA MDISK schemes now being installed by Atari computer users. Most fall into the category of AXLON type, of 13OXE type Even though your RAM DISK may have mote, the AXLO N type is considered to have 128K and the 130XE type is considered to have 64K, of RA MDIS K area. HAPPY CO MPUTERS -annot dire-_tly support every commercial or homebrew RA M DISK. There are instructions available ""e technical informati on) on how to modify our programs to use your RAMDIS K.

PERSONAL COMPUTER MEMORY SIZE

The personal computer memory size displayed by the menu program is the total amount of contiguous random access memory available for holding programs and data. The amount of memory is usually 16K, 29K, 32K, 90K, 98K, or 52K A MOSAIC RAMDISK may be displayed as 52K, with no RAMDISK shown. If you have an XL or XE computer it is not necessary to hold down the option key while loading the WARP SPEED SOFTWARE. The WARP SPEED SOFTWARE menu program will automatically detect and disable the built in BASIC so you will have 98K Although your computer may have more than 98K, only the contiguous unsegmented address space of the computer is shown.

Each WARP SPEED SOFTWARE program uses the first part of the personal computer's memory to hold the program itself. The remainder of the personal computer's contiguous memory is used 6y the program for storing the disk data being copied. With one disk drive it is especially desirable to have as much personal computer memory as possible available to the WARP SPEED SOFTWARE programs, so that fewer insertions of source and destination disk are needed.

Certain plug in or switch selected options in the personal computer cause less contiguous memory to be available For example any cartridge plugged m may cause 8K, 16K, or even 20K less memory to be available in .a 98K or 52K computer. Paying attention to the personal computer memory size will alert you to the need to select switch options, plug in memory boards and or remove cartridges that may be temporarily stealing memory

HAPPY DRIVES - WHICH DRIVE NUMBERS

The menu program displays the drive numbers of all ENHANCED disk drives which are plugged into the personal computer and turned on HAPPY 1050 drive numbers are displayed in inverse video while HAPPY 810 drive numbers are not. An ENHANCED disk drive which has been placed in the "UNHAPPY" mode does not appear to the system as a HAPPY drive. The menu program will "re-detect" which drives are in HAPPY mode each time you press the RESET button (may be Tabled SYSTEM RESET). Older HAPPY 810 drives that do mat have the rev 7 upgrade will not be displayed as a HAPPY drive in the list; they cannot be used with rev 7 software

Most of the programs on the WARP SPEED SOFTWARE disk will only work with HAPPY ENHANCED disc drives that are recognized by the menu program. Always press RESET after changing the ENHANCED drive configuration once the menu program is loaded. Having the menu program recognise the proper HAPPY drive configuration allows the WARP SPEED SOFTWARE to select the source and destination drives.

HAVE TRACER INFORMATION - YES or NO

This display tells you whether or not the "T RAC KS USED" information of the program 'traced' has been recovered during the load of the WARP SPEED MENU PROGRAM. See the section on ENABLE TRACER, COPY PER TRACER, and COMPACT program for further information. If "YES" then the tracer information will be available to other WARP SPEED SOFTWARE programs when they .are loaded.

LOADING THE WARP SPEED SOFTWARE

The HAPPY COMPUTERS WARP SPEED SOFTWARE programs disk is a self booting disk that is booted in disk drive ? Drive I may be any ATARI serial bus connected, single density compatible disk drive. Boot this disk as with any other self booting disk. It is suggested that you remove any plug in cartridges before booting. Do not use any "TRANSLATOR" with the WARP SPEED SOFTWARE.

Once the WARP SPEED SOFTWARE disk is booted, the menu of functions is displayed. Examine the information at the bottom of the screen as per the previous page before proceeding to select a function. Press the number corresponding to the function you wish to execute as displayed on the menu.

DRIVE OPTIONS

Pressing 1 from the WARP SPEED SOFTWARE menu selects the enhanced DRIVE programming OPTIONS. If you have more than 1 HAPPY drive you will be asked to select which drive you which to program. The list of options for that drive will appear, select the number corresponding to the function you desire. Pressing 0 allows you to return to the main menu with no programming change. None of the DRIVE programming OPTIONS remain in a 1050 HAPPY when the HAPPY BACKUP, COMPACTOR, or MULTI DRIVE programs are used as these programs initialise the 1050 HAPPY before it is used 6y them.

SET UNHAPPY MODE

Pressing 1 causes the selected drive to be programmed into the UNHAPPY mode. From that point on, until the drive power is turned off and on, that disk drive will function as a standard disk drive. In the case of the 1050 HAPPY drive the true double density features are still available, which differentiates the UNHAPPY 1050 from the standard 1050. The UNHAPPY drive will be slow (unenhanced speed) and unable to perform any of the HAPPY functions. It will not be recognized by the system as a HAPPY drive.

To reverse this effect you should first remove any disk that may be inserted if this is an 810 drive (not needed on 1050?, turn the disk drive off, wait 10 seconds, and then turn the disk drive back on. An easy way to set UNHAPPY mode in drive I is to hold the "1" key down while the WARP SPEED SOFTWARE is booting. Once the MENU program is ready, holding 1 down will select drive options, drive 1, and UNHAPPY mode, without the need to look at each menu

DRIVE OPTIONS CONTINUED - INIT DRIVE

Pressing 2 causes the drive to execute its 'power on reset' routine. The drive is then in a state as if it's power switch had just been turned en. Any other programming previously done to the drive is lost. This includes the CHANGE DRIVE # that may have been programmed (see below). It is necessary that you press SYSTEM RESET if you want the MENU program to correctly detect which drive number that drive is in this case. Also in this case you may wind up with duplicate drive number assignment, see below.

CHANCE DRIVE

This command allows the user to change the physical drive number without changing the 'drive select switches at tie tear of the drive. Although direct programming (see technical info) would allow you to select any drive number there is a limit here of selecting drive 1-4 as the new drive number. The change takes place immediately on the 810 HAPPY. The change does not take place on the 1050 HAPPY until the door is opened and closed, or lust closed if it is already ,open. If the change you request results in more than one drive having the same select number (duplicate drive number assignment) you will not be able to properly access either drive until the situation is remedied, i.e.. turn one of the drives off, or turn it off and on to reset this drive number change. Always press RESET so the MENU program knows which drives ate HAPPY

SET NEW OFF DELAY

This command allows modifying the drive's motor turn off delay. The motor turn off delay time is the time from when the last read or write operation accessed the disk (there is a difference between slow and fast modes) to the time when the disk drive turns off. It therefore also controls the fast writing delay time (see below). For most applications, the built in delay time with no programming option change, provides the best drive utilisation. The actual resultant delay time may be affected by activity in other disk drives or peripherals connected to the personal computer. Delay values from A-Z are allowed. These values have substantially different effect on 810 and 1050 HAPPY drives. The shortest delay for the 810 is the setting "Z", the shortest delay for the 1050 is a setting "A". The 810 has a much longer maximum delay at the ether end of the settings.

SET TO U.S. EMU - 1050 HAPPY DRIVES ONLY

If year 1050 HAPPY has a rev 2 or higher rom (see ENHANCEMENT DIAGNOSTIC), selecting SET T 0 U.S. EMU mode only enables fast writing. Rev 2 rom 1050 HAPPYs are the only type we have shipped for over a year. See SET TO FAST WRITE. For those with rev I rom, this option programs the rev 1 1050 to have the faster ULTRA SPEED (trademark of IC D) commands. This allows slightly higher speed than the rev 1 rom 1050 HAPPY would otherwise have with the SECTOR COPIER, WARP SPEED DOS, WARP SPEED DOS XL, and TOP DOS 1 5 Selecting SST TO U. S EMU will make either rev HAPPY 1050 work with some versions of SPARTA DOS (by IC D) which is not available from HAPPY COMPUTERS. Those versions of SPARTA DOS that work with the HAPPY 1050 are not fully compatible unless either FAST WRITE or SET T.0 U.S. EMU is selected for rev 2 rom 1050 HAPPYs, or SET TO U S. EMU is selected for rev 1 1050 NAPPIES.

SET WRITE ENABLE/PROTECT - 1050 HAPPYs with CONTROLLER ONLY

For 1050 HAPPYs with CONTROLLER only. These options utilize the CONTROLLER hardware to control the disk drive write protect as indicated. For a CONTROLLER with the factory installed jumpers unchanged, these commands take priority over the position of the CONTROLLER write protect switch. For example, once the option for write enable is selected, no matter what position the switch is in you can still write to the disk. If you select write protect and then select UNHAPPY mode, which prohibits any further drive programming, then no mortal influence can make your disk drive write to a disk until its power is reset!

SET TO FAST WRITE

The 1050 ENHANCEMENT and 810 rev 7 equipped ENHANCEMENT have one of the most exciting innovations in "smart" disk drive technology. The buffered fast write with verify built into these NAPPY products allows much faster write with verify than the unenhanced drive. FAST writing is not normally enabled when the disk drive is first turned on. If you have a SLOW/FAST switch option you can enable the fast writing by first having the switch in the SLOW position when the drive is first turned on, or when you init the drive per that option, and then moving the switch to the FAST position. This is especially useful when using utilities which have to boot in the SLOW reading mode with the switch in the SLOW position; but then after booting you can operate on your data disks using both fast reading and writing by moving the switch to the FAST position.

With fast writing enabled both write without verify and write with verify commands are translated into fast buffered write with verify, since the buffered write with verify can be faster than write without verify! It should be noted that the SECTOR COPIER, WARP SPEED DOS XL, WARP SPEED DOS, and TOP DOS 1.5 all automatically turn on the HAPPY's fast writing The below mentioned cautions therefore also apply to these programs. Once the fast writing is enabled it stays enabled until the disk drive is reset, although it may be temporarily disabled by the SLOW/FAST switch, by special information on the disk inserted, or by specific programming commands issued to the drive. Although the HAPPY BACKUP, COMPACTOR, and MULTIDRIVE use a fast write with verify, this is in dependant of the fast write selectable by the user, and the restrictions listed below do not apply. (continued)

c

J

DRIVE OPTIONS - SET TO FAST WRITE CONTINUED

It is important to understand that the HAPPY drive does not actually write the data to the disk at the moment the computer sends the data and write command to the drive. Rather, it usually collects the data and writes and verifies usually more than 1 sector sometime later. Although the computer may tell you that your writing operation is finished, the disk drive may not be finished doing the job. Although there are other ways to force the disk drive to finish, the only easy way you can be certain is to waft until the disk drive busy light goes out before removing the disk just written to. The 'fast writing delay time' (same as motor turn off delay time) causes the actual write/verify of data to the disk surface to occur just before the disk drive spindle motor turns off. Again as the first rule in these instructions states, you should not remove the disk while the busy light is on.

The drive may not be able to complete the write with verify correctly. This can be caused by a bad diskette, a hardware malfunction, or most likely by the user removing the diskette before the busy light went out. In this case the disk drive may have already told the personal computer's software it was finished writing Since the computer cannot be told at this later time that an error occurred, the disk drive may indicate this error condition by flashing the busy light on and oft several times per second. The disk drive will continue to do this and not respond (give errors only) to any subsequent request from the personal computer until the user takes appropriate action to recognize this error condition.

The easiest way to tell the disk drive that you acknowledge this error condition is to open the door and remove the disk (if you haven't already), insert the same or another disk, and close the door. This does not correct the write/verify error, it merely informs the disk drive that you are aware of the problem. It is still your responsibility to take steps to save whatever your data is that was not written/verified correctly. You may simply have to re-do the operation using a good disk. Note that if you removed the disk while the drive was writing it is possible (but very unlikely) that you glitched other nearby areas on the disk.

Since the 1050 disk drive has a separate switch on the door latch, it can always tell when you have opened and closed the door and it will always be able to display the flashing busy light (error indication). The 810 disk drive has no door sensing switch. The only way the 810 can sense the change in disks is through the write protect sensor. With an 810 it is possible, but unlikely, that you removed the disk while writing was occurring, that the disk drive began to display the flashing busy light error condition, and then saw the change in the write protect sensor while you were removing the disk and thought (?, no I guess even HAPPYs don't think) you had responded to the error condition. In this case with the 810 you missed the error display! DON'T TAKE THE DISK 0 UT TILL THE BUSY LIGHT GOES OFF. If you are impatient change the "off delay".

Note that if the last operation requested to the drive is not a write there is no worry in removing the disk before the busy light goes out. So if you display the directory, or open a file for reading after all your writing is finished, there can be no loss of data if you take the disk out before the busy light goes out.

Note that the SECTOR COPIER and WARP SPEED DOS in the older revision 5 WARP SPEED SOFTWARE did use fast write with verify but the personal computer had more control of the drive doing this, and the personal computer never told you it was done before the write/verify was actually completed. The rev 7 versions of these programs for both 8:0 and :050 are not like that.

The above section was not meant to scare you! If your drive is working okay (we provide diagnostics) and you use good quality disks, and YOU LEAVE THE DISK IN UNTIL THE BUSY LIGHT GOES OUT, you should never have a problem. We have been using the buffered fast write with verify in our office at HAPPY COMPUTERS for over two years and with our mix of 10 drives both 810s and 1050s have never had a failure except when we took the disk out too soon, and were always able to recover from this easily. This whole deal of taking the disk out too soon is why we made fast write/verify an option you had to select rather than having it enabled when the drive is first turned on

For 1050 HAPPYs, if you have a rev 2 ROM (see previous page SET I TO U.S. EMU) you should be able to

work with SpartaDOS at high speed. You must enable FAST WRITING before writing with SpartaDOS. The

WARP SPEED DOS, WARP SPEED DOS XL, TOP DOS 1.5, and SECTOR COPIER all automatically enable fast

writing. This must remain enabled when writing to the disk with these programs or the data written will

be incorrect!

THE TRACER FUNCTION - PURPOSE

The TRACER function of the ENHANCEMENT is a valuable tool. The TRACER determines which tracks on a disk are used by a particular self booting disk program. The data resulting from the TRACE of a self booting disk is used by the COPY PER TRACER command in the HAPPY BACKUP programs, and by the COMPACT. disk command in the HAPPY COMPACTOR program. The COMPACTOR program would not be possible without the TRACER function. Copying disks with the TRACER will usually take less time than copying the whole disk, especially if more than 1 copy is made, since less than the total 40 tracks of the disk are copied

With disks that are formatted by DOS, the directory may be examined using one of the many disk utility tools to determine which sectors on the disk are used. With a purchased software item which does not have a directory, the software supplier will usually not provide the software user with a list of which tracks on the disk are used The tracer function of the WARP SPEED SOFTWARE provides the capability to determine the tracks used without a disk directory or published information from the software supplier.

6

THE TRACER continued - FUNCTION

The TRACER programs the HAPPY ENHANCED disk drive to observe and record the loading process of a self booting disk while that disk is loading. The tracer does N 0 T examine the program itself to determine the loading sequence Instead the tracer is a program which runs in an ENHANCED disk drive that sees and remembers each read and write operation that occurred while a program was loading.

If the TRACER is enabled on a NAPPY drive in the drive 1 position, that drive will be temporarily in the UN HAPPY & SLOW READING mode and will. without the personal computer knowing it, remember the number of each track that was accessed while a program is loaded. If the TRACER is enabled on a HAPPY drive which is not in the drive 1 position, !when drive 1 is not a HAPPY ENHANCED drive) that drive will watch the ATARI serial 110 bus and remember the number of each track that the personal computer told drive 1 to access, without drive 1 or the personal computer knowing this. While the TRACER is enabled on a HAPPY drive which is not d.-ive 1, that drive will not respond to the personal computer at all until the TRACER is unlocked

The HAPPY drive programmed in the TRACER mode will continue to TRACE the loading process of disks until the WARP SPEED SOFTWARE menu program is re-booted, which unlocks the TRACER which was enabled en any HAPPY drive and returns the "TRACKS USED" information to the personal computer for use by the various WARP SPEED SOFTWARE programs.

WHICH DISKS FAKE SENSE TO TRACE

The loading of any self booting disk may be traced. The trace of some self booting programs does not make sense. Programs which do not make sense to trace are those which re-access the disk drive while the program is being executed. These programs include such items as graphic adventures, which re-access the disk for each new screen displayed, or games which re-access the disk after finishing a particular level of play

come self booting disks may first hoot and display a title screen, and then continue booting. At some point the program may never need to re-access the disk This type of program may be successfully traced, provided you have executed it to the point where the disk is no longer accessed.

The most precise determination for the user to select whether or net to use the tracer on a particular self booting disk is as follows. it you =an cause the self booting program to access all data on its disk that it will ever need, then z trace of that program execution will be complete and will contain all of the needed "TRACKS USED" information if it cannot be determined that a self booting disk has accessed all of the tracks it will use, do not use the TRACER

Disks that can only be copied by the HAPPY BACKUP using PDB mode copying do not make sense to trace, since the PDB file already contains all the information needed by the HAPPY BACKUP to efficiently make a backup of that disk.

USING ",'HE TRACER FUNCTION

The TRACER function of the WARP SPEED f-OE'TWARE is automatic From the menu of the WARP SPEED SOFTWARE press the number ^ to select the ENABLE TRACER FUNCTION You must have a HAPPY ENHANCED drive turned on, connected ca the personal computer, and selected as any drive from 1 to 9 for the TRACER to be enabled. Alter pressing 7 the menu program will display the drive number on which the tracer was enabled This will he the fast ENHAN CED drive in the system, starting with drive 1. If there is some problem encountered by the menu program when you request ENABLE TRACER, this problem will be reported and the tracer will not he enabled

Once the menu program reports that the tracer has been enabled, you should remove the WARP SPEED SOFTWARE disk from drive one and turn off the personal computer. Do not turn off the ENHANCED disk drive that the trace: Was enabled on. If you do turn off the disk drive on which the tracer was enabled, the tracer function will not occur and any tracer data will be lost

Insert the self booting disk you wish to trace into drive i and follow the booting procedure needed for that self booting disk as specified by the documentation for that self booting disk. Some programs may require a cartridge, such as BASK , to boot correctly Other programs demand that no cartridge be inserted, or require that you hold down the option button on the newer XL computers which have built in BASIC when turning on the persona! computer, so that the built in BASIC is disabled.

You must execute the program you are tracing to the point where you are sure that all the data that program uses on the disk has been accessed at least once for the "TRACKS USED" tracer information to be correct. If you cannot be sure of this, the TRACER should not be used with this disk. If this program is a gam e, you may play the game and even try to beat your old score. This is just for fun and riot required. No matter what you do with the program the ENHANCED disk drive that has the tracer enabled will remember all the !racks that the program accessed.

Once the self booting disk yen are tracing has completely accessed all tracks it uses, you may then turn off the personal computer De not turn off the ENHANCED disk drive on which the tracer was enabled

Re-boot the WARP SPEED SOFTWARE disk menu program. If the tracer was enabled on drive 1, drive I will return to the FAST MODE automatically. If the tracer was enabled on an ENHANCED drive which was not drive 1 , that drive will now respond to the computer as normal. (continued)

DRIVE OPTIONS - SET TO FAST WRITE CONTINUED

It is important to understand that the HAPPY drive does not actually write the data to the disk at the moment the computer sends the data and write command to the drive. Rather, it usually collects the data and writes and verifies usually more than 1 sector sometime later. Although the computer may tell you that your writing operation is finished, the disk drive may not be finished doing the job. Although there are other ways to force the disk drive to finish, the only easy way you can be certain is to welt until the disk drive busy light goes out before removing the disk just written to. The 'fast writing delay time' (same as motor turn off delay time) causes the actual write/verify of data to the disk surface to occur just before the disk drive spindle motor turns off. Again as the first rule in these instructions states, you should not remove the disk while the busy light is on.

The drive may nut be able to complete the write with verify correctly. This can 6e caused by a bad diskette, a hardware malfunction, or roost likely by the user removing the diskette before the busy light went out. In this case the disk drive may have already told the personal computer's software it was finished writing Since the computer cannot be told at this later time that an error occurred, the disk drive may indicate this error condition by flashing the busy light on and oft several times per second. The disk drive will continue to do this and not respond (give errors only) to any subsequent request from the personal computer until the user takes appropriate action to recognise this error condition.

The easiest way to tell the disk drive that you acknowledge this error condition is to open the door and remove the disk (if you haven't already), insert the same or another disk, and close the door. This does not correct the write/verify error, it merely informs the disk drive that you are aware of the problem. It is still your responsibility to take steps to save whatever your data is that was not written/verified correctly. You may simply have to re-do the operation using a good disk. Note that if you removed the disk while the drive was writing it is possible (but very unlikely) that you glitched other nearby areas on the disk.

Since the 1050 disk drive has a separate switch on the door latch, it can always tell when you have opened and closed the door and it will always be able to display the flashing busy light (error indication). The 810 disk drive has no door sensing switch. The only way the 810 can sense the change in disks is through the write protect sensor. With an 810 it is possible, but unlikely, that you removed the disk while writing was occuring, that the disk drive began to display the flashing busy light error condition, and then saw the change in the write protect sensor while you were removing the disk and thought (?, no I guess even HAPPYs don't think) you had responded to the error condition. In this case with the 810 you missed the error display! DON'T TAKE THE DISK 0 UT TILL THE BUSY LIGHT GOES OFF. 1f you are impatient change the "off delay".

?late !hat if the last operation requested to the drive is not a write there is no worry in removing the disk before the busy light goes out. So if you display the directory, or open a file for reading after all your writing is finished, there can be no loss of data if you take the disk out before the busy light goes out.

Note that the SECTOR COPIER and WARP SPEED DOS in the older revision 5 WARP SPEED SOFTWARE did use fast write with verify but the personal computer had more control of the drive doing this, and the personal computer never told you it was done before the write/verify was actually completed. The rev 7 versions of these programs for both 810 and 1050 are not like that.

The above section was net meant to scare you! If your drive is working okay (we provide diagnostics) and you use good quality disks and YOU LEAVE THE DISK IN UNTIL THE BUSY LIGHT GOES OUT, you should never have a problem. We have been using the buffered fast write with verify in our office at HAPPY COMPUTERS for over two years and with our mix of 10 drives both 810s and 1050s have never had a failure except when we took the disk out too soon, and were always able to recover from this easily. This whole deal of taking the disk out too soon is why we made fast write/verify an option you had to select rather than having it enabled when the drive is first turned on

For 1050 HAPPYs, if you have a rev 2 ROM (see previous page SET I TO U.S. EMU) you should be able to

work with SpartaDOS at high speed. You must enable FAST WRITING before writing with SpartaDOS. The

WARP SPEED DOS, WARP SPEED DOS XL, TOP DOS 1.5, and SECTOR COPIER all automatically enable fast

writing. This must remain enabled when writing to the disk with these programs or the data written will

be incorrect!

THE TRACER FUNCTION - PURPOSE

The TRACER function of the ENHANCEMENT is a valuable tool. The TRACER determines which tracks on a disk are used by a particular self booting disk program. The data resulting from the TRACE of a self booting disk is used by the COPY PER TRACER command in the HAPPY BACKUP programs, and by the COMPACT. disk command in the HAPPY COMPACTOR program. The COMPACTOR program would not be possible without the TRACER function. Copying disks with the TRACER will usually take less time than copying the whole disk, especially if more than 1 copy is made, since less than the total 40 tracks of the disk are copied

With disks that are formatted by DOS, the directory may be examined using one of the many disk utility tools to determine which sectors on the disk are used. With a purchased software item which does not have a directory, the software supplier will usually not provide the software user with a list of which tracks on the disk are used The tracer function of the WARP SPEED SOFTWARE provides the capability to determine the tracks used without a disk directory or published information from the software supplier.

USING THE TRACER FUNCTION continued

When a trace has been successfully completed, the menu of the WARP SPEED SOFTWARE will show the following: HAVE TRACER DATA = YES. The tracer information may then 6e displayed by the DISPLAY TRACE function of the menu program, or may be used in the COPY PER TRACER command of the NAPPY BACKUP programs, or the COMPACT program command of the the HAPPY COMPACTOR program. If you turn off the personal computer the tracer's "TRACKS USED" information will be lost, and will not be available the next time you boot the WARP SPEED SOFTWARE disk, unless you re-enable the tracer and trace that self hooting program again.

DISPLAY TRACE FUNCTION

When the menu program reports "HAVE TRACER DATA = YES", you may display the tracks used information of the trace of a self booting disk by pressing the number 8 key. The track numbers for the 40 tracks numbered in decimal from D to 39 will appear on the video screen and next to each track number will be the word "YES" or the word "NO". A "YES" means that the self booting program just traced did access that track, a "NO" means it didn't Also displayed at the bottom of the screen is the number of tracks used by the program m decimal, which is the number of tracks which had "YES".

If you write down the name of the program lust traced, the tracks which were used, and the total number of tracks used and save this information in your records you will never have to use the tracer function again on this program, The 14APPY BACKUP programs and the NAPPY COMPACTOR program allow you to enter the "TRACKS USED" information directly without tracing the program again. Knowing the quantity of tracks used by a program is useful when you proceed to compact your library with the NAPPY COMPACTOR in an efficient manner, more on this later

DIAGNOS71C PROGRAM - PURPOSE

The DIAGNOSTIC on the WARP SPEED SOFTWARE disc checks the HAPPY ENHANCEMENT hardware, in addition to some of the standard hardware of your disk drive. It does not test all functions and elements of your disk drive, this is performed by qualified service technicians. Please instruct your service technician to use the UNHAPPY MODE program option before running any DIAGNOSTIC other than the WARP

PEED SOFTWARE DIAGNOSTIC, since the fast reading may interfere with other diagnostics. Note that the 1050 disk drive's Atari diagnostic will not function with the 1050 HAPPY; the HAPPY 1050 ENHANCEMENT must be removed and the original chips installed if Atari's 1050 diagnostic is to be run. The WARP SPEED SOFTWARE DIAGNOSTIC is intended to check the HAPPY ENHANCEMENT hardware and other hardware in your disk drive which is critical to proper ENHANCED disk drive performance.

The DIAGNOSTIC program= the ENHANCED disk drive to perform certain tests and measurements and reports to the user any failures. The DIAGNOSTIC tests may only be used with ENHANCED disk drives. The user selects which test to run and the use: must interpret the results. Due to the interaction of the computer and all peripherals connected, it is possible that a diagnostic test failure can be caused by a problem not in that disk drive.

RUNNING THE DIAGNOSTIC

Boot the WARP SPEED SOFTWARE disk as described in these instructions. Be sure that the WARP SPEED SOFTWARE disk is still in drive 1 and press the number 2 to select the diagnostic. The diagnostic will be leaded from the disk and take control of the system You cannot go back to the menu unless you re-hoot the WARP SPEED SOFTW ARE disk. As soon as the DIAGNOSTIC finishes loading, you will see the DIAGNOSTIC title screen. The WARP SPEED SOFTWARE MENU and DIAGNOSTIC program do not require an ENHANCED drive to load. If for some reason your ENHANCED drive is not reading correctly, you can still load the diagnostic on an unenhanced drive

1f you press reset while the diagnostic is running, the title screen will again be displayed and the program will restart. Sometimes if you press reset while a drive test is in progress, and then try to select that drive to run a test, you will get an error message This is caused by the fact that the drive is still running the last test, and is not reset by the RESET key on the computer. If you wait a few seconds the problem will go away when the drive finishes the test and again responds to the personal computer.

To start the DIAGNOSTIC program enter the drive number (1-8) that you wish to run the diagnostic on. Only ENHANCED drives in the HAPPY mode can 6e selected, or an error message will result. The HAPPY drive type, i.e. 810 or 1050, and a list of the tests/m easurem ant features will then be presented. You select the feature you want by pressing the letter on the keyboard which is shown to the left of that feature It is required that you initialise the drive after running any diagnostics to remove any programming that may remain in the drive, prior to using the drive for any other purpose. A description of each test and interpretation of the results is presented below. The DIAGNOSTIC will always stop at the first failure with an error message Do not use a disk drive which fails the diagnostic. Nave it filed immediately. While a test is running there may or may not be sound from the speaker on the TV. Sometimes when there is no sound fur several seconds, the disk drive is still working, running the test. Always wait for the disk de ve to finish and for the computer to report the test results.

DIAGNOSTIC continued - ENHANCEMENT DIAGNOSTIC

After selecting the ENHANCEMENT DIAGNOSTIC you have the option of running either a single cycle or a continuous cycle test. Enter 0 to run a single cycle test, or enter 1 to run a continuous cycle test. The continuous test mode is provided in the ENHANCEMENT DIAGNOSTIC to allow repeated testing without the need for the user to restart the test. The user can for example test the drive all night. If a failure occurs at any time, the testing stops and the test that failed, and the continuous cycle test count will be displayed on the screen.

On 810 H APPYs only, if you enter 0 for the single cycle test, and if all of the first tests .are passing, you will be instructed to insert and remove any disk from the disk drive for the WRITE PROTECT test. The disk you are using need not be write protected. If you have a special WRITE PROTECT DEFEAT switch on your disk drive be sure that this switch is disabled. Then press start. D 0 NOT TURN OFF THE DISK DRIVE white doing this test. Failure of this test is not a failure of the ENHANCEMENT. This write protect test verifies that the 810 HAPPY can sense when the disk is changed. See also the SWITCH OPTION TEST. The continuous cycle test mode does not perform the WRITE PROTECT test.

On 1050 HAPPYs only, the ENHANCEMENT diagnostic displays the ruin revision indirectly. After the ROM TEST runs, look at the letters in the word "PASS". If you see all large letters you have a rev 1 ruin. If you have the letters "Pass", a large P and small a s s, (don't laugh) you have a rev 2 ruin.

HI SPEED XFER DIAGNOSTIC

HAPPY COMPUTERS believes that people should have to wait as short a period of time as practical for their computer system to do its job. HAPPY COMPUTERS uses data transfer rates which exceed the standard rates used by the Atari peripherals to do this. If the HI SPEED XFER is failing the HAPPY ENHANCEMENT circuit board is NEVER at fault since the data transfers occur in circuitry outside the HAPPY board.

Most hi speed xfer failures can be corrected inside the personal computer, even though it may appear that a particular disk drive is more sensitive Aside from the instructions printed when this test is selected, the fix that most often solves the problem is to remove all capacitors inside the personal computer that connect the SERIAL OUTPUT DATA (pin 5 of the I/0 connector) to ground. The 130YE is especially a problem as it has 2 capacitors, whereas older computers have only 1. The extra capacitor inside the 13OXE may be C304, or may be directly soldered between pin 5 of the I/O connector and ground on the circuit board. Even though all your drives may pass the hi speed xfer test you may have a problem running the MULTI DRIVE program, where the hi speed transfers are marginally failing and the MULTI DRIVE is operating more slowly than it should. Use an ohmmeter to verify which capacitors connect pin 5 to ground. Removing these specific capacitors should not affect any other operation of the computer system More information on the correction of this problem is available on request, and may have been included with your ENHANCEMENT shipment; also see technical info

RPM MEASUREMENT

The RPM measurement program determines the rotations per minute of the drive spindle motor. HAPPY COMPUTERS' RPM measurement program is the most accurate and efficient measurement available for your ENHANCED drive Most other RPM programs have to measure many rimes and take an average. HAPPY's program can accurately measure the drive speed to within plus or minus 0.5 RPM in less than 1/2 second. Also, most other RPM programs require that the HAPPY drive be placed into the SLOW or UNHAPPY mode before they will work

The RPM measurement requires that a formatted (initialised) disk be inserted in the drive, then you press start Use any DOS to create a single density formatted disk for this test. No files need to be placed nn the disk Any DOS formatted disk can be used.

If you do not understand this section don't worry about it. Older versions of the DIAGNOSTIC's RPM measurement used track 00 sector 1 for the measurement. The rev 7 RPM measurement uses track 01 sector 1. This means that with rev 7 you can switch between the READ/WRITE TEST and the RPM measurement without changing disks The older RPM measurement could not work with a disk that was just used with the READ/WRITE test, since the READIWRITE test erases track 00.

After inserting the formatted disk and pressing start, the DIAGNOSTIC displays the RPM measurement. While the measurement is running the value for the RPM measured is continuously displayed and updated about once every half second. There is no measurement averaging. The RPM measurement continues to run until you press RESET on the personal computer or some error condition occurs; which properly working and turned on hardware should not incur. If the drive cannot measure 'the RPM it will indicate "RPM = ERROR can't measure". This is an indication that the drive cannot read from the disk. This can be caused by a bad disk, bad disk format, disk drive hardware failure, or drive speed too far out of range. The measurement updating is slightly slower when the drive cannot measure.

The maximum speed indication for a properly adjusted disk drive is "RPM = 289.5 is just fine.". The minimum speed indication is "RPM = 285 0 is just fine." 0 ur diagnostic allows the range of 285.0 to 289.5 as a passing range of measurement. The ideal setting is 286.5 to 287.5 RPM although any reading in the allowed range is okay. Any reading outside that range will cause a message "is too slow!" or "is too fast" as the case may be. It is permissible that a single measurement printed may once in a great while be outside the allowed range, so long as most of the measurements are inside the allowed range. Large variations in readings are an indication that there is soma problem such as with the disk disk drive, or the disk used for measurement. Getting an "ERROR can't measure", often or even

occasionally, is also an indication of a problem		(continued)

DIAGNOSTIC - RPM MEASUREMENT continued

The drive spindle motor control circuitry should be able to compensate for different loading conditions while still maintaining the correct speed range. Different disks have different dynamic friction coefficients, and place different loads on the motor. You can test for this without changing disks, by lightly applying continuous light pressure on the rotating spindle with your finger. Do not touch the disk surface. At the moment you apply this additional friction, the measurement may drop below the allowed range but should come back to within the allowed range within Z measurements. You can hear each measurement as a beep in the TV speaker At the moment you release your finger from the spindle, the RPM may jump above the allowed range, but should come back to within the allowed range within 2 measurements, Do not apply excessive friction, use only a light force!

SWITCH OPTION TEST

This first part of this test will appear only when testing a 1050 HAPPY. This is the CONTROLLER diagnostic. If you do not have the CONTROLLER skip that portion. If you do have the CONTROLLER follow the instructions indicated. The second portion of this test is the "USER VERIFY MODE". The screen is continuously updated, showing the write protect status and SLOW/FAST switch mode. If you do not have a SLOW/FAST switch option, you should ignore that indication on the screen. Even if you do not have a write protect control switch, you can verify that your write protect sensors and circuitry are working by inserting and removing disks. Press RESET to end the continuous display.

HEAD POSITIONING

This is not a test by itself. This feature is used to move the read/write head in the disk drive to a particular track, with the spindle motor turned on. No disk reading or writing operation occurs, but the read circuitry in the drive should be working. The primary use for this feature is to verify or adjust the read/write head parameters such as azimuth, track zero sensor, track 0 stop, amplitude, and alignment. All of these require special instruments, tools, disks, and procedures which are beyond the scope of this document There is no way for you to check these parameters without these additional tools and procedures. Head alignment problems are to be suspected when you have trouble reading or writing disks that are written on other drives. It is also possible that the other drive is out of alignment. This feature may also be used with a special cleaning disk, since the motor is turning and you have control over the head position, while no data is needed on the disk.

While the test is running, you press SELECT to move the head to higher track numbers (step in), or press OPTION to move to lower track numbers (step out) The screen continuously displays the track number in hexadecimal and decimal, where it has instructed the head to be. The program has no way of verifying that the head is actually at that position If you press OPTION (step out) when the head is already at track 00, the drive will do a restore to track 00 operation. On the 1050 HAPPY this will cause the head to first move 10 tracks inward, and then step outward until the track 00 sensor is detected. On the 810 HAPPY which has no track 00 sensor, this will cause the head to step 44 times outward toward track 00, which should result in the head hitting the track 00 stop, and resting at track 00; also this causes the grinding noise. Press START to stop this test. If you press RESET, the test will stop but the drive motor may continue running for many seconds

READ / WRITE TEST

The READ I WRITE TEST (RWT) is a fast quality assurance test for a large portion of the disk drive's circuitry. The read write test verifies that the drive can format, read, write, and verify data on the disk surface without errors at three track positions. The 1050 RWT also tests 3 densities. The RWT will erase any data that is on the disk used for the test. He sure to use a high quality disk, and be sure disk writing is enabled. The disk drive speed should be properly adjusted using the RPM MEASUREMENT program before running the RWT. The RWT will fail if the drive speed is too fast.

The RWT reports the density currently being tested (1050 only), and the track number being tested. The track number is shown in hexadecimal. The decimal track numbers tested (not displayed in decimal) are track 0 which is the outermost track on the disk, track 20 which is near the middle of the disk, and track 39 which is the innermost track used on the disk.

If there is an error, a test number is reported. If failure occurs at test number 01 this probably means the disk is write protected. Failure of other test numbers can be caused 6y a variety of reasons including hardware failure, improper adjustment, or a bad disk was used for the test. Failure usually never means the NAPPY ENHANCEMENT is defective, most likely something else is wrong!

The RWT is not very forgiving. If even a single bit error occurs an error will result. It is possible that a properly working disk drive can on occasion fail the read write test, say once in 50 tries, even though soma disk drives may never fail the test even after 1000 tries. Under normal conditions the disk drive can detect and recover from a 'soft' error. A 'soft' error is a reading error that can be eliminated simply by reading the disk again Soft errors that occur during the RWT result in test failure, failures may even occur on each test and the disk drive may appear to work fine otherwise. This is an indication that the disk drive has a marginal problem, and is right on the border of not working. This is especially true when the test fails at track 39 (27 hexadecimal), which is the inner most track.

The R WT does not check the disk drive R/W head alignment, and does not check the head positioning motor. If the head positioning motor were disconnected, and the head moved anywhere on the disk, the RWT would probably still pass if it was passing before

1 0

SEC70R COPIER PURPOSE

The SECTOR COPIED PROGRAM (SCP) is used to duplicate standard format disks Copyguarded disks should be baked-up with the HAFFY BACKUP PROGRAM, not the SCP. The SCP may be used with both EN14ANCED and non-E1IHANCED disk drives. The SCP may be used with 1 or 2 disk drives. A maximum of Z source and destination disk insertions is needed to copy all 720 sectors of a single density disk with a 98K computer (and no cartridges inserted) The AXLO N and 130XE TYPE RAMDISKs are supported by the SCP. Since only one Source disk insertion is needed in single density with a RAMDISK, you can make as many copies as you need

The main advantage of the SCP over other sector copier programs is the speed when ENHANCED disk drives are used Maximum um speed of duplication is realized when both source and destination drives are ENHA14CED disk drives With ENHANCED drives, the SCP uses fully buffered reading and write with verification, along with higher serial bus transfer speeds to accomplish this speed improvement.

FUNCTION

The SC° makes a sector by sector copy of the source disk to the destination disk. The sector copier functions by first filling all available memo" with the data read from the source, and then writes with verification this data to the destination disk. The SCP does not examine the directory (if any) on the disk. Instead the user instructs the SC P precisely which sectors are to be copied. Errors encountered during reading or write verification are reported and copying terminates in this case.

USING THE SECTOR COPIER

Since the SCP uses the built in buffered fast write with verify on HAPPY ENHANCED drives, it is important to nut remove the destination disk from the drive after writing, before the busy light goes out! Be sure that the WARP SPEED SOFTWARE disk is in drive one and press the number 3 on the keyboard. Once the SCP is loaded you have the option to display some introductory instructions. Examine these instructions for a quick reminder as to the purpose and use of the SECTOR COPIER. After the introductory instructions the SELECTION MENU will appear The selection menu allows one to change the operating para meters of the S CP from the default conditions.

Once you leave the selection menu and begin copying the source disk, you cannot return to this menu without rebooting the program, unless you have a RAMDISK. Without a RAMDISK, the memory used to hold the selection menu portion of the program is overwritten by the data on the source disk. With a RA MDISK the selection menu may always be re-entered.

SELECTION MENU DISPLAY AND COMMANDS

The selection mew displays the current environment of the SCP. Values will either be the default state from initial loading, or the last state which was selected. The default values only occur when the SCP is first loaded ''he only displayed parameter which may not be changed by user keyboard entry is the "RAM DISK TYPE" parameter. If you have a RAM DISK and the selection menu says "NONE", then either you have the 129K selection switch in the wrong position something in the computer's memory system is defective, or that RAM DIS K is not presently supported by the SECTOR COPIER. The SECTOR COPIER can be user modified to support other RAMDISKS, and sizes of RAM DIS KS; see technical info

The starting an.; ending Sector number for copying ire &splayed ;n both decimal and hexadecimal for your convenience The default starting sector is 1; the default ending sector is M AX. M AX means that the highest sector num ber -.f the density type selects: or detected will be used. Or single and true double density this is 70 decimal: for ;00 medium (ENHANCED) density this is 1040 decimal. The default starting and ending sector numbers result in the entire disk being copied. Change the start of end se :a or number only ii you know exactly where your copying is `.., begin and end. Entry of starting and ending sector number can be in decimal or hexadecimal Note that toggling the density option restores the default starting and ending se d or numbers. if you enter a starting sector number greater than the ending sector number, or if you inter an ending sector number less than the starting sector number, it will be ignored The only way to enter a starting or ending sector number larger than 720 (decimal) is t> tog gle the density to MBD

The toggle densit y default is "AUTO" In this mode the density present on the sour=a disk will be the density of the destination disk. One reason to not use "AUTO" is to force the SCP to test the source disk density. in any case both the source and destination drives must be compatible with the density of copying. For example 910 drives can only read and write single density. Standard 1050 (unenhanced) disk drives cannot read or write true double density. Another reason to not use "AUTO" is to set the start and end sector numbers for medium density above 9,,°9; see the above section.

The default source and destination drive is searched for as the first drive that responds to the computer starting at 1; this is usually drive 1. Pressing the "S" key advances the source drive number, pressing the "D" key advances the destination drive number The SCP automatically advances the drive number until it finds a drive that responds to the computer The drive number currently selected, and that drive's type is displayed HAPPY 810 drives or HAPPY 1050 drives with rev 1 row that are not set to U S EMU chow as NAP°Y WSPD. Unenhanced single density drives show as UNHAPPY. Unenhanced true double density drives show as UNHAPPY DNS PRG HAPPY 1050 drives with roe 2 row show as HAPPY USPD LENS PIG The DNS PRG means density programmable, which is common for drives that will accept the cow wands to read, format, and write in true double density. WSPD means the drive accepts the HAPPY 110 and 1050 compatible WARP SPEED data transfer commands HAPPY USPD means the drive accepts the HAPPY 1050 ULTRA SPEED commands

SECTOR COPIER - SELECTION MENU CONTINUED

The FORMAT BEFORE WRITE option default is YES. In this case the destination disk will be formatted to the same density as the source disk before any writing on the first insertion. If the destination drive is an ENHANCED drive and the format before write option is "YES", then any sectors on the source disk which had all zeroes will not be copied to the destination disk since it is known that the ENHANCED drive writes all zero sectors during formatting. Skipping of zeroes is never done with non-ENHANCED drives since some disk drives do not write all zeroes during formatting. Note that the format placed on the disks by the FORMAT BEFORE WRITE option is operating system independent, that is, no special data is written to directories, boot sectors, or VTOC This has no gearing when the entire disk is copied.

When format before write is selected as "NO" the destination disk must have been previously formatted and that density must match the density of the source disk or a disk error will result during copying.

SECTOR COPIER - COPYING MENU

Exit from the SCP SELEC TIO N MENU to the copying menu only when you ate sure that you have selected the options you need; by pressing the "X" key on the selection menu. If the copying menu is displayed and there is a RAM DIS K, or you have not yet started copying by pressing "C", insetting the source diet and pressing return, then you may press "R" from the COPYING MENU to return to the SELECTION MENU. Otherwise you cannot return to the SELECTION menu without rebooting the WARP SPEED SOFTWARE disk.

Pressing "C" on the copying menu starts the copying process. Insert the source and destination disks into the disk drives indicated when you are told to do so Copying is finished when the "DONE" message is printed Always wait for the destination drive busy Tight to go out before removing the destination disk

White the reading and writing processes are or-curing the hexadecimal sector number being read or written is displayed in 'he lower right corner of the screen Also when reading the source disk the letter S, M, or D will be displayed just to the left of the sector number. This indicates the density detected in the source disk as single, medium, or true double density Errors in reading, formatting, or write verification are reported as disk errors Pay close attention to the abbreviated messages that appear on the screen. At first glance a "DISK ERROR" may appear the same as the "DONE" message. If a disk error occurs while reading, the sector number where the error occurred is the one displayed. It a write/verify error occurs when writing to an ENHANCED drive the sector number displayed will indicate the last sector on the track that was written to when the error occurred. The error sector may be any sector on that track 1f an error occurs during formatting, the. sector number displayed is meaningless.

If you have a RAMDISK you may easily and quickly produce many copies of the same disk. Select the starting and ending sectors if you do not wish to copy the whole disk. He sure that a HAPPY ENHANCED drive is the destination drive and exit to the copying menu. Then begin copying using the "C" key to copy the first disk After the first disk is copied use the "X" key to make each additional copy of the same source disk, without re-inserting the source disk The "X" will not function if the starting and ending sectors selected cause the RAM DISK to fill up before the whole disk is read in.

HAPPY BACKUP and HAPPY COMPACTOR PROGRAMS

9,4O 'T 3 C F:

THE COPYRIGHT LAW OF THE UNITED STATES (TITLE 17 UNITED STATES CODE) AND OTHER INTERNATIONAL LAWS GOVERN THE MAKING OF COPIES OF COPYRIGHTED MATERIAL, INCLUDING COMPUTER PROGRAMS THE PERSON USING THE PRODUCT DESCRIBED IN THIS MANUAL TO MAKE UNAUTHORIZED COPIES, OR COPES NOT PERMITTED UNDER THE COPYRIGHT LAW, IS LIABLE FOR ANY INFRINGEMENT BEFORE YOU VIOLATE THE COPYRIGHTS OF OTHERS REMEMBER THAT SOFTWARE SUPPLIERS WORK VERY HARD TO PROVIDE THESE PRODUCTS AND ARE VERY DESERVING OF THE SMALL CHARGE THE`; ASK

The HAPPY BACKUP PROGRAM (HBP) the MULTI DRIVE PROGRAM (MDP), and the HAPPY COMPACTOR PROGR AM (H CP) are intended for making duplicate backup copies of ATARI executable disks. These programs are all part of the WARP SPEED SOFTWARE programs disk from HAPPY COMPUTERS Inc. These !hire programs each have a specific purpose which makes their usage different from the other.

PURPOSE

LOADING TILE HOP HCP or MDP

From the menu of the WARP FEED SOFTWARE disk, press the number which corresponds to the desired program as follows HAPPY BACKUP - 9, HAPPY COMPACTOR - 5, MULTI DRIVE - 6. The menu program will not allow these programs to be executed if there is no HAPPY drive in the system. The menu program will not allow the MDP to be executed if there are no' at least 2 HAPPY drives in the system.