THE XF SPEEDY REVIEWED

----------------------

By FM and TML

Supplier   : Klaus Peters

Price      : DM 179,-

It is almost unbelievable, but there are still new hardware kits

designed and produced for the Atari 8-bit. The latest product of

the German company 'Klaus Peters Elektronik + Software' is an

upgrade-kit for the XF-551 disk-drive.

This kit allows you to upgrade a standard Atari XF-551 to load

and write in Ultraspeed. This means that all your disks will

load approximately 6 times faster than a normal XF-551. Klaus

Peters already made such an upgrade for the 1050 a few years

ago. That upgrade was called a speedy, so he calls this upgrade

the XF speedy.

The kit consists of two printed boards, which are quite big, and

a small switch. The printed boards replaces the CPU of the

disk-drive. Because the CPU is soldered into the board of the

drive, you have to solder it out and replace it with a socket.

After that you also have to remove the ROM of the XF-551 drive

and a small wire, which was not included, must be connected to

one of the pins of the I/O socket. Last but not least, the small

switch must be glued into the drive-mechanism. TML of

ANG-software fitted this upgrade into my drive and he thinks

that fitting this upgrade into the drive is far too difficult

for people without soldering-experience.

The manufacturer claims that the XF speedy is 100% compatible

with the 1050 speedy. So lets take a look.

I tested the speedy with several disks. All programmes which

switch to Ultraspeed automatically are loading much faster than

normally. When you own the Q-meg operating system the fun really

starts. All disks will load and save in Ultraspeed. Also the

disks which were made with standard drives. There is only one

little problem: the density. A standard 1050 checks the density

of the inserted disk when you close the drive latch. The XF does

not have that feature. That is why you have to glue a switch

into the drive mechanism. This switch will take care of this

problem. Now when you close the drive latch the XF will also

check the density. The only drawback of this solution is that I

wonder if the switch will stay at its place.

Back to the compatibility. There are several programmes

available for the 1050 speedy to copy protected software.

Examples of these programmes are Ultra Copier and MS copy. Klaus

Peters has informed ANG that MS copy will be updated to work

with the XF speedy. So I already knew that MS copy would not

work so I tried to run Ultra Copier. The disk loaded perfectly,

but when I tried to copy a disk the XF speedy crashed

completely. The drive-busy light stayed on while the motor was

not running. The only way to get the XF speedy running again was

to switch the drive off. So Ultra Copier didn't run either.

Together with the 1050 speedy you'll receive a disk with copiers

and utilities which will work with the speedy. The disk loaded

perfectly but that cannot be said of the programmes. One of the

three copiers, namely Backup, does not work with the XF speedy.

Fortunately the other two copiers, HSS copy and Copy 2000, work

perfectly. It is the same story with the utlities. The utilities

which test the speedy are working fine, but the utility to slow

down the speedy will crash the speedy. Everything seems to work

perfectly but when you load a disk the first sectors are read

and then a very strange sound comes out of the drive. The drive

will not load any further. But why slow down the drive? Some

protected disks will not load in Ultraspeed. When you slow down

the drive, the drive will simulate a standard drive and load all

protected disks.

An explanation of these problems might be that the disk, which

you receive with the 1050 speedy, is specially made for the 1050

speedy. The XF speedy does not come with any disk because the

necessary programmes are stored in ROM. When you boot your

system with the drive latch open you will automatically see a

menu. In this menu you can slow down the drive and load a copier

(copy 2000). Also a DOS is included in ROM, namely Bibodos. The

slow down option in this menu works perfectly. All protected

disks will load and run, but there is one exception: the

original Polish version of Operation Blood (the ANG-version

loads perfectly).

After some while it was clear to me that the XF speedy simulates

a 1050 drive. The XF drive was always very quiet, but now the

drive makes the same noise as the 1050 when the read/write head

is moving. An advantage of this is that the drive now runs disks

which did not run on a standard XF-551. Examples are Boulderdash

2 and the original Polish version of Inside. Also the problems

of the index hole have been dissappeared. The disk can now be

formatted at both sides without the need of a second index hole.

According to the manual the XF speedy can also read and write in

quad density format (360K). This is a feature of the standard

XF-551 drive. I still have not managed to format a disk in quad

density with the XF-speedy, but the speedy does recognize the

format. It is strange that the built-in DOS, Bibodos, does not

have the option to format in quad density. I had expected that

such option was provided.

The conclusion of TML is that this upgrade is far from

perfection. The kit is very big and is not very stabil. Also the

price of DM 179, which is about 65-70 pounds, is very high and

the upgrade can not be fitted by a unexperienced solderer. The

upgrade is also NOT 100% compatible with the 1050 speedy. I

agree with him. This kit gives mainly the advantage of reading

and writing at a much higher speed. It will make your drive more

compatible but the price which must be paid for these advantages

is very high.