Note to SIO2PC users:

KEYER.OBJ is a program for ham radio which took up about 2 years

of my programming time before I started SIO2PC. I'm throwing it in

just as a curiosity, to fill in disk space. If you have friends who

are ham operators, they may be interested in it.

If you want to play with it, plug a joystick

into stick #2, run KEYER.OBJ and send some Morse code.

Note: On the XL computer, boot without BASIC (hold down OPTION while

booting) before running the KEYER.

Instructions: The WA5BDU Keyer

OPTION: Enter command mode.

SELECT: Keyboard to MEMO/TRANSMIT window toggle.

C O M M A N D S

[OPTION] plus ...

HS: Help Screen

HP: Help text dump to printer

SP: Speed

RT: Ratio, doT

RH: Ratio, dasH

SW: Spacing, Word (in 1/2 dot length units)

SC: Spacing, Char (in 1/2 dot length units)

CS: Console Speaker toggle (400/800 only)

VO: Video Off/On toggle

KD: Key Down (tune)

OA: On Air/Off Air toggle

IK: Inverse Keying (Invert output logic)

TO: Sidetone frequency

OM: Open message (Create or Edit; ESC to exit)

BE: Buffer Enable (Start/Stop sending)

TAB key: Same as above

BC: Buffer Clear (SHIFT-CLEAR is the same thing)

MA: Thru MZ: Send message A-Z immediately.

Inverse A-Z: Put message A-Z in buffer to send

PL: Parameter Load (Set up keyer the way you like it)

PS: Save current parameters to file

NOTE: File AUTOPARA.KEY will auto-load

TL: Textfile Load (Load message file)

TS: Save messages to file

NOTE: File AUTOTEXT.KEY will auto-load.

Messages can call each other (embed inverse A-Z in message)

A message can call itself.  Hit any key to stop the infinite

loop.

Commands may be embedded within messages, "*" signals start

of command.

INTERFACING: Stick 2: Pin 1: Output, Pin 2: Command button

(optional), Pin 3: Dash paddle contact, Pin 4: Dot contact,

Pin 8: Common

Special codes from paddle: .-... (AS) enters COMMAND mode

(same as OPTION), .----- (EOM) exits message edit mode (same

as ESC key).

WA5BDU, 300 S. VANCOUVER, RUSSELLVILLE, AR 72801