- 1 -

A.  #[4mAppendix#[0m_#[4mfor#[0m_#[4mas6502#[0m_#[4mFrankenstein#[0m_#[4mAssembler#[0m

A.1  #[4mPseudo#[0m_#[4mOperations#[0m

A.1.1  #[4mStandard#[0m_#[4mPseudo#[0m_#[4mOperation#[0m_#[4mMnemonics#[0m

End                                END

File Inclusion                     INCL INCLUDE

If                                 IF

Else                               ELSE

End If                             ENDI

Equate                             EQU

Set                                SET

Org                                ORG

Reserve Memory                     RESERVE RMB

Define Byte Data                   BYTE DB FCB

Define Word Data                   DW FDB WORD

Define String Data                 FCC STRING

Define Character Set Translation   CHARSET

Define Character Value             CHARDEF CHD

Use Character Translation          CHARUSE

A.1.2  #[4mMachine#[0m_#[4mDependent#[0m_#[4mPseudo#[0m_#[4mOperations#[0m

A.1.2.1  #[4mDefine#[0m_#[4mWord#[0m_#[4mData#[0m  The define word pseudo-operations

generates the byte reversed (low byte, high byte)

representation.

A.1.2.2  #[4mInstruction#[0m_#[4mSet#[0m_#[4mSelection#[0m

CPU string

The instruction set can be specified in the source file with

the CPU pseudooperation.  The string, delimited by quotes or

apostrophes, is scanned for a substring which selects which

instruction set is used.  When the program is invoked, this

operation is performed on the name of the program, then the

-p optional arguement, if any, and then any CPU statements.

The last one selects which subset of the instructions the

assembler will accept.  The instruction set can be changed

at any place in the source file.

Instruction Set          Substrings

ROCKWELL CMOS            R65C r65c

CMOS                     65C 65c

ROCKWELL Extended        R65 r65

R65C00/21                21

Standard NMOS            65

- 2 -

A.2  #[4mInstructions#[0m

A.2.1  #[4mInstruction#[0m_#[4mList#[0m

Opcode   Syntax                       Selection Criteria

ADC       '#' topexpr

ADC       '(' topexpr ')' ',' INDEX   DIRECT INDEXY

ADC       '(' topexpr ')'             DIRECT INSTCMOS

ADC       '(' topexpr ',' INDEX ')'   DIRECT INDEXX

ADC       topexpr ',' INDEX           DIRECT INDEXX

ADC       topexpr ',' INDEX           EXTENDED INDEXX

ADC       topexpr ',' INDEX           INDEXY

ADC       topexpr                     DIRECT

ADC       topexpr                     EXTENDED

AND       '#' topexpr

AND       '(' topexpr ')' ',' INDEX   DIRECT INDEXY

AND       '(' topexpr ')'             DIRECT INSTCMOS

AND       '(' topexpr ',' INDEX ')'   DIRECT INDEXX

AND       topexpr ',' INDEX           DIRECT INDEXX

AND       topexpr ',' INDEX           EXTENDED INDEXX

AND       topexpr ',' INDEX           INDEXY

AND       topexpr                     DIRECT

AND       topexpr                     EXTENDED

ASL       ACCUM

ASL       topexpr ',' INDEX           DIRECT INDEXX

ASL       topexpr ',' INDEX           EXTENDED INDEXX

ASL       topexpr                     DIRECT

ASL       topexpr                     EXTENDED

BBR0      topexpr ',' topexpr         INSTROCKWELL

BBR1      topexpr ',' topexpr         INSTROCKWELL

BBR2      topexpr ',' topexpr         INSTROCKWELL

BBR3      topexpr ',' topexpr         INSTROCKWELL

BBR4      topexpr ',' topexpr         INSTROCKWELL

BBR5      topexpr ',' topexpr         INSTROCKWELL

BBR6      topexpr ',' topexpr         INSTROCKWELL

BBR7      topexpr ',' topexpr         INSTROCKWELL

- 3 -

Opcode   Syntax                       Selection Criteria

BBS0      topexpr ',' topexpr         INSTROCKWELL

BBS1      topexpr ',' topexpr         INSTROCKWELL

BBS2      topexpr ',' topexpr         INSTROCKWELL

BBS3      topexpr ',' topexpr         INSTROCKWELL

BBS4      topexpr ',' topexpr         INSTROCKWELL

BBS5      topexpr ',' topexpr         INSTROCKWELL

BBS6      topexpr ',' topexpr         INSTROCKWELL

BBS7      topexpr ',' topexpr         INSTROCKWELL

BCC       topexpr

BCS       topexpr

BEQ       topexpr

BGE       topexpr

BIT       '#' topexpr                 INSTCMOS

BIT       topexpr ',' INDEX           DIRECT INDEXX INSTCMOS

BIT       topexpr ',' INDEX           EXTENDED INDEXX INSTCMOS

BIT       topexpr                     DIRECT

BIT       topexpr                     EXTENDED

BLT       topexpr

BMI       topexpr

BNE       topexpr

BPL       topexpr

BRA       topexpr                     INST21

BRA       topexpr                     INSTCMOS

BRK       '#' topexpr

BRK

BVC       topexpr

BVS       topexpr

- 4 -

Opcode   Syntax                       Selection Criteria

CLC

CLD

CLI

CLV

CMP       '#' topexpr

CMP       '(' topexpr ')' ',' INDEX   DIRECT INDEXY

CMP       '(' topexpr ')'             DIRECT INSTCMOS

CMP       '(' topexpr ',' INDEX ')'   DIRECT INDEXX

CMP       topexpr ',' INDEX           DIRECT INDEXX

CMP       topexpr ',' INDEX           EXTENDED INDEXX

CMP       topexpr ',' INDEX           INDEXY

CMP       topexpr                     DIRECT

CMP       topexpr                     EXTENDED

CPX       '#' topexpr

CPX       topexpr                     DIRECT

CPX       topexpr                     EXTENDED

CPY       '#' topexpr

CPY       topexpr                     DIRECT

CPY       topexpr                     EXTENDED

DEC       ACCUM                       INSTCMOS

DEC       topexpr ',' INDEX           DIRECT INDEXX

DEC       topexpr ',' INDEX           EXTENDED INDEXX

DEC       topexpr                     DIRECT

DEC       topexpr                     EXTENDED

DEX

DEY

EOR       '#' topexpr

EOR       '(' topexpr ')' ',' INDEX   DIRECT INDEXY

EOR       '(' topexpr ')'             DIRECT INSTCMOS

EOR       '(' topexpr ',' INDEX ')'   DIRECT INDEXX

EOR       topexpr ',' INDEX           DIRECT INDEXX

EOR       topexpr ',' INDEX           EXTENDED INDEXX

EOR       topexpr ',' INDEX           INDEXY

EOR       topexpr                     DIRECT

EOR       topexpr                     EXTENDED

- 5 -

Opcode   Syntax                       Selection Criteria

INC       ACCUM                       INSTCMOS

INC       topexpr ',' INDEX           DIRECT INDEXX

INC       topexpr ',' INDEX           EXTENDED INDEXX

INC       topexpr                     DIRECT

INC       topexpr                     EXTENDED

INX

INY

JMP       '(' topexpr ')'

JMP       '(' topexpr ',' INDEX ')'   INSTCMOS INDEXX

JMP       topexpr

JSR       topexpr

LDA       '#' topexpr

LDA       '(' topexpr ')' ',' INDEX   DIRECT INDEXY

LDA       '(' topexpr ')'             DIRECT INSTCMOS

LDA       '(' topexpr ',' INDEX ')'   DIRECT INDEXX

LDA       topexpr ',' INDEX           DIRECT INDEXX

LDA       topexpr ',' INDEX           EXTENDED INDEXX

LDA       topexpr ',' INDEX           INDEXY

LDA       topexpr                     DIRECT

LDA       topexpr                     EXTENDED

LDX       '#' topexpr

LDX       topexpr ',' INDEX           DIRECT INDEXY

LDX       topexpr ',' INDEX           EXTENDED INDEXY

LDX       topexpr                     DIRECT

LDX       topexpr                     EXTENDED

LDY       '#' topexpr

LDY       topexpr ',' INDEX           DIRECT INDEXX

LDY       topexpr ',' INDEX           EXTENDED INDEXX

LDY       topexpr                     DIRECT

LDY       topexpr                     EXTENDED

LSR       ACCUM

LSR       topexpr ',' INDEX           DIRECT INDEXX

LSR       topexpr ',' INDEX           EXTENDED INDEXX

LSR       topexpr                     DIRECT

LSR       topexpr                     EXTENDED

MUL                                   INST21

NOP

- 6 -

Opcode   Syntax                       Selection Criteria

ORA       '#' topexpr

ORA       '(' topexpr ')' ',' INDEX   DIRECT INDEXY

ORA       '(' topexpr ')'             DIRECT INSTCMOS

ORA       '(' topexpr ',' INDEX ')'   DIRECT INDEXX

ORA       topexpr ',' INDEX           DIRECT INDEXX

ORA       topexpr ',' INDEX           EXTENDED INDEXX

ORA       topexpr ',' INDEX           INDEXY

ORA       topexpr                     DIRECT

ORA       topexpr                     EXTENDED

PHA

PHP

PHX                                   INST21

PHX                                   INSTCMOS

PHY                                   INST21

PHY                                   INSTCMOS

PLA

PLP

PLX                                   INST21

PLX                                   INSTCMOS

PLY                                   INST21

PLY                                   INSTCMOS

RMB0      topexpr                     INSTROCKWELL

RMB1      topexpr                     INSTROCKWELL

RMB2      topexpr                     INSTROCKWELL

RMB3      topexpr                     INSTROCKWELL

RMB4      topexpr                     INSTROCKWELL

RMB5      topexpr                     INSTROCKWELL

RMB6      topexpr                     INSTROCKWELL

RMB7      topexpr                     INSTROCKWELL

- 7 -

Opcode   Syntax                       Selection Criteria

ROL       ACCUM

ROL       topexpr ',' INDEX           DIRECT INDEXX

ROL       topexpr ',' INDEX           EXTENDED INDEXX

ROL       topexpr                     DIRECT

ROL       topexpr                     EXTENDED

ROR       ACCUM

ROR       topexpr ',' INDEX           DIRECT INDEXX

ROR       topexpr ',' INDEX           EXTENDED INDEXX

ROR       topexpr                     DIRECT

ROR       topexpr                     EXTENDED

RTI

RTS

SBC       '#' topexpr

SBC       '(' topexpr ')' ',' INDEX   DIRECT INDEXY

SBC       '(' topexpr ')'             DIRECT INSTCMOS

SBC       '(' topexpr ',' INDEX ')'   DIRECT INDEXX

SBC       topexpr ',' INDEX           DIRECT INDEXX

SBC       topexpr ',' INDEX           EXTENDED INDEXX

SBC       topexpr ',' INDEX           INDEXY

SBC       topexpr                     DIRECT

SBC       topexpr                     EXTENDED

SEC

SED

SEI

SMB0      topexpr                     INSTROCKWELL

SMB1      topexpr                     INSTROCKWELL

SMB2      topexpr                     INSTROCKWELL

SMB3      topexpr                     INSTROCKWELL

SMB4      topexpr                     INSTROCKWELL

SMB5      topexpr                     INSTROCKWELL

SMB6      topexpr                     INSTROCKWELL

SMB7      topexpr                     INSTROCKWELL

- 8 -

Opcode   Syntax                       Selection Criteria

STA       '(' topexpr ')' ',' INDEX   DIRECT INDEXY

STA       '(' topexpr ')'             DIRECT INSTCMOS

STA       '(' topexpr ',' INDEX ')'   DIRECT INDEXX

STA       topexpr ',' INDEX           DIRECT INDEXX

STA       topexpr ',' INDEX           EXTENDED INDEXX

STA       topexpr ',' INDEX           INDEXY

STA       topexpr                     DIRECT

STA       topexpr                     EXTENDED

STX       topexpr ',' INDEX           DIRECT INDEXY

STX       topexpr                     DIRECT

STX       topexpr                     EXTENDED

STY       topexpr ',' INDEX           DIRECT INDEXX

STY       topexpr                     DIRECT

STY       topexpr                     EXTENDED

STZ       topexpr ',' INDEX           DIRECT INDEXX INSTCMOS

STZ       topexpr ',' INDEX           EXTENDED INDEXX INSTCMOS

STZ       topexpr                     DIRECT INSTCMOS

STZ       topexpr                     EXTENDED INSTCMOS

TAX

TAY

TRB       topexpr                     DIRECT INSTCMOS

TRB       topexpr                     EXTENDED INSTCMOS

TSB       topexpr                     DIRECT INSTCMOS

TSB       topexpr                     EXTENDED INSTCMOS

TSX

TXA

TXS

TYA

A.2.2  #[4mOperands#[0m

ACCUM               The a or (A) reserved symbol

INDEX               The x or y (or X or Y) reserved

symbols

- 9 -

A.2.3  #[4mSelection#[0m_#[4mCriteria#[0m_#[4mKeywords#[0m

DIRECT              The instruction can use the direct

addressing mode. The expression

must have a value between 0 and 255

that is defined when the

instruction is processed in the

input pass.

EXTENDED            The instruction can use the

extended (16 bit address) mode.

INDEXX              The instruction can use the X

index.

INDEXY              The instruction can use the Y

index.

INSTCMOS            The instruction is in the CMOS and

ROCKWELL CMOS instruction sets.

INSTROCKWELL        The instruction is in the ROCKWELL,

ROCKWELL CMOS, and R65c00/21

instruction sets.

INST21              The instruction is in the R65c00/21

instruction set.

A.2.4  #[4mApostrophes#[0m  The apostrophes in the syntax field are

a notation used for the parser generator and are not put in

the assembler source statement.

A.3  #[4mNotes#[0m

A.3.1  #[4mTop#[0m_#[4mExpressions#[0m  The syntax of some of the

instructions combined with the standard expression syntax

resulted in confusion whether an operand was an expression

surrounded by parenthesis, or a indexed reference. To get

around this, the expressions in these ambiguous cases were

restricted to those forms of expression that don't have

surrounding parenthesis at the top level.  Subexpressions

may be parenthesized, but only if an operator seperates or

precedes the subexpression.

Example

lda 47h,y       ; load absolute, y

lda +(47h),y    ; load absolute, y

lda (47h),y     ; load (indirect), y

lda ((47h)),y   ; error

- 10 -

A.3.2  #[4mReserved#[0m_#[4mSymbols#[0m

A.3.2.1  #[4mMachine#[0m_#[4mDependent#[0m_#[4mReserved#[0m_#[4mSymbols#[0m  A X Y a x y

A.3.2.2  #[4mStandard#[0m_#[4mReserved#[0m_#[4mSymbols#[0m  AND DEFINED EQ GE GT

HIGH LE LOW LT MOD NE NOT OR SHL SHR XOR and defined eq ge

gt high le low lt mod ne not or shl shr xor

CONTENTS

A.  Appendix for as6502 Frankenstein Assembler...........  1

A.1  Pseudo Operations...............................  1

A.2  Instructions....................................  2

A.3  Notes...........................................  9

- i -