QMI 1200ST MODEM USER'S GUIDE

INTRODUCTION

This guide describes the QMI 1200ST Modem for use with the Atari

ST   Computer.   For  the  average  user,  the  instructions on  the

Installation  sheet and the ST-Talk documentation will be  sufficient

to get you started. However, if you are using the QMI Modem to run

with  other software such as a Bulletin Board System (BBS),  you  may

need the detailed information supplied here.

MODEM COMMANDS

Modem Commands are enabled when the DATA/VOICE switch is in the

DATA position. Commands are disabled when in the VOICE position and

while a connection is established to a remote computer.

The following commands are entered with a <RETURN> after them:

COMMAND        DESCRIPTION

-----------------------------------------------------------

AT             Attention. Precedes all modem commands.

ATD            Dial a telephone number.

ATP            Switch to Pulse Dialing.

ATT            Switch to Tone Dialing.

ATA            Answer in the Answer mode.

ATO            Answer in the Originate Mode.

ATZ            Reset the modem.

ATAA           Enable Auto-Answer Mode (default).

ATAD           Disable Auto-Answer Mode.

A/             Redial last number (DO NOT hit RETURN).

The ATD command must be followed by the telephone number

then a RETURN. Use a comma (,) in the phone number for a four

second pause. Other characters are ignored. The dialing menu

in ST-Talk adds the ATD command in front of your number when

you auto-dial.

The Modem sends the following messages:

OK            The Modem command has completed successfully.

CONNECT       The Modem has connected at 300 baud.

CONNECT 1200  The Modem has connected at 1200 baud.

NO CARRIER    The Connection failed or the number was busy.

ERROR         An unknown command was entered.

FRONT PANEL

The  QMI Modem has eight LED Indicators on the front which  give

the following status:

POWER - Lights when the modem power is turned on.

TR    - Terminal Ready indicates that the modem's Data Terminal

Ready (DTR) input is on.

SD    - Lights when the modem is sending Data.

RD    - Lights when the modem is receiving Data.

HS    - High Speed indicator lights for 1200 baud operation.

MC    - The Modem Check flashes for errors in self test.

TM    - Test Mode indicator lights up when in the test mode.

RI    - Ring Indicator lights up when the modem detects a ring.

The three buttons on the front work as follows:

- DATA/VOICE

When in the OUT position, the modem is inactive and your

phone may be used for talking. When in the IN position, the

modem is active and ready for commands and data.

If the DATA/VOICE switch is changed to the voice mode while

the modem is in use, the modem will be disconnected from the

phone line and the call will be lost. Do this if you need to

force the modem to hangup.

When in the DATA mode, the modem will answer a call when

the phone rings. Be careful not to leave it in the DATA mode

when not using the modem.

- 300/1200

When in the out position, the modem is in the 1200-baud

speed. Remember to set the software for 1200 baud when using

this mode.

When the button is in, the modem is at the 300-baud speed.

Remember to set the software for 300 baud when changing to this

mode.

When using the modem to automatically answer a call, set the

300/1200 baud button to the 1200-baud speed. The modem will set

for the correct baud rate automatically.

- ON/OFF Switch

Push  this  button  in to turn the  power  on  to  the  modem.

Likewise, push to the out position to turn off the modem.

BACK PANEL

The OPTIONS Switches should all be in the up (OFF) position

for normal use. These switches have the following functions:

OFF (UP)       ON (DOWN)           FUNCTION

------------------------------------------------------------

1)   8 bits         7 bits              bits/character

2)   RDL inactive   RDL active          Remote Digital Loop Test

3)   DL inactive    DL active           Digital Loop Test

4)   No Self Test   Self Test           Self Test Mode

5)   AL inactive    AL active           Analog Loop Test

6)   DTR active     DTR override        Data Terminal Ready

7)   CD active      CD override         Carrier Detect

8)   CTS active     CTS override        Clear To Send

The POWER ADAPTER input comes from the 12-Volt AC Adapter.

Do not use any other power adapter with the Modem!

The RS-232C Interface connector is a standard computer inter-

face with the following pin functions:

PIN #    In/Out     FUNCTION

---------------------------------------------------------

2         In        Transmitted Data (TD)

3         Out       Received Data (RD)

5         Out       Clear to Send (CTS)

6         Out       Data Set Ready (DSR)

7         Out       Signal Ground

8         Out       Carrier Detect (CD)

18        In        Analog Loopback (AL)

20        In        Data Terminal Ready (DTR)

21        In        Remote Digital Loopback (RDL)

22        Out       Ring Indicator (RI)

The TELEPHONE JACK is used to connect a telephone on the

same line that the modem is connected to.

The TELEPHONE LINE is a standard modular telephone cord with

a single-line plug.

TECHNICAL SPECIFICATIONS

Bell 212A Compatible Frequencies. (Full Duplex)

300-baud Originate: Mark=1270Hz, Space=1070Hz FSK

300-baud Answer:    Mark=2225Hz, Space=2025Hz FSK

1200-baud Originate: 1200Hz PSK

1200-baud Answer:    2400Hz PSK

RS-232C Modem-Computer Interface.

RJ11C Standard Telephone Jack.

600 Ohm balanced Telephone Line Impedance.

--------------------------------------------------------------------

Quantum Microsystems, Inc.

PO Box 179

Liverpool, NY 13088

315-451-7747

315-457-7216 (via Modem, 24 hours)