LINKLINE:   (LINKLINE.EXE)     Written by Patrick Bass     (Originally published in the Aug.  '86 ANTIC.     Enhanced ST version is brand new.)     LinkLine is a PAIR of programs which let you quickly transfer filesbetween information between your 8-bit and your ST computers.     LinkLine will quickly and reliably transfer programs, text files,picture files -- any type of file shorter than 33K (about 270 5-1/4"disk sectors).  It's easy and it's fast (4800 baud, about sixteen timesthe speed of an Atari 1030 modem)!     HOW TO GET LINKLINE     Use XMODEM to download *both* LINKLINE programs.  Use a modemprogram for your 8-bit computer to download the 8-bit side of LinkLine,and download the ST side with an ST modem program.     USING LINKLINE     You need:     A - 8-bit Atari computer     B - An Atari 850 Interface Module     with a modem cable (RS-232 to D-9.)     C - Null modem connector     D - Gender changer (if required)     E - Any Atari ST computer     | A |=>| B |=>| C |=>| D |=>| E |     Sample porting configuration     WALK-THROUGH DEMO     1) Rename your ST version of LinkLine to "LINKLINE.ACC". Rename the8-bit version to "AUTORUN.SYS".     2) Turn both computers off.  Connect the two computers together byplugging the ST end of the Null Modem cable into the 25 Pin RS-232connector on the back of the ST, and the 8-bit Atari end into Port 1 ofthe 850 Interface.  Now insert your LinkLine disks, and turn bothcomputers on again.     3) At the ST, point to the "Desk" item and click on the menu choicecalled "LINKLINE".     4) To transfer a file from the 8-bit Atari into the ST, choose theLOAD A FILE option and type in the name of the file you want totransfer.  LINKLINE will read the file off the 8-bit disk, and copy itto an ST disk.     5) To transfer a file from the ST into the Atari 8-bit, choose theSAVE A FILE option and type in the name of the ST file to transfer.LinkLine will read the file in and then prompt you for the name to callit on the Atari 8-bit disk. When you type in the destination filenameand click on OK, LinkLine will copy the desired file from the ST ontothe 8-bit disk.     ADVANCED USE     LinkLine will copy any file under (roughly) 33K long. It cannottell the difference between a picture file and a text file.  Atari 8-bitword processors use a special character, number #155, to mark wherelines of text end. Atari ST computers use a standard carriagereturn/line feed at the end of each line.  Just before LinkLinetransfers a file, it will ask you if you wish to convert Atari 8-bitEnd-Of-Line (#155) characters to Atari ST CR/LF pairs.     If you are transferring a *text* file, use the EOL and CR/LFtranslations.     Some ST word processors place a CR/LF pair at the end of each lineonscreen, with a double CR/LF pair between paragraphs.  Most Atari 8-bitword processors want their EOL characters only between paragraphs.  Toaccount for this, LinkLine will ask if you desire any "single CR/LFs"stripped out of the file.  Again, if you are transferring a *text* filefrom the ST into the Atari 8-bit, and aren't satisfied with the results,try stripping all single CR/LFs.     That's it.  You're done.!ZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZZ