FIXING OLD DISK IMAGE FILES FOR REV 3.00

*** NOTE: SIO2PC NOW HAS A BUILT IN FEATURE TO FIX YOUR PRE - REV

3.00 FILES: JUST PUT /X AFTER THE FILENAME (NO SPACE) WHEN  LOAD-

ING.   DON'T DO THIS ON NEW "BIG" REV 3.00 DISKS - IT  WILL  MESS

THEM UP.  I'M INCLUDING THIS FILE ANYWAY, FOR ANYONE WHO WANTS TO

READ MORE ABOUT IT, OR WANTS TO LEARN ABOUT USING DEBUG ***

The problem:

Here's where I messed up.  The disk image files have a 16 byte

header to tell SIO2PC the file size, sector size, etc.  Up  to

version  2.12, 6 of these bytes were used and 10  were  spare.

The  spares were initialized to zeroes - I thought.  Due to  a

bug,  the spare bytes may not be zeroes. Since Rev  3.00  uses

the  7th and 8th bytes to help hold the larger  number  needed

for  the new BIG files, it needs to see 0s there for  the  old

64K  thru 184K files.  If it sees something other than  0,  it

thinks the file is much bigger than it really is.

The fix:

There  are a number of ways to turn the 7th and 8th  bytes  of

your old files to 0's.  If you have a utility, such as Norton,

which  allows  you to look at hex bytes of a file  and  change

them,  you could use it. (Note: be careful about  finding  out

which  is the 7th byte.  Often, computer texts  and  utilities

start  counting  with 0.  So the 7th byte would  be  byte  #6.

Confusing enough?)

Since  everyone has the DOS utility DEBUG, I'm going  to  show

how to fix old disk images using it.  I'm hardly a DEBUG whiz,

but  it only took me a few minutes to look up the commands  in

my DOS book and a few more to fix a bunch of files.  You don't

have  to understand the comments I'll include in  my  instruc-

tions below, just follow them  exactly.  It may be a good idea

to make backup copies of your disk image files, just in  case.

OK, here we go:

1) First,  get  into the directory which holds your  SIO2PC  disk

image file(s).  Now, type the command, DEBUG MYFILE.ATR  [CR],

where  MYFILE.ATR is the file you want to fix, and [CR]  means

press enter. You will see DEBUG's dash "-" prompt.

2) Now  type d [CR].  DEBUG will list out 8 lines of numbers.  At

the  left,  you see XXXX:YYYY which is the  address  where  it

loaded your file. The XXXX number will vary with your computer

setup.  The first YYYY number will be 0100.  To the  right  of

this, on the first line will be 16 two digit hex numbers, with

a  "-" between the first and second group of eight. The  first

two pairs of numbers are 96 02.  This is the SIO2PC disk image

identifier.  The next two pairs tell the disk size.  The  next

two  tell the sector size.  The last pair before the dash  are

the  offending numbers.  If they aren't both 0s, they must  be

changed.  Here's  an  example of what you could  see  at  this

point:

debug mydos45.atr

-d

0C66:0100 96 02 00 23 80 00 FC B0-68 53 51 ... etc

3) From  the DEBUG "-" prompt, enter the command e CS:0106  [CR].

This  tells DEBUG that you're going to ENTER bytes at  address

CS:0106.  It responds by repeating the address and showing you

the first byte (two digits) followed by a decimal point,  like

this:

e cs:0106

0C66:0106  FC.

You then enter two 0s and press the space bar. DEBUG shows the

zeroes,  moves  over a couple of spaces, and  shows  the  next

byte. You again enter two 0s, then press ENTER. This takes you

back  to the dash prompt.  If you think you've  done  anything

wrong,  you can still bail out without affecting the file,  by

pressing Q [CR].

4) Now,  you will write the corrected file back to the disk.   Do

this  by  entering  w [CR] and the dash  prompt.   DEBUG  says

"Writing  xxxxx bytes" then you get the prompt.  Now  press  q

[CR] to quit.

5) Repeat  the process for your other disk image files.   Believe

me, it's MUCH faster doing it than reading (or writing)  about

it.

6) Here's an abbreviated procedure, without the comments:

Go to the directory containing the file to be fixed.

TYPE:  DEBUG filename.atr

TYPE:  d [CR]         (This step is optional)

TYPE:  e CS:0106 [CR]

TYPE:  00

PRESS SPACE BAR

TYPE:  00 [CR]

TYPE:  w [CR]

TYPE:  q [CR]

Note:  replace  filename.atr with the actual filename  of  the

disk image to be fixed.  [CR] means press ENTER.

###############################################