D I S C L A I M E R

I  have tried to ensure that my software is free of  harmful  de-

fects (bugs) and of viruses.  However, either could slip  through

my  defenses.  Use the software at your own risk.   Similarly,  I

believe  my hardware design to be in accordance with  good  prac-

tice,  and I have a number of satisfied users.   However,  wiring

errors,  errors in my design, and configuration  differences  are

possible and I cannot guarantee that these problems will not lead

to damage to your equipment or software or other unforeseen  dam-

ages.  Use at your own risk.  Furthermore, if you choose to  con-

struct my interface yourself, you must be experienced with  elec-

tronic  building  techniques.  Even if you are  experienced,  you

could burn or cut yourself or experience other damage or  injury,

for which you must assume responsibility.

I consider the SIO2PC system to be a service to the Atari  user's

community. Although I ask a shareware contribution, and do make a

charge  for  assembling interfaces or kits, I still  consider  my

efforts  to  be a hobby and a service.  IF YOU HAVE SENT  ME  ANY

PAYMENT FOR ANY SOFTWARE, KIT, OR ASSEMBLED INTERFACE AND DO  NOT

WISH TO ASSUME FULL RESPONSIBILITY FOR USE OF OR CONSTRUCTION  OF

THESE  ITEMS,  YOU MAY RETURN THEM TO ME AND I WILL  REFUND  YOUR

MONEY.   IF YOU ARE CONSTRUCTING THE INTERFACE AND CANNOT  VERIFY

THE  ADEQUACY OF THE DESIGN ON YOUR OWN, SEEK THE ADVICE  OF  ONE

COMPETENT IN ELECTRONICS.  OTHERWISE, DO NOT ATTEMPT TO BUILD AND

USE THE INTERFACE.  DON'T USE THE SYSTEM ON COMPUTER SYSTEMS USED

FOR COMMERCIAL PURPOSES OR FOR CRITICAL APPLICATIONS.

INTRODUCTION

Greetings Atari and PC users! SIO2PC is now a "Shareware" product.

If you know little or nothing about the system, let me give you

a rundown. A separate .DOC file, SIOINSTR.DOC, gives more detailed

information on running the system and describes its features.

SIO2PC is a hardware interface and software utility package to

allow storage of your Atari files on your PC's hard and floppy

disks,  file  conversions in both directions, and  Atari  printer

output redirection to your PC's printer, screen, or file.

FEATURES:

Create 1 to 4 Ramdisks in your PC's ram space.

Save Ramdisks to PC hard or floppy disks.

Boot from the PC. Real drive not needed to start-up.

Create Single or Double density ram-drives.

No special Atari software patches. Use your favorite DOS.

Almost twice as fast as an Atari 810 drive.

Co-exists in the Atari daisy chain with "real" drives.

Software written in assembler: Compact and Fast.

Compatible all the way down to the hardware level -

CIO, SIO, even direct hardware calls to drives will work.

Print-Thru mode sends your Atari's printer bound output to

your PC's  Printer, Screen, or file.

File Conversion: Convert and transmit PC files to Atari,

or Atari files to PC files.

Won't endanger your PC's hard disk. Your Atari ramdisk

images  are  stored as ordinary PC files.  No  direct  sector

writes used.

HARDWARE REQUIREMENTS:

The  interface  between your PC's serial  port  and  your

Atari's  SIO port requires a simple interface. It uses  two  IC's

and  a  few capacitors and can be built on a 2"  X  3"  prototype

board.  If  you  are  handy  with  a  soldering  iron,  read  the

BUILD_IT.DOC  file on your disk. It gives complete  instructions,

including  addresses of parts suppliers. If you wish, I'll  build

one  for  you, or sell you a kit of all parts with  drawings  and

detailed instructions. See BUILD_IT.DOC.

HISTORY:

SIO2PC has been about 2 years in the making. Before I got

my  PC,  I considered my alternatives for my  trusty  Atari  800.

Should  I  invest in an adapter and hard disk for  the  Atari?  I

realized  that  the huge PC market made an entire  PC  with  hard

drive  almost as cheap as adding a drive to my Atari. So why  not

do  both? The PC could store my Atari programs (get rid  of  that

huge pile of floppies), and I'd have a new computer to boot!  The

project has worked out better than I could have imagined. The  PC

is  a great computer - in some areas. In areas where it is  weak,

the Atari still shines. Curing the Atari's mass storage  weakness

makes it even better.

SHAREWARE INFO:

As you no doubt know, Shareware is a system which  allows

you to receive freely distributed software, and pay for it if you

find  it  useful. If you build the interface and  find  that  the

system is useful, I request the modest sum of $10 for the  design

and software. If you choose to buy the kit or assembled interface

from  me,  no additional payment for the  software  is  expected.

Either  way, registered users may upgrade to the latest  revision

by sending a disk sized SASE and $3.00. Your suggestions will  be

considered for future revisions. So far, I have always been  able

to personally respond to questions and problem reports.

READ ALL ABOUT IT:

Don't you hate to write off for a shareware program  that

looks  good  in the ad, only to find an endless README  file  de-

scribing the limitless complexities of the package? Now that  I'm

on  the  creating end of that particular  method  of  information

overload,  I can see how documentation files just grow...  But  -

SIO2PC  is a simple menu driven program which can, I believe,  be

mastered without instructions. So, skim through the rest of  this

file and the other .DOC files for tidbits, and return only if you

have  problems  or specific questions after you get  your  system

going.  Of course, if you decide to build it yourself,  you  will

want  to  read  BUILD_IT.DOC carefully. Also, go  ahead  and  run

SIO2PC.COM and play with the menu options a bit, even without the

interface. It won't hurt anything. Note: This DOC file gives some

problem  resolution information, technical information, and  more

information on file transfer features. For a more concise  user's

guide to the program, see the SIOINSTR.DOC file.

CONTENTS:

1) TECHNICAL INFORMATION ON SIO2PC

2) REMOTE CONTROL PROGRAM USE

3) MAKADISK PROGRAM USE (FILE TRANSFER PC TO ATARI)

4) FILE2PC PROGRAM USE (FILE TRANSFER ATARI TO PC)

5) PRICING INFORMATION AND FUTURE PLANS

6) REVISION LOG

7) PROBLEM DIAGNOSIS QUESTIONS

8) POSSIBLE PROBLEMS & SOLUTIONS

9) USER'S QUESTIONS

10) WHO'S NICK KENNEDY?

1) TECHNICAL INFORMATION ON SIO2PC

SIO2PC  is 100% written in PC assembly language.  REMOTE.OBJ  and

FILE2PC.OBJ  are written in 6502 assembly language.  MAKADISK.EXE

is written in 'C'.

SIO2PC recognizes the basic SIO bus commands: READ SECTOR, WRITE

SECTOR, PUT SECTOR, STATUS, FORMAT, FORMAT 1050 1.5 DENSITY.

Because  it works at this elementary level, it should  work  with

any DOS and with non-DOS disk loader programs. You can also  make

boot disks (without DOS) and boot them from the PC.

SIO2PC  uses only MS-DOS calls to do file I/O on the PC.  Because

there are no direct sector writes, etc., SIO2PC won't trash  your

PC disks. Just use the right path/filename when doing writes.

SIO2PC uses file handles, so I assume it will require MSDOS or

PCDOS 2.0 or later.

SIO2PC redirects the timer 0 interrupt for it's timing  purposes.

This  is the timer which supplies the BIOS its "timer  tick",  18

times per second, 55 milliseconds per tick. SIO2PC also  directly

reads the timer as it is counting down in a routine for more fine

timing (the microsecond level). If your PC doesn't use the stand-

ard  timer  chip (or compatible) with  standard  port  addresses,

there could be problems. Note: Most PC Clones are HIGHLY hardware

compatible.

SIO2PC gets the port address for the specified COM port from bios

data area words at 040:0000, 040:0002, 040:0004, 040:0006 for COM

1,2,3,  and  4. I've found out that the  POST  routine  typically

doesn't set up the addresses for ports 3 & 4. Therefore,  version

1.01  will use default port addresses 03F8, 02F8, 03E8, 02E8  for

COM  1 - 4 if it finds a zero in the BIOS data area word. If  you

know your port addresses to be other than this, a third option is

now available. Use your computer's documentation to find the base

address,  and  use SIO2PC's  "E" command to enter  the  addresses

directly.

The  Atari serial bus reserves device ID's 31 - 38 hex  for  disk

drives  1  thru  8. I have used ID #39h for  the  remote  control

function.  When the SIO2PC program finds this ID # in  a  command

frame,  along with a Write Sector command, it expects to  receive

one sector (128 bytes) containing a command string. It then  acts

on  the string as if it were a command tail from the DOS  command

line.  Any  error,  or the Atari EOL  character,  will  terminate

command processing and flush the line of any further commands. It

is best to end the string with a space as a delimiter, especially

if your command ends in a pathname, else your PC may pause  wait-

ing for input or an ENTER keystroke. A REV 2.1 change added  this

function  to the REMOTE process: When DEVID = 039h and  DCOMND  =

'R', the SIO2PC sends a 128 byte data frame to the Atari contain-

ing  the RAMDISK status information string shown on  the  screen.

The  string  sent  depends on the contents of  DAUX1  which  must

contain 1 - 4 for ramdisks 1 - 4. This function allows REMOTE.OBJ

on the Atari to update the user as to the status of the ramdisks,

before  and after the execution of the remote function.  In  this

way,  the user can see if his remote command went to  error  free

completion.  In some cases, the PC may be busy carrying  out  the

command when the data request is received. For example a  command

to  write a ramdisk out to a floppy disk may take 10  seconds  or

so.  Therefore,  REMOTE.OBJ tries 3 times, pausing  4.25  seconds

between retries, before giving up on getting the information.

The FILE2PC program also uses its own bus ID as a trigger for the

FILE2PC  program. ID# 03Ah puts the  program into  File  Transfer

mode.  The  Atari always asks for Status of  device  03Ah  before

putting a sector. (Only Status and Write are recognized by device

03Ah.) If the first status byte (of 4) is normal (010h), then the

PC  is  ok for the transfer mode. If it is 0FFh, then  the  Atari

should abort because the PC has encountered an error. The process

works  like this: The Atari gets Status, then, if OK, it sends  a

sector  to the PC. The PC assumes that this sector  contains  the

pathname  of the destination file, terminated in an ATASCII  EOL.

The  Atari pauses 4 seconds while the PC attempts to  create  the

file. (Note, any existing file of the specified pathname will  be

overwritten.) After the Atari gets another good status, it starts

sending sectors of file information.

The AUX bytes, which usually carry  sector # information,  commu-

nicate  the following to the PC:

AUX1 = 0:       Full sector,  128 bytes

AUX1 = 1:       Last sector, AUX2 contains the byte count

AUX1 = 2:       Atari encountered an error, abort process.

As I said, the Atari asks the PC for Status after every write.

Structure of an SIO2PC Atari Disk Image:

It's  extremely simple. There is first a 16 byte header with  the

following information:

WORD = special code* indicating this is an Atari disk file

WORD = size of this disk image, in paragraphs (size/16)

WORD = sector size. (128 or 256) bytes/sector

10 (ten) unused (spare) header bytes

After the header comes the disk image. This is just a  continuous

string  of bytes, with the first 128 bytes being the contents  of

disk sector 1, the second being sector 2, etc.

* The "code" is the 16 bit sum of the individual ASCII values  of

the  string  of  bytes: "NICKATARI". If you try to  load  a  file

without  this  first WORD, you get a "THIS FILE IS NOT  AN  ATARI

DISK FILE" error message. Try it.

SIO2PC  uses, or can use, the PC's hard or floppy disks to  store

Atari disk information. However, to give maximum speed, the  disk

information  is  buffered via a ramdisk set up in  the  PC's  ram

space.  When  the Atari reads/writes its SIO2PC  "disks",  it  is

actually communicating with the ramdisks in the PC's memory.  The

user  can  choose  to load a ramdisk image or save  one  back  to

physical disk at any time via menu selection.

Timing  information:  The  Atari SIO2PC  bus  protocol  specifies

minimum and maximum times between Atari commands and disk drive's

responses. The minimums give the Atari time to get ready to  take

the  data  from the bus if it has to do some other  work  in  the

interim.  The maximums let the Atari decide that  the  peripheral

isn't ever going to answer and it will give a "time out" error. I

have found that the system is very flexible and forgiving. I have

used wide variations and still had a workable system. But for one

guy  (you  know who you are, Joe!) or maybe two, I've  made  this

menu to allow you to experiment with different values. The system

goes through this little dance of command - acknowledge - data  -

acknowledge - complete, etc with timings in between. In  addition

to  these  times inside the bus frame, I've made  a  couple  more

(normally  zero)  time delays available.

First: On a serial bus, data is normally sent continuously,  with

no  time (except start and stop bits) between bytes. Now you  can

experiment  by adding some time here. (My system slowed down  but

still ran normally.)

Second: Some publications say that some UART (serial) chips can't

stand  to  be addressed as fast as some PC's are capable  of  ad-

dressing  them. I already had a bit of time delay in  there,  but

now  you can add some more. The time delays work like this:  Each

unit is 850 nano-seconds which is almost 1 micro-second.  (Multi-

ply units by .85 to get micro-sec.) (Exception: the value for the

printer  delay is in 1/18ths of a second.) Also, you  must  enter

the  units as a 4 digit hex number. Don't panic, it's easy.  Here

are some conversions:

DECIMAL        HEX

0005           0005

0010           000A

0050           0032

0100           0064

0250           00FA

0500           01F4

0750           02EE

1000           03E8

2000           07D0

The  timing menu gives default values you can use as  a  starting

point. If you want to see the menu updated with your choices,  an

item is available from the menu (choose #8).

One  problem with the TIMINGS menu is that it can't be  addressed

from  the  command line tail, so if your system  does  need  non-

standard timings, you have to set them up manually, each time you

run the program. Not any more, Joe! Here's a procedure to  perma-

nantly alter your program:

This procedure uses DOS's little utility "DEBUG," because  every-

one  has it. But you could use NORTON or similar if you  want.  I

moved  the timing values to the front of the program and  put  an

ASCII string in front of it to make it easy to spot. Try this:

Get into the directory containing SIO2PC.COM. On the DOS  command

line, type DEBUG SIO2PC.COM <ENTER>. Now you see a hyphen,  which

is the DEBUG prompt. Now, type D <ENTER>. A table of hex  numbers

appears. On the left is the corresponding SEGMENT:OFFSET  address

for the first entry of each line. On the right is the correspond-

ing ASCII (text) representation of the hex bytes. You notice that

my  little flag, "TIME DELAY VALUES: T1-T8, 2 BYTES  PER  ENTRY:"

takes  you up to the line starting with offset :0130.  That  line

contains the timing values and looks like this:

????:0130 64 00 B0 04 2C 01 2C 01-F4 01 00 00 00 00 20 20

Here's  the slightly tricky part: Each of those 2 digit hex  num-

bers  is a byte. But each timing value is a WORD (2  bytes).  And

the PC stores words as lower part first. So the values are: T1  =

0064, T2 = 04B0, T3 = 012C, etc.

The  good part is, you don't need to worry about all  this  junk,

we're gonna fix it in a very easy way. If you have been following

me and are now in DEBUG, get out of it by typing Q <ENTER>.  Now,

I recommend that you copy SIO2PC.COM onto a floppy because it's a

little scary using DEBUG to write to one's hard disk. Ok,  you've

done  that  and you have the floppy in drive A:. Get into  A:  by

typing  A: <ENTER>. Now, type DEBUG SIO2PC.COM <ENTER>. Give  the

"GO"  (run)  command by typing G <ENTER>. SIO2PC  runs  from  the

floppy. First give it a port number, then go to the "A" menu  and

change the timings values to those you have found to work on your

system. Now, leave the timings menu and QUIT SIO2PC. You will see

by the hyphen that you are back in DEBUG. Type W <ENTER> and  the

program will be written back to the disk file. Quit DEBUG with  Q

<ENTER>.  Now run SIO2PC without DEBUG, and call up  the  timings

menu again. You will see that the DEFAULT column stayed the  same

but the CURRENT values are now permanently altered. Hey, I didn't

know that DEBUG could be so useful, did you?

DOUBLE DENSITY: The Atari expects the first 3 sectors on a disk to

be single density. This is necessary if you are to be able to boot

from the disk. SIO2PC uses this format for its double density mode.

FORMATS vs DISK SIZES:

I  generally haven't restricted these things, but  recommend  the

following:  Use  143K size only for "enhanced" or "1.5"  or  "DOS

2.5" density. The 183K size was created for double density. A lot

depends on your DOS. Some DOS's can figure out the actual size of

a  disk,  others  may assume a default. MYDOS  is  pretty  smart.

SPARTADOS  probably is also. Currently SIO2PC doesn't respond  to

"get  configuration" and "set configuration" commands. If  I  can

think of a reason why it should, I'll add the capability.

PRINT-THRU:

The serial bus ID for the Atari printer is 040h. With  PRINT-THRU

in  effect,  the  program answers bus commands  directed  to  the

printer  and  routes the data to the PC's printer or  screen,  as

selected. Note that the Atari uses an EOL character (09B hex)  to

mark the end of line, while the PC uses carriage return and  line

feed (CR/LF) characters. So SIO2PC gives you the option of making

the translation. If you are printing text, you should  translate.

For  graphics, probably don't translate. You can also  strip  the

high  bit  if you want. The reason? ASCII is a 7 bit  text  code.

Computers  use 8 bit data. So, text with the high bit set is  not

defined under ASCII, and different computers use it  differently.

On the Atari, text with the high bit set is printed to the screen

as inverse video. On the PC you may get graphics characters.  So,

when  printing text, you usually want to clear the high  bit.  If

you are printing 8 bit graphics data, you usually don't want to

change it.

Summary:

TEXT: Translate EOL's? : YES. Strip high bit? YES

GRAPHICS:     "   "          MAYBE*     "      "      NO

*  It  depends on how the graphics program you're  using  handles

EOL's.

For revision 2.6, I've made some changes to the PRINT_THRU  func-

tion. The reasons, and other comments, are summarized below:

PRINTER NOTES, REV. 2.6

Due  to reports of problems with the PRINT-THRU mode  in  certain

cases, I've reworked the print-thru routine.

My basis for this feature is to try to provide 100% compatibility

with  the Atari 850 Printer/Serial interface, since it should  be

the standard for the Atari. I went back and reread the functional

description in my 850 manual. Here is a summary:

The Atari printer handler (in the computer's CIO, not in the 850)

sends fixed length records (40 characters) to the printer. If  an

EOL (RETURN key) is encountered, it fills the rest of the  buffer

with spaces and sends it. Note that software on the Atari doesn't

necessarily  have  to use CIO. It can send data directly  to  the

serial  port using SIO. Fixed length, 40 byte, records are  still

necessary  because the interface expects them. Now, when the  850

receives  a  40 byte record from the computer, it goes  by  these

rules:

1) Send all bytes to the printer, as 8 bit data (no clearing  the

high bit) except,

2) If an EOL (9B hex) is encountered, convert it to a CR  charac-

ter, and ignore the rest of the buffer (assumed to be  extraneous

spaces supplied by CIO) except,

3)  If several consecutive EOLs are found, translate them all  to

alternating CRs and spaces and send to the printer. NOTE: rev 2.8

took out the spaces. My version was adding a space even if  there

was only one EOL, causing alignment problems in the printout.

My interface was designed to provide several modes of  operation,

one  of which would emulate the 850 exactly. However,  it  wasn't

doing some of the things above. It didn't ignore characters after

the EOL. It didn't translate multiple EOLs to multiple CRs,  only

the  first.  It also translated EOLs only to CR plus LF,  not  CR

only. Now you have the option of doing either.

What do the 850 rules above mean to the Atari printer programmer?

Well,  as I see it, you can NEVER send a hex 9B to  the  printer,

because  the conversion is irrevocably hard coded into the  850's

ROM. Since graphics data by nature can represent any dot pattern,

it must include all possible combination of bits, including  $9B.

This really isn't a huge problem. The software would just have to

detect  EOLs and change one bit. One dot's worth of error out  of

every  256 * 8, on average, won't hurt anything.  Another  "cure"

would  be  to have a printer which requires 7, not 8,  bit  data.

Since  $9B  has its high bit set, it would never be  required  to

send graphics data. (Just as it is not required to send  standard

ASCII text, a 7 bit system.)

All this info is presented for your amusement only. Hopefully, if

my emulation of the 850 is correct, your software will play  just

like it did on your Atari printer. Of course, the Atari  software

will  have to be configured for the characteristics of  your  PCs

printer, not your Atari's printer, assuming they are different.

Rev 2.10 information: I added a value to the TIMINGS menu for the

printer. This is experimental at this time. One guy has a problem

with  the  system (PC) locking up after 3/4 page or so.  I  think

maybe his printer isn't handshaking properly, so I've added  this

timing value, to keep from "overrunning" his printer. The  normal

value  is 0. If you have problems, add a little time. This  value

is  in units of 1/18th of a second, so keep your number small  or

you'll be in for a long wait. It is the value used between  send-

ing of 40 byte records to the printer. Figure out about how  long

it  takes  your printer to print a line and use about  half  this

value in seconds. So if your printer printed a line in 1  second,

the value used would be 0009 for 9/18 of a second.

UNINSTALL RAMDISK:

Why  is  it important to be able to do this? Well,  if  you  have

already  used up all your memory in defined ramdisks,  you  can't

overlay  them  with new ones unless they are the  same  size  and

density  as the existing ones. This often would mean you  had  to

quit  SIO2PC and restart it to get the configuration you  wanted.

Now, you can delete ramdisks, and thanks to "garbage  collection"

performed  by  the program, all data moves down to  fill  in  the

hole.  So,  you  now have free memory equal to the  size  of  the

deleted disk.

STATUS LINE INFO:

I've  created  a special status line, which is displayed  at  the

bottom of the screen, to help identify problems and to show  that

the SIO2PC program is actively doing something. It shows:

Location: An identifier for the subroutine being executed.

Command: SIO command from the Atari (Read, Write, etc.)

Lastcom: Command received previous to the one above.

DEVID: Hex number for device being addressed by Atari. Note

31 - 38 = D1 - D8; 40 = Printer, 39 = Remote Control program

3A = FILE2PC program.

Comlin: High/Low status of the Atari command line.

Error: An error code returned by one of SIO2PC's routines.

A character here doesn't necessarily mean a problem.

PC Memory Usage:

The technically inclined may want to know how the program decides

it  has  the right to RAM for its ramdisks.   Well,  I  initially

wrestled  with this one. I was trying to use a DOS  function  for

"Allocate  Memory,"  but I got an error code meaning  "no  memory

available."   This  was crazy, because SIO2PC is a  very  compact

program. It turns out that when DOS runs a COM file, it allocates

ALL  available  memory  to the program. This is  because  DOS  is

intended to be a single tasking system.  I took advantage of that

fact.  My  program doesn't ask for memory, it just takes  it,  as

though  it's  the only player (which it should be).  (Note:  TSRs

loaded  before  SIO2PC are safe. SIO2PC only uses  memory  higher

than  that allocated to itself.)  I use an internal DOS  variable

to  decide if there is enough memory to load/create  the  ramdisk

you  request.   Why am I telling you all this?  Because  I'm  not

sure  how all this will work under new,  multi-tasking  operating

systems,  like Windows.  So far, I haven't heard of any  problems

in this area.  If you suspect one, let me know. I expect it  will

work under Windows if you aren't multi-tasking.  Otherwise, maybe

Windows  will allow you to allocate extra memory to a program.  I

am not yet a Windows user, so I don't know.

2) REMOTE CONTROL PROGRAM USAGE

This  section is to tell you how to use the Atari program  called

REMOTE.OBJ.  This  program is found on your ramdisk  image  file,

ATARI.ATR.

The program allows you to send commands to SIO2PC from the Atari,

just as if you were typing them on the PC's keyboard. So, if your

Atari and PC are in different rooms (or something), you can  swap

ramdisks, save and load, etc. without leaving the Atari. The file

is  fairly small, and if you plan to make use of it,  you  should

copy  it  to as many ramdisk images as necessary to  assure  that

it's accessible when you need it.

When the program comes up, it shows a list of most of the  SIO2PC

commands.  If  you want to study this list, hold down  the  START

key, or else it will scroll off after 3 seconds. The program next

gets the status's of the 4 ramdisks from the PC and puts them  on

the screen. Now you are ready to enter your command. Say you want

to load a disk image from A:\ATARI.ATR into ramdisk #4. Just type

L4A:\ATARI.ATR  and  RETURN.  Note that some  commands  give  you

prompts which you must answer in your command string, so  antici-

pate them. For instance, if you tell the PC to write back a file,

you need to anticipate "USE EXISTING FILESPEC?" and "FILE EXISTS,

OK  TO OVERWRITE?". Note that if writing was successful, the  "W"

in the status line will have changed to "N" after completion. One

person  was using REMOTE.OBJ on the Atari as an AUTORUN.SYS,  and

feeding  it with a batch script from SPARTADOS to automate  setup

of  his  MIO ramdisks. Because he couldn't simulate  a  character

from the keyboard with SPARTADOS, my "Run another command  (Y/N)"

left him stranded. For this reason, you can now skip the  command

by  starting your string with a space. When REMOTE.OBJ  sees  the

space,  it  will send the command to the PC, then return  to  DOS

automatically.

Note: After transmitting a command, REMOTE.OBJ is allowing the PC

a  certain number of seconds to complete it before giving  up  on

getting the ramdisk status. If you find that not enough time  has

been allowed, let me know so I can add some more. (If you  choose

to skip the rerun prompt, you won't be getting the ramdisk status

after command completion.)

3) MAKADISK.EXE

The purpose of this program is to allow you to take a PC file and

convert it into an Atari ramdisk image so your Atari can read it.

For  instance, you may have a modem on your PC only, and want  to

download  Atari files. Or, you may want to write  program  source

code on your PC, because it has an 80 column screen, and send  it

to the Atari for compiling.

At present, this program is basic - it allows you to put only one

file to a standard single density ramdisk image. You should  then

boot the Atari up with SIO2PC and copy the file to a real disk or

a  "full featured" ramdisk image (one created by SIO2PC and  your

Atari  DOS).  Then you may as well delete  the  MAKADISK  created

ramdisk  image, it has served its purpose. You can't  write  more

files to it, the VTOC isn't correct.

Note that another utility may be useful. I want to make one,  but

they are available in public domain (PC Magazine's utilities disk

has  one).  The utility needed is to translate PC "end  of  line"

codes (CR and LF) to the Atari EOL code, which is hex 9B. If  you

are  translating text files, you will probably need this.  Binary

files (such as executable programs) shouldn't generally be trans-

lated.

Note that MAKADISK asks for three things:

1.  The  name  (or pathname) of the PC file to  be  converted  to

Atari.

2. The name of the PC's ramdisk image file. This is the name  you

will  give SIO2PC when you load the ramdisk image. I use an  .ATR

extension for all my ramdisk images stored on the PC, but you can

use  what  you want. Note, MAKADISK doesn't tell you  if  there's

already  a file by that name, so be careful, existing files  will

be overwritten.

3.  The name of the Atari version of the file. This is  the  name

you will see in the first position on your Atari screen when  you

ask DOS for a directory.

4) FILE2PC USAGE INFORMATION

FILE2PC.OBJ  is  an Atari program found on  your  ATARI.ATR  disk

image file located on the distribution diskette. If you run  this

program  on your Atari, it will cause SIO2PC to go into the  file

transfer mode. Just follow the prompts on the Atari. You can read

the  technical  details in the TECHNICAL INFORMATION  section  if

you're  interested.  This file transfer function  has  been  much

requested.  A  lot  of people seem to have text  files  on  their

Atari's  and want to transfer them to their PC's so the can  pick

them  up on the PC's word processor. I should tell you  that  the

usual  warnings  apply  to this: Different  word  processors  use

different  formatting  control codes. This is true  even  if  you

aren't going to a different computer. Plus, the Atari has its own

method  of ending a line. It uses an EOL (9B hex or 155  decimal)

to  tell its editor to drop the cursor down one and move  to  the

left  margin. The PC, on the other hand, uses two codes, CR &  LF

or  carriage return and line feed. They are hex codes 0D  and  0A

(decimal  13 and 10). There are utilities available which  trans-

late  codes like these. There was one called CHANGE on  PC  Maga-

zine's utilities disk. Unfortunately, it only works with files up

to 40K in size. I want to write one without this limit. A  common

denominator is the so called ASCII file, which is free of specif-

ic  non-standard formatting commands. (Like this file.)  If  your

Atari word processor will output a plain ASCII file, you may want

to  use the option before sending it to the PC. Some  Atari  pro-

grams will allow you to use an OS feature which lets you PRINT to

a  disk file. This should yield a plain ASCII file. Also, if  you

wanted  to send it to your PC's printer, it should definitely  be

ready to go in this format. Anyway, you will probably want to use

a  filter program to change your 9Bh's to CR/LF's as  a  minimum.

Note  that 9Bh looks like a "cents" symbol on my PC. To  see  it,

hold down ALT on the PC and type 155 on your numeric keypad.

Rev  2.3  note: Now that the PRINT_THRU function  allows  you  to

"print"  to  a PC file, you have another method  of  transferring

your  Atari  text  files to PC files. And  since  the  PRINT_THRU

option has EOL to CR/LF conversion capabilities, it may be easier

to use in some cases than FILE2PC. Remember, Atari DOS allows you

to COPY a file to P:.

5) FUTURE PLANS, CURRENT PRICING

I  now  feel  that SIO2PC is becoming a  pretty  mature  program.

However, there seems to be no limit to the number of enhancements

one  can  dream  up. I would like to take a stab  at  making  the

system capable of copying copy protected disks, with the  ability

to  run  them from SIO2PC storage without the original  disks.  I

don't really see SIO2PC as some sort of copy protection  cracker,

but it is a pain for users to have this system and still have  to

rely  on  an Atari drive to run some of their software.  I'm  not

sure how effective this system will be, because I'm not up on all

copy protection techniques, but ... we'll see.

On the hardware side, I am still trying to simplify the  assembly

of  the  interface.  It takes far too much  time  for  the  price

charged  to wire one up. So, I may soon make the  DB-9  connector

mount on the board and protrude thru the case, just like the ones

on your PC do. This will save a lot of time stripping cables  and

wiring/assembling  connectors. It will also mean that the  inter-

face will need a DB9S to DB9P or DB25S to DB9P cable to intercon-

nect it to the PC.

Please see BUILD_IT.DOC for the current prices. I had hoped

(and  still  do) to hold the price as low  as  possible.  Shortly

after  receiving mention in Antic, I discovered a hardware  prob-

lem which made it necessary to double the complexity and cost  of

the  interface. SIO2PC is still just a hobby to me. I don't  hope

to  recover my programming time (about 2 years). But I  seriously

underestimated the time needed to assemble the interface, and  do

hope  to  get some reward for that time. (Since  soldering  isn't

fun*, like programming.) Since I've sold only a few interfaces so

far  (about 20 as of July 1990), I haven't had a  custom  circuit

board built. Therefore, I have to hand wire the circuit on proto-

type  boards,  with  quite a few jumpers  required.   What's  the

bottom  line  of  all this whining? I expect to  have  to  either

streamline  assembly, raise my price, or both. I've also  consid-

ered making the software "shareware" and publishing the schematic

so users could pay me for the software and build their own inter-

face from scratch or optionally order from me. Any advice?    ***

DONE!!!

*Building one is fun. Building 10 or 20 is drudgery.

6)      REVISION LOG:

1.00:   First release

1.01:   Now  uses  default addresses for COM 3 & 4 if  BIOS  data

doesn't  give addresses. Added option menu item "O",  sub

option "I" to allow ignoring CTS, "I" is now the default.

If your status line will show "H" in the Atari field with

the  Atari "on", you can toggle this function. I  believe

the  program functions fine while ignoring CTS,  however,

and may eventually delete the input entirely.

1.02:   Changed the stack pointer to be 256 bytes at the  program

end.  This means the program now only uses about  8K  in-

stead of a full 64K segment. Now, on my system with  only

DOS and a 10K Mouse driver installed, I can get 3 -  143K

ramdisks  (as usual) plus an extra 92K ramdisk. Killed  a

bug  which  caused the program to not recognize  SIO  bus

command 022h, which is "format in DOS 2.5  1.5  density".

Added  clear of all disk info lines every update  when  I

noticed  that garbage sometimes remained from  old  file-

names. This change is cosmetic only. Reduced the  timeout

value  which the disk returns to SIO. This should  reduce

the  time  the atari waits when it is  given  an  invalid

format  command. Changed the method used to  put  command

codes to the status line. Now even invalid command  codes

will get listed, adding to ease of de-bugging. Got rid of

a  bug  which could cause an invalid  error  report  when

writing  a ramdisk back to an existing filename.  Changed

the  method used to control the RTS line. Now  the  other

lines of the MCR aren't affected. Note: These errors  are

primarily  the result of switching from one assembler  to

another. (now on QUICK ASSEMBLER).

1.03    4/3/90 Rewrote the routine for generating time delays.  I

had been reading the refresh timer. Now, I'm reading  the

system  timer and using a different method which should be

more reliable and compatible.

1.04    4/10/90 Added option "E" under Options. Now, the user can

manually input port addresses. The program also now  puts

to the  screen the actual address used, whether manual  or

automatic.  Put the address in Hex format.  The  standard

addresses  for COM 1, 2, 3, & 4 are: 03F8, 02F8, 03E8,  &

02E8.

1.05    Added some diagnostics. Now, on initialization of a PORT,

the software reads back register LCR and gives a  warning

message if doesn't read as programmed. Then, the port  is

tested by sending a byte in the  loopback mode. A warning

is  given  if  it fails. If these  warnings  occur,  your

program will probably fail. Either you have an incompati-

ble  COM  PORT, or the program is using  the  wrong  PORT

addresses.  Check  your documentation and  use  "E",  de-

scribed  above to set the PORT addresses. Also, the  pro-

gram now always sets up the UART chip (serial port  chip)

directly,  instead of using BIOS. Also in this  rev,  the

timer chip, timer #0, is now programmed in mode 2,  count

=  0 for a period of 55 milliseconds. This is the way  it

should  already be, but now I'll take no chances.  I  re-

placed  my "long time" timer routine, the one I  use  for

keeping  warning messages on the screen for 2 to  3  sec-

onds. The old routine would occasionally lock up, or take

far  too long. Now, I have redirected the timer 0  inter-

rupt for this purpose. Rev. 1.05 fixes a small bug  which

could result in extra characters on the filename line. It

also fixes a bigger bug - the program used to write to  a

random address (Segment Register wasn't initialized) when

it  thought it was putting the default port address  into

the BIOS data area, after it had been found to contain 0.

Now it just leaves the 0 alone.

1.06    Quite a few changes. Added the FILE2PC subroutine,  which

works with the FILE2PC.OBJ Atari program. This gives  you

the capability  of sending Atari files to the PC to become

PC  files, rather than Atari disk images. This means  you

can  read the files with your PC's word  processor.  Rev.

1.06  also changed the logic on switching the  PC's  data

out line to tri-state (off line) level when it is not  in

use.  Some  people were having trouble with  erratic  and

slow  operation, and the switching of this line  was  be-

lieved  to be causing spurious pulses. Now it only  turns

off  when a non-SIO2PC device is addressed, less that  1%

as  often as before. Another addition to rev 1.06 is  the

addition  of  double  density.  Now you  can  choose  183K

ramdisks  and double density sectors. Note  that  sectors

1,2,  &  3 are single density, so the ramdisk  can  be  a

bootable  disk. Currently, you can only specify a  double

density  ramdisk by choosing "Z" instead of "C"  for  the

"Create Ramdisk" function. Then use a DOS that recognizes

double  density, such as MYDOS. I also made some  changes

to the LOOPBACK test performed on the UART (serial  chip)

upon  initialization.  I found I was  getting  occasional

"failed  loopback  test"  messages  when  everything  was

really  OK. I did some fine tuning and  haven't gotten  a

bogus message recently.

I  also added a function ("R"), which allows you  to  re-

store a trashed screen. Despite my best efforts, DOS will

sometimes manage to sneak in one of its messages, causing

a  scroll and messing up the screen. Use this feature  to

fix it. (As of rev 2.8, ESCAPE does the same thing.)

1.07    I added a directory display function, under Options.  You

can  just  give  it a filename,  including  wildcards  to

search  the current directory, or a complete pathname  to

search other directories. Now, when you're in SIO2PC  and

can't  remember  the filenames of your disk  images,  you

don't  have to quit the program to find them. I name  all

my  Atari disk images with the  extension ".ATR" and so  I

search for "*.ATR". Also in this release, when you choose

"Quit", the program warns you if you have written to  any

ramdisks but not yet saved them back to disk.

Rev. 1.07 was to include the "PRINT THRU" option, but it's not

quite done. Rev. 1.08 will have it soon.

1.08    PRINT-THRU mode incorporated. Now, you PC can serve as an

Atari  to  printer interface. You can have  your  Atari's

printer  output go to either the PC's printer or  screen.

You  can also choose  whether to convert EOL's to  CR/LF.

Also in this rev, I added an Author credits screen.  This

screen goes away as soon as you give the port #. I  moved

the  "Restore Screen" function from the Options  menu  to

the Main menu. After having Atari output printed to  your

screen just press 'R' to restore the menu.

1.09    Added  feature to allow choice of stripping high  bit  on

PRINT-THRU data. This way, Atari inverse video text  will

come  out  as normal text. (Without, it  would  print  as

garbage.)

2.0     Added  function  to  allow deletion of  a  ramdisk.  This

function fills  in the hole created in memory by  the dele-

tion  and allows the freed memory to be used in  creating

other ramdisks.

2.1     Changed  the Atari program REMOTE.OBJ and changed  SIO2PC

to correspond. REMOTE.OBJ now asks for ramdisk info  from

SIO2PC  at program start and after each  command.  SIO2PC

sends  that info to the Atari. Now, the remote  user  can

get a fair idea of whether his command was successful  or

not. Also, now a leading space in the command string will

cause  REMOTE.OBJ to go back to DOS after  completion  of

the  command  without asking if the user  wants  to  send

another command string.

2.2     Changed  the  point  at which the program  looks  at  the

device number in the command frame and disables the  data

out line if a non-SIO2PC device is being addressed.  This

is in hopes of  curing an interference problem some  users

have  experienced when using real drives and  SIO2PC  to-

gether. 08/05/90

2.3     AARGH!!   Rev. 2.2 still didn't take care of the  elusive

"problem".  So, this version adds a switch  which  allows

the  user to lock SIO2PC's data out line into  the  "ena-

bled" state. This was  verified earlier to work. Also,  I

finally did away with the old CTS or "Atari" line,  which

didn't seem to have any advantage. The idea was to  carry

the 12 volt line from the Atari to the  PC to tell the  PC

it was on. It turned out that some PC's didn't   recognize

it,  or some Atari's didn't have the 12 volts  out.  But,

anyway  the program worked fine without it.  Another  big

change  is the redesign of the user interface screen.  It

has  become  obvious to me that  the  separate  "Options"

screen   was cumbersome and unnecessary. So I've  combined

all  choices onto one screen. This means there  are  some

functional  changes,  for those of you  using  batch  and

remote script files. First, option "O" is no more.  Also,

the  "D"  display status line choice is now  "T",  toggle

status  line. Another enhancement added with  this  revi-

sion: Now, you have a  third option with PRINT_THRU:  You

can specify that the Atari's printer bound output be sent

to a PC file.

Still another change is to the Status Line. Replacing the

"Atari" line status is the DEVID identifier. This is  the

SIO  bus ID of  the device being addressed.  Elsewhere  in

this  README  file are the ID's of various  bus  devices.

Also,  as of this revision, the program updates the  Com-

mand  and DEVID fields even when a non-SIO2PC  device  is

being  addressed.  Now  you can see what  your  Atari  is

telling  other  devices, if it's trying to  address  non-

existent devices, etc.

2.4     Changed  the directory print-out from 2 to 4 entries  per

line.

Now your directory can have more than 90 entries  without

scrolling off the screen. Wrote handlers for the Critical

Error  Vector  and the Control - C vector.  The  Critical

handler is the one which gives you the Abort, Retry, Fail

message if your drive door is open, the printer is out of

paper,  etc. It turned out that this would let  you  bail

out of the program without going through my normal "Quit"

routine.  This  restores  the timer  interrupt,  for  one

thing. Anyway, now you get plain English messages and can

still  bail out if you want, but won't lock up your  com-

puter  anymore if you do! Also added some  obscure  func-

tions  to Print-Thru that used to work with the  original

Atari 40  column printer, the one that plugged right into

the  SIO  port. I don't know if any  software  out  there

sends  these  commands or not. Also added  the  SHAREWARE

message at the end of the program.

Rev 2.5: Adding user command "A" to adjust timing values.

Allows  user to set critical time delays used in the  bus

data  transfers, to try to iron out  differences  between

different  systems. This addition is menu choice "A"  and

adds the TIMING menu. This shouldn't be necessary in  99%

of  cases, but here it is in case. See the "timing"  sec-

tion  or  the TECHNICAL INFO part of this  DOC  for  more

info.

Rev.  2.6: Worked over the PRINT_THRU routine to  make  it

more flexible and more compatible with the 850  interface.

Also moved the default "timings" value to the front of the

program to make it easier to find and change with DEBUG or

NORTON's utilities.

Rev. 2.7: Changed the PRINT_THRU routine so it will detect

a  "disk  full" error when printing to a file,  alert  the

user,  terminate  the PRINT_THRU process.  This  need  was

pointed out in Bob Wooley's AIM review.

Rev.  2.8:  Fixed a bad printer bug caused by rev  2.6  in

which lock up of the PC would occur if you were converting

EOL's  to  CR/LF's and an EOL occurred at byte 40  of  the

printer  buffer. I also got rid of the space  added  after

CR/LF  because it was causing problems. Also fixed so  the

ESCAPE  key works the same as 'R' to restore  the  screen.

Most people jab the ESC key when they get confused.

Rev.  2.9:  Did  a little more work on  the  "disk  full"

problem so it gets addressed anytime you are writing to a

disk: PRINT_THRU, FILE2PC, or WRITE DISK FILE.

Rev.  2.10: More work on Print-thru. Added more  LOCATION

and ERROR  codes to help in trapping errors. Also added a

time  value to the TIMINGS menu to be used when  printing

to a real printer. It causes a wait between records  sent

to the printer.

Rev. 2.11: Added a byte count to the sign off message  so

the  user  can check if a virus has changed  the  program

size.

Rev. 2.12: Added directory function to extract the  Atari

disk  directory from a disk image file.  Now working  for

Atari DOS 2 & 2.5 compatible DOSes only.

7) PROBLEM DIAGNOSIS QUESTIONS

If you have trouble, I would like some information about your

system. (Actually, I'm interested even if you don't have problems.

Please send your comments.)

What kind of Atari? 800? 400? XE? XL? 1200? 600?

What Atari DOS? What type of formatting?

What kind and class of PC? XT? AT? '386? Original PC? Clock Speed?

Has your port been verified functional with other equipment?

What did BIOSDATA.EXE say about your ports?

What version of PC/MS DOS?

What does the status line at the bottom of the program show?

How long does TESTIME1.EXE take to run? (Should be close to

10 seconds.)

Any detailed information from your PC's tech manual about your

serial ports and port addresses in general will be useful.

If you only have problems part of the time, what specific steps

lead to the problem?

Please read the REV_LOG and TECHINFO DOC sections for some more

information which may help with problems. Read also any other

scattered through various files which may be of use to you.

8) POSSIBLE PROBLEMS AND SOLUTIONS

If  you are unable to address certain ramdrives,  such as  3 &  4

(error  160  results), it may be that your DOS isn't  set  up  to

include  these drives. On Atari DOS 2 and 2.5, you can  find  out

by booting in BASIC and giving a PRINT PEEK (1802)  command.  The

resulting  number is the sum of the following: 1 for drive  1;  2

for  drive 2; 4 for drive 3, and 8 for drive 4. So if   you   get

"3," your DOS is set up for drives 1 and 2 only. To fix your DOS,

you  simply  execute a POKE 1802,15 command. Then go to  DOS  and

execute  the "H" WRITE DOS FILES command. Finally, turn  off  the

computer and reboot.

Another  error that has plagued me from time to time is  this:  I

"create" a ramdisk, then try to write to it without formatting it

first.  Remember, if you create a ramdisk from scratch, you  have

to format it. If you load up a stored ramdisk, you don't have  to

format (or create) the ramdisk.

Another  problem which isn't the fault of SIO2PC, comes when  you

try  to format a 92K ramdisk with DOS 2.5's normal  command.  You

must  use  the "format single" command here.  The  normal  format

command  of  DOS 2.5 only works with the 143K ramdisks.  On  some

format  related  errors, the Atari will  retry  "forever"  before

giving  up and reporting the error. So you may think  the  system

has locked up.

Note that on the 64K ramdisk, the program doesn't report back  to

the  Atari  that  the full set of 720  sectors  isn't  available.

You'll  get an error 138 on the Atari if you try to put too  much

data to this ramdisk. A utility is needed to mark the unavailable

sectors  as "used" on your DOS's VTOC. If this all sounds  pretty

intimidating, you may want to steer clear of the 64K ramdisk.

One user told me that his SIO2PC was susceptible to noise pickup

from power cords, etc. (I have been able to run 75' of unshielded

cable near power cords, over air conditioning units, etc, without

problems, so you never know.) Anyway, he said he wrapped the box

in foil and fixed it. I don't know if I'd go that far, but if you

have intermittent problems, you might look at the cable routing.

Yet another potential pitfall: You will usually remember when

you have a real disk installed, not to have an SIO2PC ramdisk of

the same number, since a collision will occur. But sometimes  you

(I)  will  boot up a DOS which automatically installs  a  ramdisk

handler on the Atari, forgetting that there is an SIO2PC  ramdisk

of  the  same  number. When I try to access this disk  #,  I  get

strange results not immediately identifiable as to the cause. So,

if you use Atari ramdisks, watch out for this.

Along  this  same line: I just got a letter from a user  who  was

having trouble with the PRINT_THRU function. It would stop print-

ing  at unpredictable places. Then he noticed that the Atari  was

plugged into a surge suppressor, but the PC and printer  weren't.

When he plugged those items into a surge suppressor, the  problem

was  fixed.  So this is a major Ah-Ha! for anyone  having  random

problems  in  this area. Printers generate a  lot  of  electrical

noise. I never have this problem on my system, but no wonder:  My

Atari  and  PC  aren't on the same desk, they  are  powered  from

separate  wall  receptacles,  and each has  its  own  surge/noise

suppressor.  Let me suggest a couple of other things:  Make  sure

your  SIO2PC cable and your printer cable are kept  separate.  If

your printer and serial cables have thumbscrews, tighten them up.

I  noticed this made a big improvement on the amount of noise  my

computer  outputs  to my ham radio. If you can't get rid  of  the

noise,  here's another solution, but not quite as clean:  Set  up

SIO2PC  to  capture the Atari's printout to a  file.  Then  leave

SIO2PC  and use the PC DOS COPY command to copy the file  to  the

printer  (PRN). Use the "binary switch" option, or  your  printer

control codes won't make it thru the copy process.

Rev. 2.3 added a switch (function "K") to allow locking the  data

out  line into the enabled state. This is for a user or  two  who

had a specific problem which occurred when the data line went  to

the inactive state automatically. You should leave this switch in

the  Auto  state. Otherwise, when you try to  address  non-SIO2PC

devices, interference will result. However, if you note that when

you  address an SIO2PC drive and experience hesitation,  sputter-

ing,  etc., try locking the data line on and see if it helps.  (I

now  believe the TIMINGS MENU modification is the real  cure  for

this type problem.)

Some people have told me that some copy protected programs  which

have  been  "broken"  will run from a real  drive  but  not  from

SIO2PC.  Recently,  one guy told me of a solution:  He  uses  the

TIMINGS menu of SIO2PC to slow down SIO2PC's I/O rate so it looks

like  a real drive. Appapently, some protected programs time  the

"sector get" and "sector put" processes to make sure they  aren't

running  out of a ram drive (indicating that the  protection  has

been  broken).  Since SIO2PC is a small program, one  could  have

customized  versions which are slowed down just for use on  these

problem programs. (Elsewhere in this DOC is info on how to perma-

nently alter SIO2PC's timings.)

In  my operating instructions, I tell you to turn off  your  real

drive  #1 and then boot from the SIO2PC disk #1. I haven't  heard

any complaints about this, but I recently borrowed a 1050  drive,

which, when I turned it off, interfered with the serial bus and I

couldn't  read  the SIO2PC drive. The solution was  to  turn  the

front lever on the 1050 to the EJECT position but leave the  1050

on.  Apparently this told it not to answer the call, and it  went

through to SIO2PC OK.

9)          USER'S QUESTIONS

1) Does the program reformat my PC's hard disk?

No! Your Atari disk images are stored on your PC as ordinary

PC files.

2) Could I use the hardware part of the SIO2PC system as a gener-

al  purpose  RS-232  interface? Like, to connect my  Atari  to  a

modem?

Hmmm... This is a good question, and the answer is sort  of,

"yes and no." Electrically, yes you have a true RS-232 connection

coming out of the interface. However, note the following:

RS-232  includes  several handshaking lines in  both  directions.

But,  most communications setups only use a few,  and  optionally

can  be  told by software to disregard those they do  have.  I've

seen  computer to computer PC data links that had just  Data  in,

Data  out, and ground (via RS232 serial port). My system has  one

handshaking line from the Atari to the PC, the COMMAND line which

goes to RI on the PC. You can often "dummy up" inputs to fool one

end  of a link into thinking the other end is saying, "ready."  A

null modem cable is usually made up this way.

Bit rate (BAUD) and software on the Atari end: On the Atari,  the

SIO subsystem of the OS always works at a fixed rate: 19.2  Kilo-

baud,  except for the cassette: 300 baud. Also, it uses a  strict

protocol  to send data: command frame, acknowledge,  data  frame,

acknowledge, complete. But if you want to write your own software

(or you have some already written), you can directly program  the

Atari's  POKEY  chip  (UART) to any baud you want,  and  you  can

directly  put  data  into its transmitter for  sending,  and  can

directly  read incoming data from the receiver. (My  notes  under

ATARICOM.DOC  talk about how I did this once.) When you do  this,

you  are  completely  bypassing SIO and running  the  whole  show

yourself. You can also control the handshaking lines yourself. To

get  into  something this serious, you need the  Atari  Technical

User's Notes. It's fun to learn, but takes some time.

3)  How  come  SIO2PC is a dead end - no extra  port  for  adding

another device?

I'm  finding out this bothers some people. I figured,  you  could

alway make this the last device in your chain. But I guess  other

manufacturers  figured the same. It would be vastly more  compli-

cated for  me to build a box with a receptacle or two for SIO  in

it.  They generally come as PC board mounted connectors,  not  as

"in line" types like the plug I do use. So my board would be much

bigger,  I would be hand cutting rectangular cutouts in the  box,

etc.  However, someone had a suggestion, if you are building your

own:  He said the "newer" cassette recorder (the 1010?)  had  two

SIO2PC jacks in it. You could get a surplus 1010 junker for a few

bucks, gut it except for the connectors, and build your SIO2PC in

this  box. Alternately, you could just buy a couple of the  jacks

from your friendly Atari parts supplier and design them onto your

board.   Here's  another  idea:  build the  interface  into  your

Atari.  Mount  a DB-9 in the Atari's case.  Then, your  SIO  port

would still be free.

4)  Will SIO2PC run on an Atari ST running PC ditto?

From  what I've heard so far, no. SIO2PC does a lot of direct  to

the  chip I/O, bypassing both MS-DOS and the PC's built  in  BIOS

routines. I suspect that PC ditto doesn't emulate the PC down  to

this  level. However, if there are any ST experts out  there  who

want  to work with me on it, I think we could modify the  program

to  make  it work. I configure the serial  chip  (UART)  directly

because  the  PC BIOS doesn't have 19.2 kilobaud  as  an  option,

although the chip will do it. I input and output data from/to the

UART  directly for speed. I have also reprogramed the PC's  timer

chip and have a timer driven interrupt running. I think all  this

could  be worked out if we had the right info. How about  it,  ST

users?

5) Will you give out your source code for SIO2PC?

I don't routinely give it out. But if you would like to have  the

source code just to see how it's done, or to make some  modifica-

tions, or even to "borrow" the basic I/O routines for use in your

own  non-commercial programs - write and let me know.  SIO2PC  is

written  entirely  in assembly language for the  MICROSOFT  QUICK

ASSEMBLER.

10) WHO'S NICK KENNEDY?

I'm  an  electrical  engineer, but I don't  work  with  computers

professionally.  That  part is a hobby. I got the  Atari  800  in

1982,  mostly  so  I could play  Asteriods.

Data  communications  interests me. I have written a  Morse  code

program to interface the Atari with my Ham Radio station.  Later,

as  a sort of evolutionary precursor to SIO2PC, I wrote  programs

linking my two Atari 800's together, first via their serial (SIO)

ports,  and  later via their joystick ports. The  Atari  equipped

with  a disk drive would send files to the one without.

I thoroughly enjoy programming in assembly language, but hope  to

master  "C"  someday, since it's so popular.

To me, the hardest part of programming is finding a worthy cause.

The are so many programs out there, commercial, free, and  share-

ware - all good, that it's hard to find a niche that hasn't  been

filled. I think SIO2PC fits the bill.

As  I've said before, I'm very interested in your comments,  sug-

gestions,  questions, praise and condemnations. So, if you  built

it yourself and have a comment, let me hear from you, even if you

weren't moved enough to pay a shareware fee. I won't harangue you

about it.#############################################################################################################################