USING THE SIO-2-PC INTERFACE SOFTWARE

This  is  a brief description of the Release  2.12  version.  The

software is menu driven, and I hope, fairly self-explanatory.

If you specify a command by mistake, ESC will usually return  you

to  a  higher level of the menu. Here is a run-down of  the  com-

mands:

CREATE  RAMDISK AREA: Reserves ram area for a ramdisk, single  or

double  density. Prompts for size from user. Reports when  memory

isn't available. After creating a ramdisk, you must  either  load

in  a  disk image (next option) or format the ramdisk  with  your

Atari.  After  you create a ramdisk, its status is shown  on  the

appropriate ramdisk status line. The size and density are  shown.

Also,  a single letter field shows an 'N' if you haven't  written

to the ramdisk, 'W' if you have.

LOAD  DISK  FILE INTO RAMDISK: This command prompts  for  ramdisk

drive # and disk filespec. (I think that's PATHNAME in PC  talk.)

You  can use this option even if you haven't already created  the

ramdisk  with the option above. If you have created the  ramdisk,

the  sizes of the ramdisk and the disk image file need to  match,

or  an  error will be reported. Various DOS errors will  also  be

reported if there is anything wrong with the file you  specified.

The file information specified is appended to the ramdisk  status

line for your reference.

WRITE RAMDISK BACK TO DISK FILE: This is essentially the  inverse

of the above with a few exceptions. You have the choice of  writ-

ing back to the file you read from (if any), or you can specify a

new file for the disk image storage.

SET COM PORT NUMBER: This command lets you choose which port  you

are going to plug the interface into, assuming you have more than

one.  The screen will show the # of the port you have  specified.

Note  that the program begins with the port # query. So,  if  you

didn't  enter it in the batch file, or on the command  line,  you

must now enter it before proceeding. Read TECHINFO & REV_LOG sec-

tions of README.DOC for more info.

DISPLAY  STATUS  LINE  (TOGGLE): The program  status  line  gives

information  about  what the program is doing, if  anything.  The

program  writes directly to screen memory for this function  (for

speed).  Because  this  may cause noise on the screen  of  a  CGA

computer display, you have the option of turning it off.

EXCHANGE RAMDISK I.D. NUMBERS: The purpose of this command is  to

allow you to change the number of the default, or bootable  disk.

You could for instance, boot up with a physical drive as drive #1

and  load  files  into ramdisk #2, then turn off  the  drive  and

change  #2 to #1. Note that you can swap a defined  ramdisk  with

one which doesn't yet exist. No problem.

PRINT-THRU: Tells SIO2PC to intercept your Atari's printer  bound

data.  You  can choose to send this data to  your  PC's  printer,

screen, or disk file. You can also choose to translate Atari EOLs

to PC CR's or CR/LFs and clear the high data bit.

LOCK  DATA LINE ON: Usually, leave in Auto. One user needed  this

to cure a bug. See README.DOC for more info.

ENTER  PORT  ADDRESS: Allows you to specify the port  address  in

hex. Usually, the defaults are OK and you don't need this.

UNINSTALL  RAMDISK:  Deletes a ramdisk and frees its  memory  for

more ramdisks of different size of density.

ADJUST  BUS TIMINGS: You should never need to do this. Again  for

the one user with a cantankerous system. See README.DOC for more.

QUIT  PROGRAM: Returns the PC to DOS. Warns you to  save  changed

ramdisks.

DIRECTORY  SEARCH  Allows you to look at a PC  directory  without

leaving  SIO2PC.  You can use the usual PC  path  specifiers  and

wildcards.  Also, you can look into the directories of your Atari

disk image files.

USING COMMAND TAILS: The SIO-2-PC software allows you to simplify

configuration of the program through the use of the command  tail

feature  of DOS. The command tails consists of text found on  the

command line after the program name. An example will illustrate:

If you start the program by the command SIO2PC 2 C22 L1A:\DOS.ATR

R,  it tells the PC to: Run SIO2PC, use Port 2, create a  ramdisk

as disk #2, size 92K, Load a disk image into ramdisk #1 from file

A:\DOS.ATR.

USING  BATCH  FILES: The real advantage of command  tails  is  in

using  them  from batch files. If the above text  (SIO2PC  2  C22

L1A:\DOS.ATR  R) is stored as a batch file, you can set  up  your

Atari ramdisks for a computing session just by typing the name of

the  batch  file.  You could have different  batch  files  called

GAMES.BAT, PROGBAS.BAT, PROGASSY.BAT etc., automating set-up  and

making  things  much easier for you than remembering a  bunch  of

commands  and file names. You should put a space after  the  last

command of the batch file. You may have noticed that there's no S

before the PORT # in the batch file. That's because SIO2PC starts

up expecting a PORT # entry.

ERROR CODES: SIO2PC doesn't have any error numbers of its own (it

uses  plain text messages). The occasional number such as AX =  #

refers  to the DOS error code received when attempting file  I/O.

The  program uses DOS commands 3Dh to open files, 3Eh  to  close,

3Fh  to read, and 40h to write. Refer to your DOS manual for  the

error codes returned in AX.

Other Included Programs: There are a few utility programs includ-

ed on your disk, in addition to SIO2PC:

ACOMINIT.COM:  This program will initialize your serial  port  to

tell the interface to stay disconnected from the SIO bus. Reason:

If  you  have the Atari on, and the PC on, but you're  not  using

SIO2PC,  the PC's serial port may be telling the interface to  be

"active,"  thus interfering with operation of the Atari's  serial

bus.  The  program expects a single number on the  command  line,

which  is the number of the port connected to the interface.  You

should copy ACOMINIT.COM to your root directory, and add the line

"ACOMINIT  #" to your autoexec.bat file, where # is 1,2,3, or  4.

Note  that there may still be some conditions in which  the  non-

operating  but connected SIO2PC will interfere with the  SIO  bus

communications. If that occurs, just unplug the interface.

REMOTE.OBJ:   This program is an Atari program found on the  ram-

disk  file ATARI.ATR on your disk. It allows you to  control  the

SIO2PC program by remote control, from the Atari. Get into DOS on

your  Atari. Load the program and then enter SIO2PC  commands  as

though you were entering them from the PC's keyboard! You have to

anticipate  the prompts and include the answers in  your  command

string.

BIOSDATA.EXE:  This program runs on your PC. It checks your  BIOS

data area, and uses a BIOS function to see how many serial  ports

you have connected, and what the port addresses are. If you  have

trouble, run this program and let me know what it shows.

TESTIME1.EXE  Tests  your PC's timer to be  compatible  with  the

SIO2PC  program. Run and time it if you have trouble.  It  should

take 10 seconds.

FILE2PC.OBJ:  This  is  another  Atari  program  found  on   your

ATARI.ATR  disk file. It allows you to send your Atari  files  to

the  PC  as  PC files instead of as Atari disk  images.  See  the

README.DOC file on the diskette file for more information.

MAKADISK.EXE  This  is  a PC program which takes a  PC  file  and

converts  it to an Atari disk image which you can then load  with

SIO2PC and read with your Atari.

GETTING STARTED

First, you should make a copy of the distribution disk for  safe-

keeping.  Now, plug SIO2PC into the PC the Atari's  Daisy  chain.

Start SIO2PC by typing SIO2PC # [ENTER], where # is the number of

the COM port you are using. Now, load up the supplied Atari  disk

image  with the commands: "L", then "1", then A:\ATARI.ATR  (This

assumes  the floppy is in drive "A.")  Now, the status  line  for

ramdisk  #1 on the PC should show that this ramdisk is  resident.

Turn  on your Atari and try to boot from the PC. You  should  see

some  action on the status line: Command: usually "R"; DEVID:  31

(for disk 1); COMLINE: "H", flashing to "L" intermittently.  Now,

ask  for a directory of D1. If all is well, you should  see  that

the ramdisk has REMOTE.OBJ and FILE2PC.OBJ (described  elsewhere)

and TANK.OBJ (a game written by my brother) and KEYER.OBJ on it.

Now, let's create and format a new ramdisk. Press "C" on the  PC.

Answer the prompt with "3" for disk # and choose 92K or 143K (DOS

2  OR DOS 2.5) for the size. The status line now shows  that  the

ramdisk exists, with "N" for not written.

Now,  use the "I" command of DOS to format the ramdisk, and  then

write DOS files to it.  Now, you're going to want to start moving

files  from your Atari disks to PC disk image files. Use the  "X"

command to move your ATARI.ATR ramdisk out of disk #1 position by

swapping  it with, say, #4. You can now turn on your  real  drive

and  let it be #1.  Now, just copy files from the real  drive  to

the  new ramdisk. When it's full, or you're through, use the  "W"

command to write the disk image to your PC's hard or floppy  disk

for permanent storage.  You can also use a sector copier to  copy

your Atari boot disks to PC disk images.

I  suggest  you put ACOMINIT into your autoexec.bat file  as  de-

scribed  elsewhere, and also create batch files to  automate  the

set-up of SIO2PC.

#######