D I S C L A I M E R

I  have tried to ensure that my software is free of  harmful  de-

fects (bugs) and of viruses.  However, either could slip  through

my  defenses.  Use the software at your own risk.   Similarly,  I

believe  my hardware design to be in accordance with  good  prac-

tice,  and I have a number of satisfied users.   However,  wiring

errors,  errors in my design, and configuration  differences  are

possible and I cannot guarantee that these problems will not lead

to damage to your equipment or software or other unforeseen  dam-

ages.   Furthermore,  if  you choose to  construct  my  interface

yourself, you must be experienced with electronic building  tech-

niques.   Even  if  you are experienced, you could  burn  or  cut

yourself or experience other damage or injury.

I consider the SIO2PC system to be a service to the Atari  user's

community. Although I ask a shareware contribution, and do make a

charge  for  assembling interfaces or kits, I still  consider  my

efforts  to  be a hobby and a service.  IF YOU HAVE SENT  ME  ANY

PAYMENT FOR ANY SOFTWARE, KIT, OR ASSEMBLED INTERFACE AND DO  NOT

WISH TO ASSUME FULL RESPONSIBILITY FOR USE OF OR CONSTRUCTION  OF

THESE  ITEMS,  YOU MAY RETURN THEM TO ME AND I WILL  REFUND  YOUR

MONEY.   IF YOU ARE CONSTRUCTING THE INTERFACE AND CANNOT  VERIFY

THE  ADEQUACY OF THE DESIGN ON YOUR OWN, SEEK THE ADVICE  OF  ONE

COMPETENT IN ELECTRONICS.  OTHERWISE, DO NOT ATTEMPT TO BUILD AND

USE THE INTERFACE.

BUILDING THE SIO2PC INTERFACE

BACKGROUND ON HARDWARE REQUIREMENTS:

The RS-232 serial port on your PC and the SIO serial port on your

Atari  8  bitter are quite similar. There are  some  differences,

however  and  the  SIO2PC interface is  designed  to  accommodate

these:

1) The Atari puts out (and expects as inputs), TTL level signals.

In  general,  +5 volts for logic 1 and 0 volts for logic  0.  The

RS-232 standard is bipolar: roughly +5 to +15 for logic 0 and  -5

to  -15  for logic 1. Note that this is also inverted  from  TTL,

that is, the more positive signal is logic 0 here. So, the  first

hardware  requirement  is  to convert TTL to  RS-232  levels  for

signals  from the Atari to the PC, and RS-232 to TTL  levels  for

the reverse direction.

2) The RS-232 standard is designed for one to one  communication.

The  "data out" line of one device goes to the "data in" line  of

another, and vice-versa. The Atari SIO system is a data BUSS. The

Atari  computer's data out line can connect to several  peripher-

als, and their data out lines all connect to the one data in line

on the Atari computer. This means there is a potential for inter-

ference  if  more than one peripheral tries to talk at  the  same

time. This is solved by having the peripheral devices keep  their

data out lines in Tri-State (disconnected) mode when they  aren't

being spoken to. This is the next hardware requirement, the  data

line from the interface to the Atari must have Tri-State capabil-

ity which can be enabled/disabled as necessary  by the PC.

3)  Handshaking. The Atari uses only one handshaking line on  its

SIO bus. This is called the COMMAND line. The Atari computer uses

it  to tell all peripherals to "listen up" because a  command  to

one  of you will follow. The PC's serial port has  several  hand-

shaking  lines,  so the interface will just convert  the  COMMAND

line's  TTL level to  RS-232 and send it to the PC's  "RI"  hand-

shaking line. Also, another PC handshaking line (RTS) which is an

output,  will be used to  enable/disable the Tri-State  data  out

line discussed above.

4)  Baud,  Word  Size, Start/Stop bits, etc:  The  philosophy  of

SIO2PC is to require no software patches on the Atari. This is so

you  can BOOT the Atari from the PC, you can use any  DOS,  there

will  be  no software conflicts, etc. Fortunately, we  got  lucky

here.  The Atari serial bus runs at 19,200 bits per  second.  The

PC's BIOS doesn't allow speeds this high, but the hardware  does,

so  the SIO2PC software just programs the chip  directly  without

DOS  or  BIOS. The other parameters listed are also  easily  pro-

grammed into the serial chip (UART) on the PC by software. So, if

we  just handle 1 thru 3 above, we are set to go with  regard  to

hardware.

CIRCUIT DETAILS:

I now have files which draw the schematic on your PC screen using

built  in  ASCII symbols. This means you don't need  any  special

viewer   program  to  see  them.   The  files  are   1SIO2PC.ASC,

2SIO2PC.ASC,  and  3SIO2PC.ASC.  Each takes up one  screen.   The

batch  file VIEW-CKT.BAT displays the three on screen in  succes-

sion.  You can also dump these files to your printer.  Also  note

that  I have .GEM files which draw less clunkly versions  of  the

same  thing,  plus a board layout using a Radio  Shack  prototype

board. Write me for copies.

Chips used:

1) The Harris ICL232 (or MAXIM MAX232) is a single chip RS-232 to

TTL and TTL to RS-232 converter. It includes two gates to convert

TTL to RS-232 and two to do the reverse. The beauty of this  chip

is the internal 5 volt to bipolar power supply converter. Normal-

ly, you would need a separate plus and minus 12 volt or so supply

for  the  RS-232 levels, but this chip takes the  5  volt  supply

(from the Atari) and generates all the voltages needed.

2)  The  74LS368 hex tri-state inverter. This chip  has  six  TTL

inverters,  in two sets. Each set has it's own  Tri-State  enable

line.

"CIRCUIT DIAGRAM" NARRATIVE:

1) Data from Atari to PC:

Pin 5 of SIO connector to pin 11 of '232.

Pin 14 of '232 to pin 2 of PC port connector.

2) Data from PC to Atari:

Pin 3 of PC port connector to pin 13 of '232.

Pin 12 of '232 to pin 2 of '368.

Pin 3 of '368 to pin 12 of '368.

Pin 11 of '368 to pin 3 of SIO connector.

3) COMMAND line, Atari to PC:

Pin 7 of SIO connector to pin 4 of '368.

Pin 5 of '368 to pin 10 of '232.

Pin 7 of '232 to pin 9 of PC port connector.

4) GROUND connections:

Pin 6 of SIO connector to pins 8 and 1 of '368 and

to pin 15 of '232 and to pin 5 of PC port connector.

See also: (7) & (8) for other ground connections.

5) +5 Volt connections:

Pin 10 of SIO connector to pins 16 and 14 of '368 and to

pin 16 of '232. See also: (7) & (8) for other +5 V

connections.

6) Tri-State enable control:

Pin 7 of PC port connector to pin 8 of '232.    Pin 9 of

'232 to pin 10 of '368.

Pin 9 of '368 to pin 15 of '368.

7) Four 22 microfarad, 25 volt electrolytic capacitors:

(Needed for the +5 V to +/- 12 V converter)

a) (+) to '232, pin 1; (-) to '232, pin 3.

b) (+) to '232, pin 4; (-) to '232, pin 5.

c) (+) to ground, (-) to '232, pin 6.

d) (+) to '232, pin 2, (-) to +5 Volt line.

8) 0.1 microfarad 25 volt disk capacitor:

(For noise suppression/de-spiking)

+5V line to Ground, near '368 chip.

9)  I  connect the drain or shield (bare) wire in the  Atari  I/O

cable  to the drain wire in the PC cable, but I don't connect  it

to a DB-9 connector pin.

If you wish, you can use this information to draw a schematic

using gate symbols (a triangle on its side). The pins for the

gates of the two chips are:

IC-232:

TTL in, RS-232 out              RS-232 in, TTL out

INPUT   OUTPUT                  INPUT   OUTPUT

11      14                      13      12

10       7                       8       9

SUPPLY: +5 VOLTS: pin 16, Ground: pin 15

Capacitors for converter: See (7) above.

74LS368:

SECTION 1 INVERTERS             SECTION 2 INVERTERS

INPUT   OUTPUT                  INPUT   OUTPUT

2       3                       12      11

4       5                       14      13

6       7

10       9

ENABLE SECTION 1:               ENABLE SECTION 2:

PIN 1                           PIN 15

Note: LOW enables, HIGH for Tri-State

SUPPLY: +5 VOLTS: pin 16, Ground: pin 8

CONNECTOR INFORMATION:

Most  PC serial ports use a 9 pin connector known as a DB-9.  The

connector has male pins. Therefore, the connector you put on your

interface connector should be a DB-9F, with sockets. This is  the

view of the connector on the computer. It is also the view of the

rear of the DB-9F connector (the end you solder the wires to):

1 2 3 4 5

6 7 8 9

The Atari serial port uses a specialized connector which I  have-

n't  found anywhere else. Looking into the jack on the  computer,

it looks like this:

2  4  6  8  10  12

1  3  5  7  9  11  13

You  can  get the connector itself from  Atari  parts  suppliers.

However, I prefer to buy an I/O cord with connectors on both ends

and  cut  it in half. Then you can use an ohmmeter  to  find  out

which wire is wired to which pin.

25 Pin Serial Port Connector:

Many  PC serial cards use a 25 pin connector instead of a 9  pin.

Here are the equivalent pins if you have the 25 pin:

DB-9      DB-25

2         3

3         2

5         7

7         4

9         22

PARTS LIST FOR SIO2PC KIT

_____     1 - BOX, /w LID, SCREWS (R.S. 270-220, 4 X 2 X 7/8)

_____     4 - 22 uF ELECTROLYTIC CAPACITORS, 25 volt

_____     1 - 0.1 uF DISK CAPACITOR, 25 volt or more.

_____     1 - INTEGRATED CIRCUIT, I.C # 1 (74LS368)

_____     1 - INTEGRATED CIRCUIT, I.C. # 2 (ICL-232)

_____     1 - SIO CABLE & CONNECTOR (Atari I/O cable cut in 1/2)

_____     1 - 5 CONDUCTOR CABLE

_____     1 - DB9F OR DB 25F CONNECTOR (as req'd for your PC)

_____     1 - CONNECTOR HOOD

_____     1 - CIRCUIT BOARD (R.S. 276-150)

_____     6" BARE WIRE

_____     6" INSULATED WIRE

_____     2 - 16 PIN IC SOCKETS

R.S. means Radio Shack. You can get most of the common stuff from

there. I drill a hole in each end of the box and install grommets

(from  Radio  Shack) in them. Then I feed both  cables  thru  the

grommets before connecting them to the board. I put the board  in

the box component side down. Note: this board is almost a perfect

fit  in this box, but actually is a bit snug. I take a  file  and

trim  1/16" or so off the long side before working on the  board.

Or, you can use the next larger size box.

OTHER COMPONENT SUPPLIERS:

The only slightly difficult to find part is the ICL-232. You  can

get it (or the MAX-232) from:

DIGI-KEY

701 Brooks Ave. South

P.O. Box 677

Thief River Falls, MN 56701-0667

800-344-4539 or 218-681-6674

or

JAMECO ELECTRONICS

1355 Shoreway Road

Belmont, CA 94002

415-592-8097

The  above suppliers will also have the 74LS368 and capacitors  &

connectors.

For the Atari I/O connector or cable I use:

AMERICAN TECHNA-VISION

2098 Pike Avenue

San Leandro, Calif. 94577

415-352-3787

For Atari parts, also see:

B & C Computer Visions

3257 Kifer Road

Santa Clara, CA 95051

408-749-1003

If  you  want,  you can contact me about  parts.  I'll  currently

supply  a kit of all the parts listed plus drawings and  instruc-

tions  for $30 plus $5 P & H. I'll build and test it for  another

$20.  If you go this way, I don't expect  additional  "shareware"

payment for the software.

DISCLAIMER/WARNING!!!

As of this writing there are around 50 to 60 users of this  soft-

ware  and  interface, all having satisfactory  results.  However,

unless you feel comfortable with building circuits and  interfac-

ing  computers, I don't recommend trying it without  help.  Like-

wise, errors are possible in my above circuit description. I must

insist that you assume all responsibility for equipment damage or

personal injury if you decide to build and use my system.

I recommend getting the data sheets for the chips and seeing  for

yourself  that  the pin numbers I give match up with  the  mrgr's

description.

In  general, the inputs and outputs of both the Atari and the  PC

serial  port  are  fairly resistant to  damage  from  inadvertent

grounding,  connecting two outputs together, etc.  However,  your

system  may turn out to be the delicate one. Note:  the  negative

going output of the RS-232 port stands a good chance of  DAMAGING

any  TTL input it is connected to. In other words, the RS-232  to

TTL conversion performed by the ICL-232 chip is absolutely neces-

sary.

On the software side, I have taken pains to ensure against damag-

ing  your files. For instance, I only use DOS calls, I  don't  do

any  direct  disk sector writes. But the PC is a  complex  beast,

file  outputs can be redirected, resident programs can  clash  in

strange ways with TSRs, etc. So, please use this program at  your

own risk.

If  you're building it yourself, also see the  KITINSTR.DOC  file

for a detailed, wire by wire checklist of instructions.

##############################################################################################################