USING THE SIO-2-PC INTERFACE SOFTWARE

*** IMPORTANT NOTE ****

SIO2PC  is  a hobby for me and a service to Atari  users,  not  a

money  making  project.  Therefore I can't accept  any  liability

from  use  or construction of the program or interface.   If  you

have  given me any money and don't want to  accept  responsiblity

for  use  of the system, I will refund your  money.    Don't  use

SIO2PC on computers used for commercial purposes or for  critical

applications. Always back up important files.

This  is  a brief description of the Release  3.00  version.  The

software is menu driven, and I hope, fairly self-explanatory.

If you specify a command by mistake, ESC will usually return  you

to  a  higher level of the menu. Here is a run-down of  the  com-

mands:

CREATE  DISK: Reserves ram area for a ramdisk, single  or  double

density  (command  "C" or "Z").  Prompts for size.  Reports  when

memory  isn't available, and creates the file as a "file  access"

disk  instead of as a ramdisk.   After creating a disk, you  must

format  it  with your Atari. The disk's status is  shown  on  the

appropriate  disk  status line. The size and density  are  shown.

Also,  a single letter field shows an 'N' if you haven't  written

to  the ramdisk, 'W' if you have. amd 'F' for file access  disks.

These fields are also color coded, white, red, and green.

LOAD  DISK IMAGE FROM FILE: This command prompts for disk  #  and

filespec.   Various DOS errors will be reported if there is  any-

thing  wrong with the file you specified.  The  file  information

specified  is  appended to the disk status line. If  there  isn't

enough  ram for a ramdisk, file access will be used.   (To  force

file access, follow the filename with "/P", no space.)

WRITE DISK IMAGE TO FILE: This is essentially the inverse of  the

above with a few exceptions. You have the choice of writing  back

to the file you read from (if any), or you can specify a new file

for  the disk image storage.  This option doesn't apply  to  file

access  disks, which are continually updated.  (Exit SIO2PC  nor-

mally to close files - use option 'Q'.)

SET COM PORT NUMBER: This command lets you choose which port  you

are going to plug the interface into, assuming you have more than

one.  The screen will show the # of the port you have  specified.

Note  that the program begins with the port # query. So,  if  you

didn't  enter it in the batch file, or on the command  line,  you

must now enter it before proceeding. Read TECHINFO & REV_LOG sec-

tions of README.DOC for more info.

TOGGLE  STATUS  LINE: The program status line  gives  information

about what the program is doing, if anything. The program  writes

directly to screen memory for this function (for speed).  Because

this may cause noise on the screen of a CGA computer display, you

have the option of turning it off.

EXCHANGE  DISK  I.D. NUMBERS: The purpose of this command  is  to

allow you to change the number of the default, or bootable  disk.

You could for instance, boot up with a physical drive as drive #1

and  load  files  into ramdisk #2, then turn off  the  drive  and

change  #2 to #1. Note that you can swap a defined  ramdisk  with

one which doesn't yet exist. No problem.

PRINT_THRU: Tells SIO2PC to intercept your Atari's printer  bound

data.  You  can choose to send this data to  your  PC's  printer,

screen, or disk file. You can also choose to translate Atari EOLs

to PC CR's or CR/LFs and/or clear the high data bit.  A field  in

the menu tells the current status of your PRINT-THRU choices.

LOCK  DATA LINE: Usually, leave in Auto. One user needed this  to

cure a bug. See README.DOC for more info.

ENTER  PORT  ADDRESS: Allows you to specify the port  address  in

hex. Usually, the defaults are OK and you don't need this.

UNINSTALL DISK: Removes a disk and frees its memory and  position

for others.  Warns you if the (ram) disk hasn't been saved.

ADJUST  BUS TIMINGS: You should never need to do this. Again  for

the one user with a cantankerous system. See README.DOC for more.

QUIT  PROGRAM: Returns the PC to DOS. Warns you to  save  changed

ramdisks.  Closes any open files.

DIRECTORY  SEARCH  Allows you to look at a PC  directory  without

leaving  SIO2PC.  You can use the usual PC  path  specifiers  and

wildcards.  Also, you can look into the directories of your Atari

disk image files (if Atari/MYDOS compatible).

USING  COMMAND TAILS: The SIO2PC software allows you to  simplify

configuration of the program through the use of the command  tail

feature  of DOS. The command tail consists of text found  on  the

command line after the program name. An example will illustrate:

If you start the program by the command SIO2PC 2 C22 L1A:\DOS.ATR

R,  it tells the PC to: Run SIO2PC, use Port 2, create a  ramdisk

as disk #2, size 92K, Load a disk image into ramdisk #1 from file

A:\DOS.ATR.  Always  make the port # come first. Use /V  to  skip

automatic virus checking.

USING  BATCH  FILES: The real advantage of command  tails  is  in

using  them  from batch files. If the above text  (SIO2PC  2  C22

L1A:\DOS.ATR  R) is stored as a batch file, you can set  up  your

Atari ramdisks for a computing session just by typing the name of

the  batch  file.  You could have different  batch  files  called

GAMES.BAT, PROGBAS.BAT, PROGASSY.BAT etc., automating set-up  and

making  things  much easier for you than remembering a  bunch  of

commands  and file names. You should put a space after  the  last

command of the batch file. You may have noticed that there's no S

before the PORT # in the batch file. That's because SIO2PC starts

up expecting a PORT # entry.

Other Included Programs: There are a few utility programs includ-

ed on your disk, in addition to SIO2PC:

ACOMINIT.COM:  This program will initialize your serial  port  to

tell the interface to stay disconnected from the SIO bus. Reason:

If  you  have the Atari on, and the PC on, but you're  not  using

SIO2PC,  the PC's serial port may be telling the interface to  be

"active,"  thus interfering with operation of the Atari's  serial

bus.  The  program expects a single number on the  command  line,

which  is the number of the port connected to the interface.  You

should copy ACOMINIT.COM to your root directory, and add the line

"ACOMINIT  #" to your autoexec.bat file, where # is 1,2,3, or  4.

Note  that there may still be some conditions in which  the  non-

operating  but connected SIO2PC will interfere with the  SIO  bus

communications.  If that occurs, just unplug the interface.  With

design "G", none of this should be needed.

REMOTE.OBJ:   This program is an Atari program found on the  ram-

disk  file ATARI.ATR on your disk. It allows you to  control  the

SIO2PC program by remote control, from the Atari. Get into DOS on

your  Atari. Load the program and then enter SIO2PC  commands  as

though you were entering them from the PC's keyboard! You have to

anticipate  the prompts and include the answers in  your  command

string. (A few complex commands don't work from command tails  or

from REMOTE.)

BIOSDATA.EXE:  This program runs on your PC. It checks your  BIOS

data area, and uses a BIOS function to see how many serial  ports

you have connected, and what the port addresses are. If you  have

trouble,  run this program and let me know what it  shows.  There

are many utilities out there which do these checks much better.

TESTIME1.EXE  Tests  your PC's timer to be  compatible  with  the

SIO2PC  program. Run and time it if you have trouble.  It  should

take 10 seconds.

FILE2PC.OBJ:  This  is  another  Atari  program  found  on   your

ATARI.ATR  disk file. It allows you to send your Atari  files  to

the  PC  as  PC files instead of as Atari disk  images.  See  the

README.DOC file on the diskette file for more information.

MAKADISK.EXE  This  is  a PC program which takes a  PC  file  and

converts  it to an Atari disk image which you can then load  with

SIO2PC and read with your Atari.

GETTING STARTED

First, you should make a copy of the distribution disk for  safe-

keeping.  Now, plug SIO2PC into the PC the Atari's  Daisy  chain.

Start SIO2PC by typing SIO2PC # [ENTER], where # is the number of

the COM port you are using. Load up the supplied Atari disk image

with the commands: "L", then "1", then A:\ATARI.ATR (This assumes

the floppy is in drive "A.")  Now, the status line for ramdisk #1

on the PC should show that this ramdisk is resident. Turn on your

Atari and try to boot from the PC. You should see some action  on

the  status line: Command: usually "R"; DEVID: 31 (for  disk  1);

COM: "H", flashing to "L" intermittently. Now, ask for a directo-

ry  of  D1. If all is well, you should see that the  ramdisk  has

REMOTE.OBJ  and  FILE2PC.OBJ (described elsewhere),  TANK.OBJ  (a

game written by my brother) and KEYER.OBJ on it.

I  suggest  you put ACOMINIT into your autoexec.bat file  as  de-

scribed  elsewhere, and also create batch files to  automate  the

setup of SIO2PC.

###########################