Notes For SIO2PC Rev. 3.00

Where  shall  I  begin? I've been hard at  work  programming  and

debugging  revision  3.00  for weeks, and now have  the  task  of

trying  to  remember  all the changes I've made.   Here  are  the

highlights:

1) File  access to disk images means you can now use disk  images

much larger than ram (up to 16 megabytes). You can now  define

disk  image size in any random size (in sectors, up to  65535)

instead  of choosing sizes from a list.  Have all  your  Atari

programs  on one disk! Disk images will still be  accessed  as

ramdisks if possible, otherwise physical disk accesses will be

used.  The program still uses DOS files on the PC,  no  direct

sector I/O.

2) You can now have your disks numbered from D1: thru D8: instead

of  1  thru 4.  This is for you fanatics who have  4  physical

disks running already.

3) The program now answers MYDOS (and other's) GET  CONFIGURATION

and  SET  CONFIGURATION commands, so those DOSes can  be  made

aware of your new, large disk images.

4) You  now get a warning before uninstalling a  changed  ramdisk

which hasn't been written back to the disk.

5) There's a spiffy new COLOR user interface screen.  Key  fields

are color coded to make it easier to see important statuses at

a  glance.  Current PRINT_THRU status and chosen  options  are

now displayed on screen.

6) The STATUS line is now new and improved.  Two new fields  tell

you:

a) the current sector # being accessed, and

b) the amount of free ram left.

7) A possible bug was killed which may have made it difficult for

certain DOSes to recognize when a disk image was double densi-

ty. (I'm waiting for feedback to confirm this.)

8) Formatting  now clears the disk image to all zeroes - so  com-

pression programs can work more effectively.

REV 3.00 - The gory details:

Part I - How to use it, How it affects You:

File Access and BIG disks

The biggest change is the addition of file access to disk images.

Since  this type of access doesn't require setting  up  ramdisks,

the  disk  image size isn't limited by available  ram.   You  may

choose  the  size of your disk images using new size  choice  #5.

You  specify the size in sectors.  If you're interested  in  what

this  means in kilobytes, you can figure it from the fact that  a

sector  is 128 bytes for single density disks and 256  bytes  for

double density.  Remember the status line field that shows 'W' if

you've  written to a ramdisk but not saved it?  (And 'N'  if  the

file  has been saved.)  For direct file access type  disks,  this

field will contain an 'F'.

Note that you can force the system to use file access, even  when

there is enough ram for a ramdisk.  You do this by following  the

filename  by  "/P" for physical file access. Don't  put  a  space

between  the filename and the "/P".  The "P" will appear on  your

status line. (The "/" won't.)

Concerns about Speed

I was concerned that the overhead of using DOS and having  physi-

cal  disk  accesses might slow the system down.  On my  two  PCs,

this  hasn't been a problem when using hard drives.  I  use  disk

caches  on  both systems and can't tell  any  difference  between

"ramdisk" and "file access" speed.  With a floppy disk, my  fears

were justified.  Access was slow enough that the Atari would time

out  and  generate errors.  But, when I installed a  disk  cache,

floppy  disk  access  worked fine too.  Some  disk  caches  don't

support  floppies.  Smartdrive (supplied with DOS 5 and  WINDOWS)

doesn't.   I  used  one called LIGHTING.  It  worked  fine  using

conventional  memory  while  Smartdrive continued  to  run  using

extended memory.

Get/Set Configuration

Even after I first made some BIG disks, MYDOS 4.5 still  couldn't

figure  out  that they were bigger than a standard  810  or  1050

disk.   The standard Atari SIO protocol doesn't include  a  means

for the drive to tell the Atari what it is capable of (except  in

a very limited way).  I found that I had to implement the extend-

ed  SIO  commands for Get and Set Configuration  so  MYDOS  would

realize  the  actual size of the disk.  MYDOS does some  of  this

automatically (reads configuration on bootup).  But I found  that

I  had to explicitly do this in sequence so MYDOS  would  realize

the disk size:

Create the new (blank) disk image.

Use  the  Set Configuration command, answering  questions  as

follows:  Configurable? Yes.  High Capacity? Yes.  Number  of

Sector? xxxx.  Do this before formatting.

Format the new disk image.

Actually,  SIO2PC doesn't do anything with the Set  Configuration

command other than acknowledge it (and warn you if the number  of

sectors you specified is incorrect - if this happens, just do the

command again).  But it lets MYDOS know that the disk is  capable

of  more sectors than a standard disk.  The program WAS going  to

also  warn  you if you tried to set the wrong  density.   But  it

turns  out that MYDOS always tries to set high density  when  you

answer  "Yes"  to "High Capacity Drive?".  This doesn't  cause  a

problem  though: single density still works OK.   I'm  describing

MYDOS  because it's what I use.  YOUR Atari DOS may have its  own

quirks  with  regard to large drives. MYDOS can  become  confused

when  you use the SIO2PC Swap Disk I.D.s command.  MYDOS  associ-

ates a certain disk number with a certain configuration.  You may

have  to uninstall and reconfigure the drive with MYDOS when  you

swap disks of different types.

Formatting

With  regard  to formatting, it can take a long time with  a  BIG

disk image, especially on a floppy.  SIO2PC sets a flag when  you

first  create  the disk, to tell itself that the  disk  image  is

already  blank.   So,  when you format it,  the  program  doesn't

bother  clearing the disk.  This saves a lot of time.  After  you

write  to  the disk, the flag is cleared.  Then,  formatting  in-

cludes clearing the entire disk image file.  This could cause the

Atari to time out and not realize the format was successful.   So

you  may  want to avoid formatting "used" disk images.   I  can't

think of a reason you'd want to.  (CREATING a new disk image  can

take  a while too, but the Atari isn't involved in this  process,

so  it won't get impatient.)  I may improve the program to  speed

up  this process if it's a problem.  But creating new  BIG  disks

will probably be an infrequent operation for most of us.

***  NOTE: In a "rev 3.00 and 1/2" modification, I have  taken  a

big  step twoard curing the slow formatting problem.  Instead  of

writing blank sectors one at a time, I now write zeroes in blocks

of  4K at a time, dividing the number of DOS calls by  32.   This

only  works if there is at least 4K of free ram. Otherwise,  for-

matting will still work slowly, as described above.

Note that you need to be a bit more careful when formatting  file

access disks.  With ramdisks, you could always choose to not save

the  formatted disk image, so your original was safe.  With  file

access, formatting actually clears your disk.

New disk numbers: 1 thru 8

There's  not much explaining to do on this.  One  difference  you

will note is that disks are now listed top to bottom in the order

created  or installed, and the information doesn't move when  you

swap  disk I.D.'s, only the numbers move.  This is a side  effect

of  the  rather complex changes I had to make to the  program  to

make this apparently simple modification.

New Color Interface

I used to think real men just designed functional, plain  screens

and put the real work "under the hood", where the program's  work

gets  done.  But I've begun to realize that an attractive  inter-

face  is  important and can enhance the user's  understanding  of

what  the  program is doing.  In addition to just  improving  the

program's look, I improved (I hope) the interface by using  color

coding.   User settable fields in the menu area are set off by  a

contrasting  color.  Filename information for the disk images  is

color  coded.   White is for a ramdisk which has been  saved,  or

hasn't  been written to.  Red (warning) is for a changed  ramdisk

which hasn't been saved.  Green is for a file access disk.  (File

access disks are continually updated, so there's no need to  save

them.  But remember to always exit the program  normally  (option

"Q")  rather  than just rebooting or turning  off  the  computer.

This assures that files are closed.)  I've also added a field  in

the menu window to show the exact status of the PRINT_THRU  func-

tion.

Improved Status Line

The new SECTOR # field lets you see what sector is being read  or

written by the Atari.  This gives you something to look at  while

booting up, or could help you technical types analyze problems.

The status line is now updated whether or not you have it  turned

off (most fields, anyway).  When you turn it back on, the  fields

contain the latest status.  The status line tells you the disk  #

and  sector  # being accessed, even if it's not an  SIO2PC  disk.

This enhances my view of SIO2PC functioning as a "SIO Bus Analyz-

er."

The  new RAM field tells you how much memory you have  available.

Now you can see if a disk image will fit as a ramdisk.  Note that

creating, loading, or uninstalling ramdisks changes this  number,

but doing the same to "file access" disks doesn't.

Incidentally, you may wonder why your old 92K and 143K disks  are

now  90K and 140K.  This has to do with the difference between  a

kilobyte  as  1000  bytes and a kilobyte as  1024  (2^10)  bytes.

Revision 3.00 divides the disk size in bytes by 1024 to establish

the disk size.

The logic of the CMND and LAST fields has changed too.  The  LAST

field formerly contained the command received immediately  before

the one in CMND.  But the Atari usually sends lots of R's or lots

of W's in a row, so both fields would hold the same thing.   Now,

LAST  contains the last command received which is different  from

that  in  CMND.  I had to abbreviate some of the field  names  to

make  room for Sector # and RAM.  What was once COMLINE  (status,

High or LOW of the COMMAND line from the Atari) is now just COM.

Bug Fix?

I said I may have fixed a bug regarding double density.  I'm  not

sure  because it hasn't caused me a problem (using MYDOS).   When

the Atari asks a drive for its status, the response includes  one

byte  which  tells if it is a regular (720  sector)  or  enhanced

(1120 sector) drive and whether it is double density.  The bug  I

found  caused  SIO2PC to fail to set the  "double  density"  bit.

Apparently,  some  Atari DOSes make use of this  information  and

others don't.

Part II: Some Technical Stuff

File Access: A bug!

"File Access" type disk images are exactly the same as  "Ramdisk"

type  image  files.  The 16 byte description header  had  several

spare  bytes.  I used two to hold the high part of the  size  for

the new bigger disks.  All of the spare bytes were supposed to be

zeroes  in  the old design, but a bug caused these two  bytes  to

contain  non-zero  numbers.  This necessitates  fixing  your  old

files  prior to using them with Rev. 3.00.  A separate  file  de-

scribes how to do that.

*** REV 3.00 & 1/2 FLASH:  The program now includes a feature  to

automatically  correct  old style disk images.  You  do  it  like

this:  Load the file, but follow the filename by "/X". (No  space

between  the  filename and the "/X")  If the  file  really  needs

fixing, a message will ask you for confirmation that you want  to

fix  the file. If you type "Y" the file will be fixed -  in  RAM.

You  still  have to write the file back to disk to make  the  fix

permanent.  This is sort of a dangerous operation.  If you  "fix"

a  file  that doesn't need fixing, that is, a new BIG  disk,  you

will truncate it to a smaller size.  For this reason, you  should

fix  all your disk image files at once when you start  using  rev

3.00.  Then you should not have to use the "/X" option again.   I

will probably delete this option in a future revision.

File Access: How it works & tips on efficiency

It's  pretty simple.  MS-DOS maintains a pointer into open  files

which can be set by the program. SIO2PC maintains a corresponding

pointer to the next Atari sector. When SIO2PC receives a read  or

write  sector  request  from the Atari, it  calculates  the  file

offset from the sector number.  It compares this to its record of

where the pointer is now. If they differ, it asks DOS to point to

the new location.  Then it asks DOS to read or write 128 (or 256)

bytes  and increments its internal sector pointer by  one,  since

DOS's  pointer  will now be at the next 128 or 256 block  of  the

file.  As you can see, if the Atari asks for sectors sequentially

the pointer automatically moves along in step and no separate re-

quests  to  reposition the pointer are needed.  This  is  a  good

reason  to try to keep your disk images unfragmented.   Actually,

it's possible to have things DOUBLY fragmented: fragmented  Atari

disk images and fragmented PC disks.  To avoid Atari fragmenting,

you should first put files on the disk which you won't be  delet-

ing or changing: DOS, games, application programs.  Last, put  on

any stuff that will change: data files, text files, etc.  If  you

suspect  your  disk is fragmented it's easy to undo.  Just  do  a

file copy (*.*) from the fragmented disk image to a newly created

and formatted blank disk image.

Screen Colors

I  put the attribute bytes for the various screen colors on  near

the beginning of the file, with a text message, "ATTRIBUTE VALUES

FOR  SCREEN COLORS FOLLOW."  Here is the assembly  language  code

which creates the bytes:

DB 'ATTRIBUTE VALUES FOR SCREEN COLORS FOLLOW'

ATTR1   DB 07h   ; white on black: Normal background

ATTR2   DB 017h   ; white on blue: Title line

ATTR3   DB 02h   ; green on black: Menu box outline

ATTR4   DB 071h   ; blue on white: Menu box contents

ATTR5   DB 07h  ; white on black: first part of disk status line

ATTR6   DB 04h  ; red on black for ramdisk changed/not saved

ATTR7   DB 07h  ; white on black for ramdisk not changed

ATTR8   DB 02h  ; green on black for file access disk

ATTR9   DB 0Eh  ; yellow on black: NORMAL status messages

ATTR10  DB 0Dh  ; violet on black (bright): ERROR status messages

ATTR11  DB 07h  ; white on black: STATUS line field names

ATTR12   DB  05h  ; violet on black (normal): STATUS  LINE  field

entries

ATTR13  DB 03Eh  ; yellow on cyan: User fields within menu box

The  idea  here is that you can use DEBUG, NORTON,  or  something

similar  to change the attributes to suit your tastes.   Most  PC

programming  books give a table of bytes.  What exists, near  the

beginning of the file is this:

ATTRIBUTE BYTES FOR SCREEN COLORS FOLLOW     [as text]

07 17 02 71 07 04 07 02 0E 0D 07 05 3E       [on a HEX display]

Now, if you don't like the color scheme I've chosen, you can edit

the  bytes  using DEBUG or a file editing utility and  write  the

file  back to disk, making YOUR color choices permanent.  See  my

separate  file  on fixing the file bug for some tips  on  how  to

change bytes using DEBUG. If you type DEBUG SIO2PC.COM  [return],

then "D" [return], you will see the attribute bytes.  The program

makes  all attributes 07h when it detects a  monochrome  monitor.

If you have suggestions on improved attributes for color or mono,

let me know.

###########################################################################################################