FILE2PC.DOC:

FILE2PC.OBJ is an Atari program found on your ATARI.ATR disk image

file located on the distribution diskette. If you run this program

on your Atari, it will cause SIO2PC to go into the file transfer mode.

Just follow the prompts on the Atari. You can read the technical

details in the TECHINFO.DOC file if you're interested.

This file transfer function has been much requested. A lot of people

seem to have text files on their Atari's and want to transfer them to

their PC's so the can pick them up on the PC's word processor.

I should tell you that the usual warnings apply to this: Different word

processors use different formatting control codes. This is true even if

you aren't going to a different computer. Plus, the Atari has its own

method of ending a line. It uses an EOL (9B hex or 155 decimal) to tell

its editor to drop the cursor down one and move to the left margin. The

PC, on the other hand, uses two codes, CR & LF or carriage return and

line feed. They are hex codes 0D and 0A (decimal 13 and 10). There are

utilities available which translate codes like these. There was one

called CHANGE on PC Magazine's utilities disk. Unfortunately, it only

works with files up to 40K in size. I plan to write one without this

limit. A common denominator is the so call ASCII file, which is free

of specific non-standard formatting commands. (Like this file.) If your

Atari word processor will output a plain ASCII file, you may want to

use the option before sending it to the PC. Some Atari progarms will

allow you to use an OS feature which lets you PRINT to a disk file.

This should yield a plain ASCII file. Also, if you wanted to send it

to your PC's printer, it should definitely be ready to go in this

format.

Anyway, you will probably want to use a filter program to change your

9Bh's to CR/LF's as a minimum. Note that 9Bh looks like a "cents"

symbol on my PC. To see it, hold down ALT on the PC and type 155 on

your numeric keypad.##################################################