NOTE TO SIO2PC USERS:

I threw in this Morse code program I had written for the 8 bit

Atari just as a filler. If it's of no interest to you, just delete

the related files to eliminate the clutter.

NRK

A R T I C L E

A Computer Keyer for the Atari

8 bit Computers

Working CW in the computer age, I've noticed something:

Real CW men don't use keyboard senders. They do own them - I

have one that I built, plus a couple of commercial programs

for my computer. But I go back to my paddle.

The purpose of this programming project was to create a

computer CW generating program which would:

1) Allow the CW Op to use his paddle.

2) Allow a degree of customized and personal "feel" and

"sound."

3) Give the option of keyboard generated code, with the

special features only a computer based keyer is capable of

giving.

The keyer is loosely based on the AEA "Contester," and

the computer screen is influenced by both the Macrotronics

RM-1000 and Kantronics Hamsoft packages. Keyer features

include adjustable dot and dash ratios, and stored message

(memory) capabilities.

"CW Keyboard" features include a type-ahead transmit

buffer, a real-time "message transmitted" line, and a "memo"

pad area for taking notes while receiving. (I can never find

a pencil or paper after I start a QSO.)

Design philosophy was to make commands and controls

either unnecessary or easy to remember and enter. For

instance, there's no command to toggle from "keyboard" to

"paddle" mode. Just start sending (or typing). A unique

feature allows you to give commands (speed change, etc.)

directly from the paddle, in Morse. A special "command"

character causes the keyer to interpret following characters

as a command sequence. Text sent with the paddle is

interpreted and put to the screen, so your non-CW literate

guests can see that you're really saying something.

DETAILED DESCRIPTION:

The paddle and keyer output are wired from joystick jack

#2. (Plug in a joystick to try it out.) The screen has three

main windows and two one line windows. From the top, they

are:

1) Status Window: Parameters such as speed and ratio -

and other options, are shown here.

2) Command Line: This line lights up to show command

inputs, prompts, and statuses.

3) The Middle Window is normally the Memo Pad area. In

the message edit mode, messages are composed here.

4) Transmit line: Text, as it is transmitted, is

scrolled across this line.

5) Transmit Buffer: This is the type-ahead transmit

buffer area.

OPERATION

Most keyer functions are programmed by entering a

COMMAND sequence, followed by parameters, if needed. There

are three ways to enter the COMMAND mode:

1) Press OPTION

2) Send (by paddle) the COMMAND character (currently the

"wait" character: ._... ).

3) Connect the joystick port COMMAND line to ground.

(This enables you to mount a pushbutton at the paddle, so you

don't have to reach over to the console or send the COMMAND

character to enter the COMMAND mode.) Pull back on the

joystick to demonstrate this method.

The Command Window will light and request input. If you

enter the mode in error, just send an invalid first letter to

abort. (I've reserved "E" - a single "dit", for this.) The

keyer will take itself off the air during command input. Now,

a description of the commands:

SP:  Speed. Enter (from keyboard or paddle) up to 5 digits,

including decimal. Yes, fractional speeds, such as 7.96 wpm,

are allowed. Per Morse convention, you may send the letter

"R" as the decimal point, if you wish. Command terminates

when 5 characters are entered, or when a non-valid character

is sent, as in "20X".

RT:  Ratio, doT: Input the ratio in the same way as the speed

was done.

RH:  Ratio, dasH: Same as above.

CS:  Console Speaker - if you have an '800 or '400, side tone

will be via the console speaker, in addition to the

TV/monitor output. Send CS again to toggle off.

VO:  Video Off: This turns off the screen, making the CS tone

a bit less raspy. (The idea of this and the above command is

that you may want to use the keyer without a monitor. I think

it can be operated "blind" fairly well.)

KD:  Key Down: This is the "tune" command. Any action

(keyboard, paddle, or console button) releases.

OA:  ON/OFF Air: Determines whether the transmitter will

actually be keyed. Now you can play with the keyer without

putting the rig in standby.

IK:  Inverse Keying - Inverts the keying output logic. If you

design your interface with inverted logic - you can correct

via this software command. Also, alternating normal/inverse

output pins are provided on stick 1's jack. Normal logic is

"high" for key down.

TO: Tone. Input the desired monitor tone frequency. (Affects

the monitor/TV speaker output only.)

OM:  Open Message - This command opens a message for creation

or editing. Messages are labeled A through Z. To exit the

Message Edit mode, press ESCAPE, or send my End-Of-Message (

.----- ) code via the paddle.

BE:  Buffer Enable/disable: When enabled the buffer is sent

until empty. It starts when you start typing. When disabled,

you can type into the buffer and send it later. Hitting the

paddle also stops the keyboard sender, but it re-starts when

a key is pressed, unless it is disabled. Pressing the TAB key

duplicates the BE function, for easier control.

BC:  Buffer Clear: Empties the transmit buffer. Shift-CLEAR

may be used from the keyboard.

MA thru MZ: Transmits message memory A thru Z. This command

is primarily for use from the paddle, so any pending text

holding in the transmit buffer will be cleared out and

transmission of the message will start immediately. From the

keyboard, messages are sent by typing the desired label in

inverse video. (Press the Atari key, the the desired letter.

The inverse mode is un-shifted automatically after each

entry.)

PL: Parameter Load (Set up Keyer the way you like it)

PS: Save current parameters to file

NOTE: File AUTOPARA.KEY will auto-load

TL: Textfile LOAD (Load Message File)

TS: Save messages to file

NOTE: File AUTOTEXT.KEY will auto-load

HS: Help Screen: summarizes all commands and some other

useful information on the screen.

SELECT:  The SELECT key toggles the keyboard from the

Transmit Window to the Memo Pad Window and back. In the Memo

Pad mode, you can take notes with no effect on the keyer.

Bugs: I think, and hope, that the program is fairly bug free.

However, I have tested each new feature briefly, killed all

obvious bugs, then moved on to the next one. I have noticed

that when using the CS and/or VO features, I've

occasionally experienced lockup.

More on Messages: Messages can call each other, nested up to

eight deep. If you program an infinite loop (A calls A; A

calls B, B calls A; etc.), the software will recognize it (to

prevent internal stack overflow) and tell you. Any operator

action breaks you out of the loop.

I also recently added a feature to embed control scripts

within  messages.  An asterisk (*) within  message  text

tells the program to interpret the following  characters

as  command  entries. This is mainly for  fun:  You  can

program  up simulated QSOs with each guy having his  own

speed, tone, ratio, etc. For example, with this  message

text:

*SP28  *TO700  *RH3.0 *RT1.0 CQ CQ CQ DE  WA5BDU  K*SP20

*TO500 *RH3.5 *RT.8WA5BDU DE W0XYZ K*SP28 *TO700  *RH3.0

*RT1.0 W0XYZ DE WA5BDU R R = TNX FER CALL =

Here WA5BDU calls CQ at 28 wpm, sidetone of 700 hz, etc.

and  is answered by W0XYZ at 20 wpm, tone 500  hz,  etc.

Hey,  I  just realized it would be easier to  store  the

parameters for each guy under separate messages and have

the main message call up the parameters by one key.

I  guess one real use would be a code  practice  session

which would automatically increment up in speed. Another

use is to assign "Speed Key" like commands to make certain

commands faster, for instance, I have my "F" message

designated for FAST speed, it contains: *SP30, and "S" for

SLOW contains: *SP20.

Future Plans:

File handling capability - with separate files for

parameters and for messages, both to autoload if the correct

filename is present on the disk. (Done - version 1.1)

A Real Time Clock.

Software selectable right/left hand paddle logic.

An auto-logging contest version.

Receiving? Maybe someday. The receive programs I've seen

are nice novelties, but fail under real world conditions. And

I don't think sloppy sending is the big problem. It's lack of

immunity to noise and QRM, in my opinion.

I would also like to allow more configurability.

Ideally, the keyer would be a "Keyer Construction Set." I've

taken a small step in that direction with the ratio controls.

Also desirable would be controllable character and word space

timings, and latching characteristics. Then we could have

that "human" sound, and would not all sound like CW Max

Headrooms. On that subject, here's how the latching timing

works now:

I've divided each element of Morse time in half. If

you're "squeezing" both paddles, you have until the second

half of the current element (dot/dash) before the opposite

element latches in. If you have one paddle closed, the SAME

element will latch in if you still have it closed after the

first half of the following inter-character space. Some

keyers don't give you enough time to get off the paddle, and

you send "1" for "J." With others, you have the opposite

problem. I think My current scheme has a pretty good feel.

What do YOU think? I would like to hear suggestions, bug

reports, and comments.

Nick Kennedy, WA5BDU

300 S. Vancouver

Russellville, Arkansas  72801

INTERFACING

1 2 3 4 5:     Joystick port 2

6 7 8 9 :     on computer.

Pin 1: Keyer output.

Pin 2: Command button.

Pin 3: Dash Contact

Pin 4: Dot Contact

Pin 8: Common

Stick 1 pins 1 thru 4 are alternating "normal" and "inverted"

keying outputs.

######################################################################