ASSEMBLING THE SIO2PC INTERFACE

Thanks  for deciding to build it yourself! A diagram of the  cir-

cuit board is enclosed. It shows the top (component) side of  the

board.  The other side is the "foil" side. In the  drawing  there

appears  to be no space between adjacent copper pads from the  IC

pins. Of course, there really is, my printer just isn't  accurate

enough. Please read through all before starting. Since this board

is symmetrical, it's easy to get confused when you turn it  over,

and  forget that you're looking at a mirror image of the  top.  I

suggest marking "TOP" on the IC1 end, and marking pin #'s 1,8,9,&

16 on the board, both sides.

The  first work item is to solder in the two 16 pin  IC  sockets.

Orient the pin 1 identifier notch correctly.

Now,  you  should solder in the jumper wires. They are  shown  as

heavy  lines.  But note that the leads from  the  4  electrolytic

capacitors  are  also shown as heavy lines. In  places  where  no

other  wire  could contact a jumper, I use bare wire,  because  I

don't  like stripping insulation. Note that the foils really  are

close  together and it's easy to create solder bridges. Use  good

light. I also inspect my work with a magnifying glass. (I started

after a solder bridge gave me fits troubleshooting.) In case  the

diagram is hard to figure, here's a wire run list:

from:          to:            from:          to:

IC1-1          GND            IC1-2          IC2-12

IC1-5          IC2-10         IC1-8          GND

IC1-16         +5V            IC1-15         IC1-9

IC1-3          IC1-12         IC1-14         IC1-16

IC1-10         IC2-9          IC2-16         +5V

IC2-15         GND

Now, install the capacitors. On the electrolytics, the minus lead

is marked, and it is the shorter one.

component:          from:               to:

22 uf cap           (-) on +5V*         (+) on IC2-2 via jumper

22 uf cap           (-) on IC2-3        (+) on IC2-1

22 uf cap           (+) on GND*         (-) on IC2-6 via jumper

22 uf cap           (+) on IC2-4        (-) on IC2-5

* Yes, this is correct.

Now,  It's  time to plug the IC's into the sockets.  IC1  is  the

74LS368  and IC2 is the ICL-232 or MAX-232. There is a dimple  or

notch marking pin 1. Be sure to get pin 1 in the right place. You

usually  must  bend  the pins in a bit to get  them  aligned  for

insertion. After installation, inspect closely for pins which may

have bent under instead of inserting.

Now, wire the two connector cables to the board.  FIRST PULL THEM

THROUGH  THE  GROMMETS  IN THE BOX. The Atari cable  may  have  a

larger  grommet.  You might have to grease the  cable  with  some

Vaseline. Note that the board is going to go into the box  compo-

nent  side  down, foil side almost flush with the top.  You  also

need  to wire the PC's DB-9 connector per the pin #'s  given  and

the  diagram shown looking at the end the cable solders  to.  The

pin numbers are on the connector too, if you have real good eyes.

You  may want to dress the wires to the appropriate length  based

on  where they have to go on the board. Note that the green  from

the  Atari has to reach almost all the way to the other end.  Cut

back  the extra wires in the Atari cable. Obviously if  you  sup-

plied  your own parts instead of getting them from me, your  wire

colors may vary.

Cable:    Wire:     To:

Atari:    Violet    IC1-4

"         Shield    PC cable shield

"         Black     GND

"         Blue      +5V

"         Orange    IC1-11

"         Green     IC2-11

PC        Shield    Atari cable shield

"         Blue      IC2-7

"         White     IC2-8

"         Black     GND

"         Brown     IC2-13

"         Green     IC2-14

TESTING:

Plug  the interface into the Atari (not the PC, yet) as the  only

peripheral,  and  turn on the Atari. If you don't  get  the  blue

screen and "memo pad mode" message, turn if off and troubleshoot.

If  all is OK you might want to check voltages coming out of  the

DB-9 connector. Using pin 5 as common:

Pin 9 = 9v          Pin 7 = ~1.7v

Pin 3 = ~0v         Pin 2 = -9.4

Note that pins 3 and 7 are inputs and their readings have  little

meaning.  Also,  plus or minus a couple of volts on 9  and  2  is

probably OK. Now, go to your operating instructions and give it a

try.  If  all is well, put it in the box and put on  the  lid.  I

suggest  a turn or two of electrical tape around the ends of  the

cables to relieve strain and resist pull-out.

If  you  have trouble, you can check the 232's  charge  converter

output:  pin 2 should be about 10 volts to ground, and pin  6  at

about  -10  volts to ground. If this isn't the  case,  check  the

polarity of your capacitors. A leaky capacitor could also be  the

culprit, or the ever popular "solder bridge."

PARTS LIST FOR SIO2PC KIT

_____     1 - BOX, /w LID, SCREWS

_____     4 - 22 uF ELECTROLYTIC CAPACITORS, 25 VOLT

_____     1 - 0.1 uF DISK CAPACITOR

_____     1 - INTEGRATED CIRCUIT, I.C # 1 (74LS368)

_____     1 - INTEGRATED CIRCUIT, I.C. # 2 (ICL-232)

_____     1 - SIO CABLE WITH CONNECTOR

_____     1 - 5 CONDUCTOR CABLE, SHIELDED

_____     1 - DB9 OR DB 25 CONNECTOR, AS RQ'D FOR YOUR PC

_____     1 - CONNECTOR HOOD

_____     1 - CIRCUIT BOARD, RADIO SHACK 276-150

_____     6" BARE WIRE

_____     6" INSULATED WIRE

_____     2 - 16 PIN IC SOCKETS

_____     1 - ASSEMBLY INSTRUCTIONS

_____     1 - CIRCUIT BOARD LAYOUT

Please check items received against this list.

Note, if you got this file via shareware distribution, the  sche-

matic  and circuit board layout are .GEM files on your  disk.  If

you don't have access to GEMDRAW and want the drawings, send me a

dollar  or  two  and SASE. The BUILD_IT.DOC  file  describes  the

circuit  in words so you could build it without a  schematic.  It

also lists parts suppliers. You can also get a parts kit or wired

interface from me. See BUILD_IT.DOC.

###################################################################################################################