SIO2PC REV 3.01 NOTES

(ALSO REV's 3.02 & 3.03)

It's time to describe rev 3.01 already - pretty fast on the heels of

rev 3.00.  Many of the changes were made to fix bugs in rev 3.00, but

there also are some substantial new additions.

THE BUG FIXES:

Rev. 3.1: Fixed PUT_SECNO so it doesn't unless cmnd is P, W, or R.

Added JMP RED1 after CALL GET_CONFIG.  Previously, it went to U13

which updated the status line, taking too much time.  In READ_DSK,

added the line to change DSK_FLAGS when the LOAD_OPT is /N.

Now the "don't format physical disk" option DOES work.  On

the QUICKA version, I changed the IF NOT ONECHIP to IFE ONECHIP

after I realized that NOT does an EOR, so a non zero number still

ends up non zero unless it were all 1's to start with.  This had

been causing major problems with the ONECHIP version.  Changed the

CUSTOM message to say 65535 sectors max instead of 16535.  Fixed the

number of characters to be input for disk size from 5 to 6 because

the CR counts as one.  Previously, one could only go with 4 digits

maximum.  Corrected the math which converted the number of sectors

to the size in paragraphs.  There was an error for LARGE disks

because I was failing to left shift the high word for each "times 2"

operation.  Fixed the PUT_DEVID routine, because it wouldn't - in

some cases.

THE NEW STUFF:

ULTRA SPEED I/O!!!!  I don't have all the difficulties worked out yet,

but it is working.  The bit rates available on the Atari don't exactly

match those available on the PC, so I had to pick a rate where they were

practically the same: 38 KB.  This isn't as fast as the US Doubler can

go, but it IS twice as fast as SIO2PC usually goes.  A few cautions

are in order.  I couldn't fully understand all the stuff SPARTA DOS is

doing.  It goes to high speed, then back to regular, and again to high.

I get lost somewhere in the middle as to what is the signal to go back

to high speed.  As a patch, I put in the "J" command, for Jump to Ultra-

speed. It actully toggles between the two speeds. (There's also a new

field on the status line for speed: "N" for normal, "H" for High.)

Anyway, after booting Sparta Dos, you will probably need to punch a "J"

to get back to high speed.  We'll find other complications as we go

along.  For instance, trying to mix high speed stuff with functions which

work in regular speed (PRINT_THRU, REMOTE, FILE2PC) will probably

cause problems. We should probably limit the high speed stuff to loading,

saving, and copying files.  But that's most of what we do anyway, right?

MORE DISK CONFIGURABILITY:

I've heard lots of reports about the inability to format certain disk

image sizes.  My adding the "get configuration" and "send configuration"

functions in Rev 3.00 in some cases seemed to make matters worse.

I'm now getting into an area where the specific DOS one uses makes a

difference.  So my treasured concept of "DOS independence" of SIO2PC is

going out the window.  The problem is this:  How can the system tell

the Atari exactly what size and type of disk it is emulating?  The 12

byte configuration table exchanged via the GET/SEND CONFIGURATION

commands would seem to be the answer.  Unfortunately, both MYDOS and

SPARTADOS admit to ignoring most of the information exchanged.  The

variables are:

SINGLE/DOUBLE DENSITY: 128 or 256 bytes/sector

SINGLE/DOUBLE SIDED DISK

NUMBER OF TRACKS/SIDE: TYP: 35, 40, 77, 80 expected

NUMBER OF SECTORS/TRACK: TYP: 18 or 28 expected

The MYDOS doc indicates that it likes to consider the disk to be one

huge track containing all the sectors.  This seems to work pretty well

for SIO2PC.

SPARTADOS evidently expects to receive information which matches its

expectations about certain combinations.  For example, when told a disk

had 80 tracks and 28 sectors/track, it assumed 18 sectors per track.  This

made the number of sectors available as reported by SPARTADOS much less

than the number SIO2PC could actually offer with this disk image.

The bottom line is this:  Even though you can choose a disk image of any

size with the new SIO2PC option #5, you may have to adhere to certain

standard sizes when using SPARTADOS.  I've added a lookup table to the

program.  If the disk size matches a certain standard, it reports a

combination of standard values to the Atari.  Otherwise, it uses the

MYDOS technique of 1 track holding all sectors. You could compute some

standard sizes like this:

Number of Sectors = (#TRACKS * #SIDES * # SECTORS/TRACK)

Size in Bytes = (Number of Sectors) * (#Bytes/Sector)

(For the Size in Bytes, subtract 384 for double density, because the

first 3 sectors are always 128 bytes.)

Enough Math.  Here are the standard sizes you can use with option #5

which will result in standard numbers.  As noted before, not all of these

will work with SPARTADOS:

#SECTORS        KBYTES  DENSITY         TRACKS  SEC/TRK         SIDES

630             80      S               35      18              1

720             92      S               40      18              1

1386            177     S               77      18              1

1440            184     S               80      18              1

980             125     S               35      28              1

1120            143     S               40      28              1

2156            275     S               77      28              1

2240            286     S               80      28              1

1260            161     S               35      18              2

1440            184     S               40      18              2

2772            354     S               77      18              2

2880            368     S               80      18              2

1960            250     S               35      28              2

2240            286     S               40      28              2

4312            551     S               77      28              2

4480            573     S               80      28              2

630             160     D               35      18              1

720             184     D               40      18              1

1386            354     D               77      18              1

1440            368     D               80      18              1

980             250     D               35      28              1

1120            286     D               40      28              1

2156            551     D               77      28              1

2240            573     D               80      28              1

1260            322     D               35      18              2

1440            368     D               40      18              2

2772            709     D               77      18              2

2880            736     D               80      18              2

1960            501     D               35      28              2

2240            573     D               40      28              2

4312            1103    D               77      28              2

4480            1146    D               80      28              2

All YOU have to pay attention to is the density and the number of sectors.

SIO2PC will report the other numbers to the Atari. For MYDOS, just

randomly picking a number of sectors, without regard to the above

chart, seems to work fine.

As I said before, the addition of the GET and SEND CONFIGURAGION info

commands in some cases did more harm than good.  For this reason, I've

added yet another feature.  Now you can turn on or off the configuration

transfer function.  Since the main menu was getting full, I added this

to the TIMINGS menu.  Just hit 'A' to see the timings menu.  Then hit

'C'.  The status of the installed disks will be shown.  'Y' means they

DO respond to configuration commands.  Enter the disk number to toggle

its status with regard to this function.  I believe this should only

be necessary when getting ready to format, if at all.

I realize this discussion of REV 3.01 isn't exactly simple.  I hope you

can glean the necessary info from it.  It should be worth the effort

to get ULTRA SPEED going and to cure formatting problems.  Call or write

if you have problems, bugs, or suggestions.  I'd especially like any

SPARTADOS experts to tell me how it tells me to go back to high speed.

What is the '@' command SPARTA sends to device $4F?  Also any info on

how to get SPARTADOS to accept ANY size disk image would be useful.

Regards,

Nick 2/10/92

REV 3.02 Notes:

While working on a circuit change which I hoped would keep SIO2PC off

line under conditions of PC OFF and SIO2PC not plugged into PC - I

noticed that the system was failing to disable the DATA OUT line after

booting.  This means that SIO2PC might interfere with other peripherals.

(I could bring the program back to it's senses by hitting the "K" option

a couple of times.)

I made a quick small change which I hope fixed this bug.  This is the

only change on REV 3.02.  NOTE: To see the circuit change made to keep

SIO2PC off line, use the VIEW-CKT batch file on your distribution

disk or just put the file 4SIO2PC.ASC to your PC screen.

REV 3.03:

For the UNINSTALL function, the confirm required an uppercase 'Y.'  Now

it's not case sensitive.

The biggest additions for rev 3.03 are self checking functions for

virus detection.  I got a rude introduction to the world of viruses a

while back when I found that my distribution file for SIO2PC was

infected.  Since then, I've added the byte count on the sign off message

as a small first step. Rev. 3.03 adds some more complex checks.  As soon

as SIO2PC gets control, it loads a second copy of itself into memory.

Why?  Because the virus may have already "done its thing" to the running

copy, then fixed it to look unchanged.  Anyway, SIO2PC loads a second

copy, and checks to make sure it isn't any bigger than its supposed to

be. You get a message if the size is wrong.  It then does a CRC

calculation on the copy, skipping the parts which the user is allowed

to modify, such as timing values and color attributes.  If the CRC

result isn't as expected, you get another warning.  The program will

still run, however.

I translated the CRC calculation algorithm from C code in a book on

serial communications.  My version may not be exactly right, but it does

work.  I've tried toggling a single bit and got the warning.  Please

note that no system is foolproof.  Virus and counter-virus development

is like the arms race.

These additions don't seem to slow down program loading to any appreciable

extent, but for those of you who can't tolerate ANY slowdown, I've included

an override.  Just put /v or /V on your command line somewhere after the

program name and the virus checking will be skipped.  (You should put the /v

after the port # since the program comes up expecting a port #.)

Also in rev. 3.03, I made some changes to the companion Atari utility

program, FILE2PC.OBJ.  I've had a rash of complaints lately to the

effect that FILE2PC doesn't always transfer ALL of the bytes of a file.

It turned out that I did find a bug which could have caused this, so

I hope now the program will do all that you expect.  Keep letting me

know about these bugs.

One more last minute bug fix for 3.03:  The routine which changes the

file name color to RED to indicate that the disk had been written to

was taking to long and causing an error when writing to a ramdisk

image for the first time.  I fixed that.