D I S C L A I M E R

I  have tried to ensure that my software is free of  harmful  defects (bugs) and of viruses.  However, either could slip  through my  defenses.  Use the software at your own risk.   Similarly,  I believe  my hardware design to be in accordance with  good  practice,  and I have a number of satisfied users.   However,  wiring errors,  errors in my design, and configuration  differences  are possible and I cannot guarantee that these problems will not lead to damage to your equipment or software or other unforeseen  damages.   Furthermore,  if  you choose to  construct  my  interface yourself, you must be experienced with electronic building  techniques.   Even  if  you are experienced, you could  burn  or  cut yourself or experience other damage or injury.

I consider the SIO2PC system to be a service to the Atari  user's community. Although I ask a shareware contribution, and do make a charge  for  assembling interfaces or kits, I still  consider  my efforts  to  be a hobby and a service.  IF YOU HAVE SENT  ME  ANY PAYMENT FOR ANY SOFTWARE, KIT, OR ASSEMBLED INTERFACE AND DO  NOT WISH TO ASSUME FULL RESPONSIBILITY FOR USE OF OR CONSTRUCTION  OF THESE  ITEMS,  YOU MAY RETURN THEM TO ME AND I WILL  REFUND  YOUR MONEY.  IF YOU ARE CONSTRUCTING THE INTERFACE AND CANNOT  VERIFY THE  ADEQUACY OF THE DESIGN ON YOUR OWN, SEEK THE ADVICE  OF  ONE COMPETENT IN ELECTRONICS.  OTHERWISE, DO NOT ATTEMPT TO BUILD AND USE THE INTERFACE.

BUILDING THE SIO2PC INTERFACE

BACKGROUND ON HARDWARE REQUIREMENTS:

Note: The information in the BUILD_IT directory of the diskette refers to the "two chip" version of the hardware.  I am now supplying only the "one chip" version because it is easier to build and has certain technical advantages.  Both versions work equally well, but I supply software which defaults to the one chip version.  The software can be switched to either version under the "A" sub-menu.

The RS-232 serial port on your PC and the SIO serial port on your Atari  8  bitter are quite similar. There are  some  differences, however  and  the  SIO2PC interface is  designed  to  accommodate these:

1) The Atari puts out (and expects as inputs), TTL level signals. In  general,  +5 volts for logic 1 and 0 volts for logic  0.  The RS-232 standard is bipolar: roughly +5 to +15 for logic 0 and  -5 to  -15  for logic 1. Note that this is also inverted  from  TTL, that is, the more positive signal is logic 0 here. So, the  first hardware  requirement  is  to convert TTL to  RS-232  levels  for signals  from the Atari to the PC, and RS-232 to TTL  levels  for the reverse direction.

2) The RS-232 standard is designed for one to one  communication. The  "data out" line of one device goes to the "data in" line  of another, and vice-versa. The Atari SIO system is a data BUSS. The Atari  computer's data out line can connect to several  peripherals, and their data out lines all connect to the one data in line on the Atari computer. This means there is a potential for interference  if  more than one peripheral tries to talk at  the  same time.  This is solved by simulating an open collector output to the Atari using a diode and pull up resistor.  Formerly, I buffered this signal through a Tri-State driver, which was controlled via a handshaking line from the PC.  The current version uses one less chip and less complication in the software.

3)  Handshaking. The Atari uses only one handshaking line on  its SIO bus. This is called the COMMAND line. The Atari computer uses it  to tell all peripherals to "listen up" because a  command  to one  of you will follow. The PC's serial port has  several  handshaking  lines,  so the interface will just convert  the  COMMAND line's  TTL level to  RS-232 and send it to the PC's  "RI"  handshaking line. Also, another PC handshaking line (RTS) which is an output,  will be used to  enable/disable the Tri-State  data  out line discussed above.

4)  Baud,  Word  Size, Start/Stop bits, etc:  The  philosophy  of SIO2PC is to require no software patches on the Atari. This is so you  can BOOT the Atari from the PC, you can use any  DOS,  there will  be  no software conflicts, etc. Fortunately, we  got  lucky here.  The Atari serial bus runs at 19,200 bits per  second.  The PC's BIOS doesn't allow speeds this high, but the hardware  does, so  the SIO2PC software just programs the chip  directly  without DOS  or  BIOS. The other parameters listed are also  easily  programmed into the serial chip (UART) on the PC by software. So, if we  just handle 1 thru 3 above, we are set to go with  regard  to hardware.

CIRCUIT DETAILS:

I now have files which draw the schematic on your PC screen using built  in  ASCII symbols. This means you don't need  any  special viewer   program  to  see  them.   The  files  are 1SIO2PC.ASC, 2SIO2PC.ASC,  and  3SIO2PC.ASC.  Each takes up one  screen. The batch  file VIEW-CKT.BAT displays the three on screen in  succession.  You can also dump these files to your printer.  Also  note that  I have .GEM files which draw less clunkly versions  of  the same  thing,  plus a board layout using a Radio  Shack  prototype board. Write me for copies.

Chip used:

1) The Harris ICL232 (or MAXIM MAX232) is a single chip RS-232 to TTL and TTL to RS-232 converter. It includes two gates to convert TTL to RS-232 and two to do the reverse. The beauty of this  chip is the internal 5 volt to bipolar power supply converter. Normally, you would need a separate plus and minus 12 volt or so supply for  the  RS-232 levels, but this chip takes the  5  volt  supply (from the Atari) and generates all the voltages needed.

CONNECTOR INFORMATION:

Most  PC serial ports use a 9 pin connector known as a DB-9.  The connector has male pins. Therefore, the connector you put on your interface connector should be a DB-9F, with sockets. This is  the view of the connector on the computer. It is also the view of the rear of the DB-9F connector (the end you solder the wires to):

1 2 3 4 5

6 7 8 9

The Atari serial port uses a specialized connector which I  haven't  found anywhere else. Looking into the jack on the  computer, it looks like this:

2  4  6  8  10  12

1		3  5  7  9  11  13

You  can get the connector itself from  Atari  parts  suppliers. However, I prefer to buy an I/O cord with connectors on both ends and  cut it in half. Then you can use an ohmmeter  to  find  out which wire is wired to which pin.

25 Pin Serial Port Connector:

Many  PC serial cards use a 25 pin connector instead of a 9  pin. Here are the equivalent pins if you have the 25 pin:

DB-9			   DB-25

2	3

3	2

5	7

7	4

9	22

PARTS LIST FOR SIO2PC KIT

_____	1 - BOX, /w LID, SCREWS (R.S. 270-220, 4 X 2 X 7/8)

_____	4 - 22 uF ELECTROLYTIC CAPACITORS, 25 volt

_____	1 - 0.1 uF DISK CAPACITOR, 25 volt or more.

_____	1 - INTEGRATED CIRCUIT, I.C. # 2 (ICL-232)

_____	1 - SIO CABLE & CONNECTOR (Atari I/O cable cut in 1/2)

_____	1 - 5 CONDUCTOR CABLE

_____	1 - DB9F OR DB 25F CONNECTOR (as req'd for your PC)

_____	1 - CONNECTOR HOOD

_____	1 - CIRCUIT BOARD (R.S. 276-150)

_____	6" BARE WIRE

_____	6" INSULATED WIRE

_____	1 - 16 PIN IC SOCKET

R.S. means Radio Shack. You can get most of the common stuff from there. I drill a hole in each end of the box and install grommets (from  Radio  Shack) in them. Then I feed both  cables  thru  the grommets before connecting them to the board. I put the board  in the box component side down. Note: this board is almost a perfect fit  in this box, but actually is a bit snug. I take a  file  and trim  1/16" or so off the long side before working on the  board. Or, you can use the next larger size box.

OTHER COMPONENT SUPPLIERS:

The only slightly difficult to find part is the ICL-232. You  can get it (or the MAX-232) from:

DIGI-KEY

701 Brooks Ave. South

P.O. Box 677

Thief River Falls, MN 56701-0667

800-344-4539 or 218-681-6674

or

JAMECO ELECTRONICS

1355 Shoreway Road

Belmont, CA 94002

415-592-8097

The  above suppliers will also have the  capacitors  & connectors.

For the Atari I/O connector or cable I use:

AMERICAN TECHNA-VISION

2098 Pike Avenue

San Leandro, Calif. 94577

415-352-3787

For Atari parts, also see:

B & C Computer Visions

3257 Kifer Road

Santa Clara, CA 95051

408-749-1003

If  you  want,  you can contact me about  parts.  I'll  currently supply  a kit of all the parts listed plus drawings and  instructions  for $30 plus $5 P & H. I'll build and test it for  another $20.  If you go this way, I don't expect  additional  "shareware" payment for the software.

DISCLAIMER/WARNING!!!

As of this writing there are around 50 to 60 users of this  software  and  interface, all having satisfactory  results.  However, unless you feel comfortable with building circuits and  interfacing  computers, I don't recommend trying it without  help.  Likewise, errors are possible in my above circuit description. I must insist that you assume all responsibility for equipment damage or personal injury if you decide to build and use my system.

I recommend getting the data sheets for the chips and seeing  for yourself  that  the pin numbers I give match up with  the  mrgr's description.

In  general, the inputs and outputs of both the Atari and the  PC serial  port  are  fairly resistant to  damage  from  inadvertent grounding,  connecting two outputs together, etc.  However,  your system  may turn out to be the delicate one. Note:  the  negative going output of the RS-232 port stands a good chance of  DAMAGING any  TTL input it is connected to. In other words, the RS-232  to TTL conversion performed by the ICL-232 chip is absolutely necessary.

On the software side, I have taken pains to ensure against damaging  your files. For instance, I only use DOS calls, I  don't  do any  direct  disk sector writes. But the PC is a  complex  beast, file  outputs can be redirected, resident programs can  clash  in strange ways with TSRs, etc. So, please use this program at  your own risk.

If  you're building it yourself, also see the  KIT_G.TXT  file for a detailed, wire by wire checklist of instructions.