------------------------------------------

Inside the STE Part1 - Introducing the STE

Copyright c.1991 A Greenwood

------------------------------------------

There  is  little doubt that Atari's STE  is  a  significant

improvement to the successful ST range, but many people are still

unsure as to what this relatively new machine has to offer. As an

introduction to a series of articles on programming the STE, this

article  explains just what Atari have  added.  Further  articles

will  explain the tricks and techniques needed to  program  these

extra  features,  with documented 68000 source code  for  example

programs and STE utilities.

Overall,  Atari made improvements in seven main areas on the

STE:

1. A new version of TOS, version 1.6 or 1.62

2. The addition of the Blitter chip previously found on Mega STs

3. A new DMA sound chip with volume and tone control

4. An improved video shifter chip.

5. A Cookie Jar to allow programmers to identify the machine.

6. Two new ports supporting four extra joysticks plus paddles.

7. Use of SIMM cards for easier memory expansion.

------------------

TOS 1.6 (and 1.62)

------------------

This  version of TOS includes all the features of  TOS  1.4,

the latest version of TOS for the ST,  but is slightly  different

internally  to allow for the new hardware.   The main  difference

from  a user's point of view is the inclusion of a menu  item  to

allow  use of the blitter chip to be selected from  the  desktop,

and  the resulting improvements in speed when the blitter is  on.

Other enhancements,  such as the new file selector and the MS-DOS

compatible  disc format,  are the same as for TOS  1.4,  and  are

explained  in the STE addendum sheet supplied with  the  machine.

TOS 1.62 is just TOS 1.6 with the bugs which were present in  the

original STEs corrected.

On the programming side,  this version of TOS can be treated

exactly as TOS 1.4, with the same additions to the GEM libraries.

The  changes made to accomodate new hardware are  transparent  to

the  programmer,  ie  they do not affect the  way  the  operating

system  appears  compared  to  TOS  1.4.   For  most   commercial

programming  languages,  for  instance Devpac 2  and  Lattice  C,

updated  libraries  and macros for using the new  GEM  calls  are

included in versions released after the new TOS.

------------

Blitter Chip

------------

All  STEs  have  Atari's Blitter  chip  as  standard,  where

previously it was only found on the Mega STs. The Blitter chip is

a  direct  memory access (DMA) device which uses the STs  bus  to

transfer  data directly from one part of memory to  another  more

quickly than is possible using the 68000 processor.  It does this

by  using the DMA controller to take control of the 68000's  bus,

either  in  bursts  interleaved  with the CPU  or  until  it  has

finished. In this way the Blitter chip can be used instead of the

processor for fast movement of data.

The  Blitter  chip can also manipulate the data  in  several

ways as it is transferred.  Since data transfer is bit  oriented,

the  source data can be shifted to produce the destination  data,

previously  one  of  the hardest  parts  of  bit-mapped  graphics

programming.  The  Blitter  also  applies  one  of  16  different

combinations of logical operations to the source data  (including

straight copy,  of course),  and can also combine the source data

with 16 word length half-tone RAM registers for special effects.

When  copying  data the Blitter chip can  skip  a  specified

number  of bytes (only even numbers allowed,  ie  word  boundary)

after  both source and destination words,  which is  useful  when

copying  a  single bit plane in low  or  medium  resolution.  The

Blitter transfers data in lines,  and can also skip a set  number

of  words at the end of each line - again,  this applies to  both

source and destination.  The Blitter chip also allows the  source

and  destination data to overlap,  so if necessary a  section  of

memory can be read, manipulated and written back to the same area

of memory

When it is enabled, the Blitter is used by both GEM and some

of the A-line routines, though not by the sprite routines. Use of

the  Blitter in this way can be selected either from the  desktop

by using the Blitter menu item or from within a program by  using

Xbios call 64,  the blitmode call as found previously on the Mega

ST. The Blitter can also be used directly, for example for custom

graphics or sprite routines,  which although complex at first can

produce fast graphics with less code.

---------

DMA Sound

---------

The new DMA sound chip on the STE is in addition to, and not

instead of, the original Yamaha PSG chip. The new hardware uses a

digital  to analogue converter (DAC) to play 8-bit sound  samples

at  one of four different frequencies,  from 6.25 to 50  kHz,  in

either stereo or mono.  By comparison, CD players work at 47 kHz,

but with 16-bit samples - this gives some idea of the quality  of

the sound produced.

The  sound  chip is another  DMA  device,  fetching  samples

directly from memory for output.  This means that less  processor

time is needed for sample playing, which is quicker and of better

quality than with the original sound chip.

Playing  a  sample is achieved by loading the  sound  chip's

address  registers with the start and end address of  the  sample

and  then setting the enable register to either play  the  sample

once  or repeat it.  The DMA sound chip is connected to Timer  A,

and this can be used in a similar way to the hblank interrupt  to

play  a sample a specified number of times,  and to link  samples

together to produce continous sound. The address registers on the

sound chip are buffered,  allowing the address of the next sample

to be set while a sample is playing,  thus allowing samples to be

joined seamlessly.

Comprehensive volume/tone control is also provided using the

new  microwire interface connected to a  volume/tone  chip.  This

allows master volume,  left and right volume, and bass and treble

to  be set through software.  The sound output is via stereo  RCA

jacks on the back of the machine,  ideal for connecting to a  hi-

fi.

So far, the sound chip has been the best used feature of the

STE,  with  many  samplers already compatible and more  and  more

software  appearing with DMA sound as an option.  Hopefully  this

feature of the STE will also be used in conjunction with MIDI and

computer music, where the ST is almost standard.

---------------------------

Improved Video Shifter Chip

---------------------------

The  STE's video shifter chip is an enhancement of  the  old

chip,  providing access to a greater range of  colours,  vertical

and  horizontal  hardware scrolling,  and accepting  an  external

sync.

The  colour  range has been increased from 512  to  4096  by

using four bits instead of three to represent the red,  green and

blue  components of each colour.  To maintain combatibility  with

old machines,  the least significant bit of the new 4-bit  colour

component  is  actually the most significant bit  in  the  colour

nibble - if this were not so all 3-bit colour values from old STs

would appear at half intensity as the maximum intensity was 7 and

is  now  15.  This feature is useful for  graphics  applications,

particularly scanning and video digitizing where a full 16  shade

grey scale can be obtained (see 'User cover disc 56).

Vertical  scrolling is achieved by allowing the screen  base

to take any address,  rather than being on a 512 byte boundary as

before.  This is done using an extra register for the low byte of

the  screen address,  which does not affect the screen until  the

next vertical blank.  This means the screen address can simply be

changed to point to the next screen line,  giving smooth vertical

scrolling.

Horizontal scrolling is achieved through two new  registers.

The  first sets the length of a screen line past the edge of  the

screen,  allowing a screen in memory to be larger than the actual

displayed screen.  This means screen data can be scrolled on from

the sides of the screen.  The second register indicates which bit

of  the  first screen word should be used as the first  pixel  on

screen. This allows the screen to be bit scrolled horizontally up

to 16 pixels,  at which point the base address can be changed  to

continue scrolling.

Using vertical and horizontal scrolling together to  produce

four-way scrolling requires careful combination of both  methods,

due to the different way the registers are used.  The  horizontal

scrolling registers take effect immediately,  while the low  byte

of the screen address is not used until the next vertical blank.

Using the hardware scrolling on the STE,  smooth full colour

full  screen  scrolling is possible with very  little  code,  and

using only a small amount of processor time. However, this method

usually requires more memory.

The external sync input provided via the monitor port allows

the STE's video shifter to synchronise with another device.  This

is particularly useful in video genlocking, as the screen display

can  be synchronised with video equipment,  and has already  been

used on at least one such device.  Without this feature the video

shifter chip often has to be prised out and a circuit board added

to  allow the genlock to synchronise the computer with the  video

device.

--------------

The Cookie Jar

--------------

The  cookie  jar  is a welcome addition to  the  STs  system

variables which provides a list of 'cookies', each of which gives

some  information  about the features available on  a  particular

machine.  This  information includes which CPU the  computer  has

(68000 - 68040),  whether a Blitter chip is present,  whether the

machine has DMA sound and which model the machine is. This allows

applications  to  selectively  use these  features  if  they  are

present,  and  should help with integrating STE features into  ST

software.  Presumably  the  cookie jar has already been  used  in

software which is STE compatible.

Another  feature  of  the cookie jar is  the  provision  for

applications  to  add their own cookies.  This means  that  other

devices added to the ST can use the cookie jar to inform programs

that they are present.

----------------

Controller Ports

----------------

The  STE provides two new ports on the side of  the  machine

which  can be used in a variety of ways.  The intended  uses  are

either four standard joysticks (two per port),  two paddles or  a

light pen or light gun. These ports are connected directly to the

the 68000's bus,  instead of adding extra controller  chips,  and

can  be  written as well as read.  The provision  of  four  8-bit

Analogue  to Digital Converters (ADCs) for the paddles,  and  the

fact  that the outputs are driven when the registers are  written

to will no doubt give rise to other uses for these ports.

Since the two ports both use 15-pin sockets adaptors will be

needed  to connect most controllers.  How well used  these  ports

become  depends largely on games designers and  programmers  (for

joystick,  paddle or light gun use) and applications  programmers

(for  light  pen  use) but the possibilities include  up  to  six

player games,  analogue joysticks for flight simulators and light

gun games.  Another possibility is the use of enhanced joysticks,

perhaps with more than one fire button.

-----

SIMMS

-----

RAM on the STE is on up to four Single Inline Memory Modules

(SIMMs),  which can hold either 1/4 megabyte or 1 megabyte  each.

These are easy to remove and install,  and are cheaper than other

ST memory upgrades.  This allows the computer to be upgraded to 1

megabyte  using  1/4 megabyte SIMMs or 4 megabytes  using  all  1

megabyte  SIMMs.  Mixing the two sizes of SIMM is  possible,  but

requires  extra  software to ensure the computer knows  how  much

memory it has. SIMMs are easy to fit by simply removing the STE's

lid and carefully clipping the SIMMs into place.

It  is often possible to get cheap 1/4  megabyte  SIMMs,  as

anyone   upgrading over 1 magabyte will have the old SIMMS  left.

This  can make upgrading a 520 STE to one megabyte  cheaper  than

buying  a  1040,  and may result in more software making  use  of

extra memory,  particularly with the memory overheads  associated

with sample playing and hardware scrolling.

-------------------

Programming the STE

-------------------

The  rest  of this series of articles  will  concentrate  on

programming the STE,  covering hardware  scrolling,  colour,  the

Blitter  chip,  joystick  ports,  the cookie  jar  and  GEM,  and

starting next month with DMA sound and volume/tone control.