Turbo – Freezer XL / XE Reference card

System Menu (Freezer)

Space Back to the Atari (shift)

Reset Coldboot

S Save to disk

C Cassette

B Boot

F File -> [Dn:] filename input or RETURN for “CORE”

E Load and start from disk [shift] [controll]

F File -> [Dn:] filename input or RETURN for “CORE”

R Save to ramdisk

Filename

X Load and start from ramdisk  [shift] [controll]

Filename

F Save to Freezer ram

C Load and start from Freezer ram [shift] [controll]

Z Erase ram behind OS-Rom (from $C000-$FFFF)

D Dos / Debugger

[SHIFT] + E / X / C SPACE Erase player/missile collision register.

[CONTROLL] + E / X / C Load, don’t start programm

System menu (Cartridge-emulation)

M Cartridge type -> 8K / 16K / OSS / SDX / PDOS / OFF

K Keep settings after reset YES / NO

B Boot option COLD / WARM

S Select RAM / ROM

0-9 Select bank number [DELETE]

RETURN Start emulation

P Activate PDOS (=MyPicoDos)

O Cartridge emulation OFF

DOS-Commands

DIR Directory (all files)

DIR Directory specific file name

DEL Delete file

FMS Format single density

FME Format enhanced density

FMD Format double density

LOC Lock file

UNL Unlock file

REN Rename

LOA Load object file

SAV Save object file from hexadr start to hexadr end

/N Binairy file save (without object header) (Example: SAV CART.ROM/N,A000,BFFF

Q to main menu

/Q Quit [Y] confirm, other key is [Break]

/A All files

Debugger commands Input without space, only there for clarity.

D Display memory contence S = Stop, Q = Continue

I Display memory contence

L Disassemble (LIST)

DL Disassemble (DISPLAY LIST CODE)

R Show registers

R < byte,byte Change registers

G Change return address

V Show interrupt ant VCOUNT at moment of Freeze

V <byte change VCOUNT

SR Sector read

SW Sector write

/Adr / Byte from “address” search “byte”

from “address” search byte sequence

leave out a byte in the seach sequence

from-to “address” search “byte”

//Byte1….  search from $0000 till $FFFF for “byte”

/ continue search

Byte&Mask Memory is Anded with mask before comparing

SIO De…… Give SIO-command (Standard SIO)

SIOR            Reset SIO highspeed

DIO Status query of direct access to $D000-$D6FF

DIO < 1 activate

DIO < 0 deactivate

a Command   Atari SIO remote controll

VER Show system software version number

Q to main menu

Explanation:

Dn: Device	fn Filename		a Hex start	%Number Decimal input

[  ] Optional	Sn Secto number	e Hex end	(standard: Hex)